µ §

A full pattern decomposition program

Authors:

A. Altomare, G. Cascarano, C. Giacovazzo, A. Guagliardi, A.G.G. Moliterni

Istituto di Ricerca per lo Sviluppo di Metodologie Cristallografiche, CNR,

c/o Dipartimento Geomineralogico, Campus Universitario, Via Orabona 4, 70125 Bari,   Italy.

M.C. Burla, G. Polidori

Dipartimento di Scienze della Terra, Universita’, 06100 Perugia, Italy.

U S E R’ S   M A N U A L

Reference: A. Altomare, M. C. Burla, G. Cascarano, C. Giacovazzo, A. Guagliardi, A. G. G. Moliterni & G. Polidori: “Extra: a Program for extracting Structure-Factor Amplitudes from Powder Diffraction Data” (J. Appl. Cryst. 1995, 28,842-846).

1- General Information

EXTRA  is a program designed to extract integrated intensities from a  powder pattern.

Neutron or synchrotron or home diffractometer x-ray data collected in a step scan mode can be used.

To calculate the integrated intensities, the program uses the Le Bail algorithm [1] and combines it with a least squares procedure minimizing the quantity:

µ §.                          (1)

The summation in (1) goes over all the data points,

µ §

yi  = observed intensity at the i-th step

yci = calculated intensity at the i-th step

µ §.             (2)

The summation over h includes neighbouring Bragg h reflections,

s  = scale factor

LPh  = Lorentz - polarization factor

mh = reflection multiplicity

Fh   = structure factor

P  = profile function

A  = asymmetry function

ybi = background intensity at the i-th step.

The background in (2) is automatically estimated and is described by the polynomial function [2]

µ §,

N, the polynomial degree, is automatically defined by the program as the number which provides the best fit of the background interpolation points.

µ § is the µ § associated to the minimum intensity in the range in which the background is estimated.

From the pattern a standard peak is automatically selected. The standard peak fitting is performed to provide the starting values of the variables to refine in the least squares procedure.

The full pattern is automatically divided into intervals.The refined parameters derived by the fitting of each interval will be taken into account for the final  full pattern fitting.

The following profile shape functions are available for user choice:

µ §			Pearson VII

µ §						Gaussian

µ §							Lorentzian

µ §	Pseudo-Voigt

where

µ §		µ § 	µ §		µ §

µ §

µ §     	µ §

µ §

µ §   .

The default is the Pearson VII function. In this case

µ §  .

otherwise

µ § .

The peak shape, the asymmetry, the cell and the background parameters are all refined by the program.

At the end of the run, the program generates a file of reflections containing the extracted intensities which can be used as input file for SIRPOW [3]. Reflections have been generated taking into account the input cell.

2- Notes of implementation

All modifications to the program have to  be  applied  to  the fortran file SPECIF96.

Troubles can occur because  of  different behaviour of  compilers.  Frequently the sintax of the open  statement is different.These statements can be changed by editing the JOPEN and  KOPEN functions (for  sequential and    direct  access  file respectively).

Some computers have some reserved logical unit numbers.  These values  can  be  changed  by editing  the BLOCK DATA DEPEND.

For  VMS or OpenVMS implementations the user has to modify the  value of the variable 'nlen' in BLOCK DATA DEPEND.

3- Graphic interface

This feature is available on all common platforms (UNIX, VMS, DOS). The P.C. (386 or better) DOS version of EXTRA is distributed in EXE format.

To be able to implement this part the user has to compile  the FORTRAN files and one C-file and then  link  together  by using  the library of X-Window. An example of Makefile follows.

# Makefile for EXTRA program (IBM RISC 6000)

#

# following  lines have to be used for HP Apollo 700

#

#FCOM = fort77

#FOPT = -O -c +U77

#COPT = -Oc -I/usr/include

#LOPT = -O -L +U77 /usr/lib/X11R5

#WLIBS = -lX11

#

#following lines have to be used for NON-HP Apollo 700

#

FCOM = f77

FOPT = -O -c

COPT = -O -c

LOPT = -O

WLIBS = -lX11

.f.o:

$(FCOM)   $(FOPT)   $*.f

.c.o:

cc   $(COPT)   $*.c

extra:			extra.o cheamb.o extract.o modback.o vedi.o \

tutor.o list.o datakit.o \

screen96.o \

specif96.o xfines96.o sorcio96.o help96.o xmr96.o

$(FCOM) $(LOPT) -o extra extra.o cheamb.o extract.o modback.o vedi.o  \

tutor.o list.o datakit.o \

screen96.o \

specif96.o xfines96.o sorcio96.o help96.o xmr96.o $(WLIBS)

For SUN, SILICON Graphics, ULTRIX, ALPHA/UNIX and LINUX the file xmr96.c has to be replaced by xmr_96.c.

4- Description of the EXTRA program.

The  main moduli of the program are:

EXTRA, LIST, DATA, EXTRACTION.

EXTRA modulus

It interpretes commands and calls desired routines.

LIST modulus

It is the software interface between EXTRA and the direct access file on which data and results are stored.

DATA modulus

This modulus is used to introduce in EXTRA the data, i.e. cell parameters, space group symbol, wavelength and the pattern. It includes a modified version of the subroutine SYMM by Burzlaff & Hountas [4]. Symmetry operators are directly derived from the space group symbol.

EXTRACTION modulus

In this modulus diffraction integrated intensities are extracted from the powder diffraction pattern.

COMMANDS AND THEIR USE

The input is constituted by a sequence of comments, commands and directives.

The commands are headed by ‘%’ character and directives must follow the related command.

EXTRA recognizes the following commands:

%INITIALIZE 	    The direct access file is initialized (to override the previous results and data)

%DATA                         Data input routine is run

%EXTRACTION           Intensities extraction routine is run

%END                            It marks the end of the input file

%JOB caption                 A caption is printed in the output

%PATH name           Name is the  directory name containing some files necessary to run the    program: EXTRA.PAL, EXTRA.PLT, EXTRA.HLP, FONT (only for DOS)

%STRUCTURE name   Name is the name of the structure to investigate. The program creates a direct access file name.bin. If this command is not used the default name is “STRUCT”.

%WINDOW [x,y]        A graphic window is required. x and y are the dimensions in pixels of the window. The default values of x and y are 639 and 479 respectively. Optionally it is possible to increase them.

%NOWINDOW             graphic window is suppressed.

%CONTINUE            the program runs in default conditions in the EXTRACTION modulus up        to the end.

Directives are described below.

All commands and directives are in free format (column 1-80) and are case independent. Only the first four characters are significant. The keywords can start in any position. If the first not-blank character is ‘>‘ , then the record is considered as a comment.

EXTRA preserves intermediate results. For example, if data have been already stored, in a new run the command %DATA can be omitted.

DATA routine must be used before EXTRACTION routine.

The minimal information needed by EXTRA is constituted by

-cell parameters

-space group symbol

-the diffraction pattern

-the wavelength

-type of radiation (if neutron or synchrotron data are used. Home   diffractometer data is the default)

DIRECTIVES

Preparation of data for DATA routine

CELL   a   b   c   a   b   g

cell dimensions a, b, and c are in Angstrom, a   b and  g  in degrees.

SPACEGROUP string

String is the symbol of the space group, according to International Tables (1974). Blanks are necessary among the terms constituting the space group symbol (see examples at the end of this manual).

WAVELENGTH w

w is the wavelength (in Angstrom) used to collect the powder pattern.

NEUTRON

This directive is used to specify that neutron radiation has been used.

SYNCHROTRON

This directive is used to specify that synchrotron radiation has been used.

RANGE  thmin thmax thstep

thmin, thmax and thstep are the minimum 2q, the maximum 2q and the step width respectively of the powder pattern (thmin must be equal to the 2q corresponding to the first count in the pattern file).

PATTERN  filename

Filename is the name of the profile counts file (up to 40 characters).It is used to change the default. [Default is name.pow where name is defined using the command %STRUCTURE].

RECORD  n

It specifies the number of profile counts per record when n ¹ 10

FORMAT string

String is the run time format to read profile counts. Default value of string is  (f10.0)

JUMP

It is used to skip the reading of the first line in the input pattern file.

DOUBLE

It is used when the reading of the odd lines (or, of the even lines, if JUMP is activated) in the input pattern file must be skipped.

ALPHA

It is used to start the alpha2 stripping.

HISTOGRAM  n

n is the number of bars in the histogram for the alpha2 stripping procedure.

n can be equal to 3, 4 or 5.Default value is 3.

Preparation of  data for EXTRACTION routine

INTERVAL  n

n is the number of intervals in which the powder pattern is divided (max 8). n is only given if the user wants to change the automatic division of the pattern. Each interval range must be greater than 10 degrees.

POLYNOMIAL  k

k is the background polynomial degree (max 10). k is given if the user wants to use a fixed degree in each interval and not the degree automatically selected by the program.

GAUSSIAN

To use only if the gaussian function is preferred.

LORENTZIAN

To use only if the lorentzian function is preferred.

PVOIGT

To use only if the Pseudo-Voigt function is preferred.

DAMPING  x

x is the damping factor used in the least squares routine (default value 0.1).

(It is convenient to use a small x value).

FSTARTING    filename

If the “filename” file is available from any source (and also from a previous run of EXTRA),  containing h, k, l and the corresponding F-squared values, Le Bail algorithm is started again for obtaining new estimates.

FORMAT   string

String is the run time format to read F-squared file if the directive FSTARTING is used. Default value for string is (3i4,38x,f10.0).

STANDARD

To activate the procedure of selection of the profile function type (among Pearson VII, Gaussian, Lorentzian, Pseudo-Voigt) from the best fit of the standard peak .

FIRST

To activate the procedure of selection of the profile function type (among Pearson VII, Gaussian, Lorentzian, Pseudo-Voigt) from the best fit of the first interval .

ALL

To activate the procedure of selection of the profile function type (among Pearson VII, Gaussian, Lorentzian, Pseudo-Voigt)  from the best fit of each interval.

NOSING

This directive is used if the preliminary fitting of the standard peak ( automatically selected by the program ) is omitted.In this case a preliminary fitting of the first interval is carried out.

ZEROPROF x

x is the 2q shift value supplied by the user  (to be substracted from the 2q observed positions). In this case the automatic calculation of the 2q shift using the preliminary fitting of the standard peak is skipped by the program).

NCYCLE n

n is the number of cycles for each interval  (default value is 20).

FAST

This directive can be used if the graphic interface has been requested by the “window” command. In this case, the user graphic interaction in a default run is avoided.

SLOW

This directive can be used if the graphic interface has been requested by the “window” command. In this case the program stands by after some relevant calculations and waits for the user graphic interaction to go on.

WIDTH x

x is the factor which, multiplied by the FWHM of each peak, gives the total peak width. (In this case the automatic calculation of x is not taken into account).

VERBOSE

This directive is used to have a more detailed print in the output file.

* * * * *

EXTRA creates two files:

extra.hkl containing the F-squared values for each reflection which can be used as input reflection file for SIRPOW and extra.sir which can be used as input command file for SIRPOW (however the unit cell content must be supplied by the user).

After a first default run of EXTRA, it is possible to run the program starting from the refined variables in the previous run. In this case the preliminary standard peak procedure and the interval by interval extraction procedure are skipped. The command input file should be:

%structure structure_ name

%recycle

%end

If the graphic interface is activated, at the end of a first default run, the user can decide to start again the program using the extracted intensities as starting values or to continue to refine the full pattern to obtain a better integrated intensity estimates.

How to run EXTRA

If the command input file name is “structure name.pro”, to run the program:

extra “structure name”  (in VMS environment : @extra  “structure name”)

and the main output file name will be “structure name.out”.

The command input file name and the output file name can be optional. In this case, to run the program:

extra “command input file name” “output file name”

or, in VMS environment,

@extra “command input file name” “output file name”

Example of “command input file” for a default run (home diffractometer data)

%window

%structure crox

%job data from a home diffractometer

%init

%data

range 6.0 80.0 0.02

cell 5.447 6.5576 12.1147 106.382 95.715 77.970

wavelength 1.3922

space p -1

%continue

Default “profile counts file” name: crox.pow

crox.pow content:

1166.   1179.   1224.   1172.   1221.   1159.   1204.   1347.   1295.   1146.

1353.   1188.   1214.   1250.   1253.   1240.   1285.   1344.   1314.   1324.

1438.   1444.   1463.   1522.   1664.   1548.   1836.   1865.   1913.   2160.

2367.   2412.   2856.   3186.   3681.   4280.   4941.   5883.   7298.   8978.

11248. 14375. 20187. 25377. 23771.  15690.  8496.   6385.   5553.   5022.

4643.   4371.   4202.   3840.   3684.   3296.   3199.   3047.   2833.   2710.

2567.   2334.   2412.   2156.   2163.   2134.   1981.   1981.   2033.   1901.

1849.   1768.   1761.   1709.   1654.   1616.   1580.   1629.   1561.   1609.

1658.   1609.   1493.   1334.   1298.   1331.   1305.   1289.   1292.   1214.

1149.   1182.   1081.   1146.   1094.   1065.   1030.   1055.    981.    1020.

........  .......   .......   .......   .......   ......    ......   .......   ......    ......

Examples of not-default input command file

Example 1 (data are  from neutron source and the profile counts file format is not the default one).

%window

%struct cfcl

%job neutron source data

%data

range 5.0 150.0 0.05

cell    10.168 14.964 5.100 90.000 90.000 90.000

space f d d 2

wave 1.595

neutron

record 1

format (f8.0)

%continue

“cfcl.pow” content:

391.

403.

422.

427.

.................

Example 2 (the profile counts file name is not the default one and the a2 stripping with a 3 bars histogram is requested).

%window

%struct  zno

%job standard zno data

%data

pattern zno.cou

range 5.0 100.0 0.02

cell     3.249 3.249 5.205 90.0 90.0 120.0

space p 63 m c

wave 1.54056

alpha

histog 3

%continue

Example 3 (a Pseudo-Voigt function will be used and the background in each interval will be described by a constant. The profile counts file format is not the default one and the graphic window is suppressed).

%nowindow

%struct mes

%job x-ray home diffractometer data

%data

range 5.0 88.0 0.02

cell    8.588 9.931 11.105 90.0 93.754 90.0

space p 21/c

wave 1.5406

double

%extraction

pvoigt

polynomial 0

%end

Example 4 [the starting values of the integrated intensities  are supplied by the user in the file “yono.ext” and the number of cycles is increased to 40.

%window

%struct yono

%job synchrotron data

%data

range 7.0 80.0 0.01

cell     9.385 16.394 3.627 90.000 101.067 90.000

space  p  21

wave 1.2323

synchrotron

%extraction

fstarting   yono.ext

ncycle 40

%end

Example 5 [the preliminary fitting of the standard peak is skipped and the starting 2q shift value is supplied by the user (the program refines it)].

%window

%struct agpz

%job x-ray home diffractometer data

%data

range 5.0 80.0 0.02

cell   6.526 20.059 6.464 90.000 90.000 90.000

space p b c a

wave 1.54056

%extract

nosing

zeroprof   0.02

%continue

References

[1] Le Bail, A., Duroy, H. & Fourquet, J.L. (1988). Math. Res. Bull. 23,447-452.

[2] Young, R.A. (1993). The Rietveld Method, Oxford University Press.

[3] Altomare, A., Cascarano, G., Giacovazzo, C., Guagliardi, A., Burla, M.C., Polidori, G. &  Camalli, M. (1994). J. Appl. Cryst. 27,435-436.

[4] Burzlaff, H. & Hountas, A. (1982). J. Appl. Cryst. 15,464-467.