CIF-VALIDATION DOCUMENTATION (18-Jan-2001)

==========================================

This document should provide information on the CIF-validation

tests as specified in the file 'check.def' and implemented in PLATON.

# check.def contains the tests, error thresholds and warning messages

#    issued by the program PLATON when run in the 'VALIDATION' mode

#

#           This file is read when PLATON is called with the '-u' switch

#                    (e.g. 'platon -u sk1500.cif')

#

# - Test output will be in the order as defined implicitly below

#

# - Tests are identified with three-digit numbers

#   _0xx - general

#   _1xx - cell/symmetry

#   _2xx - adp-related

#   _3xx - intra geometry

#   _4xx - inter geometry

#   _5xx - coordination geometry

#   _6xx - void tests

#   _7xx - varia

#

-------------------------------------------------------------------------------

_000-_013 : Pseudo Requests

-------------------------------------------------------------------------------

This set of pseudo requests generates a listing of some relevant reported and

calculated data including transmission range and completeness of the

data set.

-------------------------------------------------------------------------------

_020 : Check on R(int)

-------------------------------------------------------------------------------

R(int) should be well below 10% and in the order of magnitude of the

reported R-values. R(int) may be relatively meaningless when based on a very

limited number of averaged data.

-------------------------------------------------------------------------------

_021 : Check ratio of observed/expected number of reflections

-------------------------------------------------------------------------------

The expected number of reflections corresponds to that in the

asymmmetric unit of the Laue group.

Expected ratio:  less-or-equal 1 for centro symmetric structures and

less than 2 for non-centrosymmetric structures.

Reasons to exceed those numbers can be:

1 - Systematic extinction not omitted from the observed data count

2 - Refinement with redundant (i.e. not averaged/unique) data set.

-------------------------------------------------------------------------------

_022 : Check for completeness

-------------------------------------------------------------------------------

Test for data completeness. The ratio of the reported number of unique

reflections and expected for the resolution given.

-------------------------------------------------------------------------------

_023 : Check Theta-max

-------------------------------------------------------------------------------

Check resolution of the data set. Alerts are issued when

sin(theta) / lambda < 0.6

-------------------------------------------------------------------------------

_024 : Check for Friedel Averaging Preferred

-------------------------------------------------------------------------------

Averaging of Friedel pairs is preferred for MoKa datasets containing

atoms not heavier than Si. Use MERG 4 for SHELXL97 refinements.

-------------------------------------------------------------------------------

_025 : Check h,k,l - range

-------------------------------------------------------------------------------

Check reported h,k,l - range with calculated range based on reported

theta-max.

-------------------------------------------------------------------------------

_026 : Check for weak data

-------------------------------------------------------------------------------

Check whether a sufficient fraction of the unique data is indeed

above the 2 sigma level.

-------------------------------------------------------------------------------

_030 Check on meaningfully refined extinction parameter.

-------------------------------------------------------------------------------

This test checks whether a refined extinction parameter is meaningful

i.e. whether its value is significantly larger than its corresponding s.u.

If not, this parameter should be removed from the model and the structure

refined without this meaningless additional  parameter.

The current default gives a warning when it value is within 3.33 s.u.

-------------------------------------------------------------------------------

_032 Check S.U. of the Flack parameter

-------------------------------------------------------------------------------

Check meaningfulness of Flack parameter

-------------------------------------------------------------------------------

_033 Check deviation of Flack parameter from zero

-------------------------------------------------------------------------------

Check validity of absolute stucture determination

-------------------------------------------------------------------------------

_040 Check for the presence of H-atoms in carbon containing compounds.

-------------------------------------------------------------------------------

Alert for 'no H-atoms' in CIF

-------------------------------------------------------------------------------

_041 Compare the Reported and Calculated Sum Formula

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_042 Compare the reported and Calculated Moiety Formula

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_043 Compare Reported and Calculated  Molecular Weight

-------------------------------------------------------------------------------

Note: atomic weights used in the calculation of the molecular weight

are those used in SHELXL97.

-------------------------------------------------------------------------------

_044 Compare Reported and Calculated density

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_045 Compare Reported and Calculated Z

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_046 Check on Reported Z, MW, D(calc) consistency

-------------------------------------------------------------------------------

D(calc) as calculated from the reported Z and MW is compared for

consistency with the reported d(calc).

-------------------------------------------------------------------------------

_050 Test for mu given

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_051 Test diference of reported and calculated mu

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_052 Test for specification of absorption correction method

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_053 Test for specification xtal_dimension_min

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_054 Test for specification xtal_dimension_mid

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_055 Test for specification xtal_dimension_max

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_056  Test for specification xtal_radius

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_057 Test for absorption correction needed

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_058 Test for Tmax specified

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_059 Test for Tmin specified

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_060 RR Test

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_061 RR' Test

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_062 Rescale Tmin & Tmax

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_063 Test crystal size

-------------------------------------------------------------------------------

Alert for crystals with at least one dimension probably too large for

the homogeneous part of the Xray beam when used for datacollection

using crystal monochromated radiation.

An exception will be datacollection using a beta-filter and a

sufficiently large collimator.

-------------------------------------------------------------------------------

_064 Test for T(max) .GE. T(min)

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_065 Test for applicability of (semi-)empirical abs.corr.

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_070 Test for duplicate atom labels

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_071 Uninterpretable label

-------------------------------------------------------------------------------

This alert relates to unintelligable labels on input and are deleted

from the model & checking.

-------------------------------------------------------------------------------

_080 Test maximum shift/error

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_081 Test for maximum shift/error given

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_082 Test for reasonable R1 value

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_084 Test for reasonable R2 value

-------------------------------------------------------------------------------

R2 will in general have a value twice of that of R1 with refinement

on F**2. Significantly larger values usually indicate a poor refinement

model.

-------------------------------------------------------------------------------

_086, _087 Test for reasonable S value

-------------------------------------------------------------------------------

S should in general be close to 1 at the end of a refinement with a

proper weighting scheme. If not, there might be significant unresolvent

problems with the model.

-------------------------------------------------------------------------------

_088 Test for reasonable Data / parameter ratio  (centro)

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_089 Test for reasonable Data / parameter ratio  (non-centro)

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_095 Test for residual density maximum given

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_096 Test for residual density maximum given

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_097 Test maximum residual density

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_098 Test for minimum residual density

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_099 Test for minimum residual density less zero

-------------------------------------------------------------------------------

-------------------------------------------------------------------------------

_11x : ADDSYM Tests for missed symmetry (with an expanded MISSYM (C) clone)

-------------------------------------------------------------------------------

These tests warn for missed or possible higher (pseudo) symmetry in the

structural model (i.e. based on the coordinate data). Close examination of

the situation at hand is indicated in order to prove/disprove the issue

(usually in combination with the reflection data).

The tests are split-up in three classes of problems:

_110 Reports on potential (pseudo/real) lattice centering or cell halving.

_111 Reports on potential additional (pseudo/real) inversion centres.

_112 Reports on potential additional (pseudo/real) rotation axes and mirrors.

_113 Report New spacegroup suggested by ADDSYM

Note: Disordered atoms are not taken into account in the tests.

-------------------------------------------------------------------------------

_12x : Tests for consistency and completeness of symmetry data items in CIF.

-------------------------------------------------------------------------------

Symmetry in the CIF should be provided in the CIF both explicitly

with a _symmetry_equiv_pos_as_xyz loop and implicitly with

_symmetry_space_group_name_H-M.

The tests done include (with some redundancy to point to the real problem)

_120 Test for consistency of the _symmetry_equiv_pos_as_xyz set with

symmetry_space_group_name_H-M.

_121 Test for valid symmetry_space_group_name_H-M symbol.

_122 Test for missing (i.e. ?) symmetry_space_group_name_H-M symbol.

_123 Test for uninterpretable or inconsistent Spacegroup information.

_124 Test for uninterpretable or absent explicit symmetry records.

-------------------------------------------------------------------------------

_130 - _140 : Test for symmetry restrictions on cell-parameters.

-------------------------------------------------------------------------------

Symmetry restrictions on cell dimensions are checked.

-------------------------------------------------------------------------------

_141 - _146 : Test Su's on cell parameters

-------------------------------------------------------------------------------

The presence of su's (where required) and value are checked.

Su's as given by the diffractometer software are often much

smaller zijn realistic.

-------------------------------------------------------------------------------

_150 Check Reported versus Calculated volume

-------------------------------------------------------------------------------

An alert is issued when the reported unit cell volume differs

significantly from the volume calculated on the basis of the

supplied cell dimensions.

-------------------------------------------------------------------------------

_155 Check for reduced triclinic cell

-------------------------------------------------------------------------------

Unless for special reasons related to the structure/content, a unitcell

and structure is best reported with reference to the Niggli reduced cell.

-------------------------------------------------------------------------------

_16x : Tests for missing s.u. on coordinates or refined C-H

-------------------------------------------------------------------------------

Positional parameters for all non-hydrogen atoms in general positions

are checked for the presence of a non-zero s.u. on them.

This includes parameters fixed to fix the origin in polar space groups

which is no longer necessary when refinement is done with modern programs

(e.g. SHELXL, XTAL).

_161 Missing or Zero su (esd) on x-coordinate.

_162 Missing or Zero su (esd) on x-coordinate.

_163 Missing or Zero su (esd) on x-coordinate.

_164 Refined C-H H-atoms in heavy-atom structure (warning)

Such H-atoms are better refined at calculated positions riding

on the atoms they are attached to.

-------------------------------------------------------------------------------

_201 Test for isotropic non-hydrogen atoms in main residue (molecule)

-------------------------------------------------------------------------------

This test reports on non-hydrogen atoms that were refined with isotropic

displacement parameters only in the main residue. Such a practice is

unusual by modern standards and only needed for minor disorder modelling.

-------------------------------------------------------------------------------

_202 Test for isotropic non-hydrogen atoms in anion/solvent (small moiety)

-------------------------------------------------------------------------------

This test reports on isotropically refined atoms in small moieties (usually

anions or solvent)

-------------------------------------------------------------------------------

_211 Test for NPD ADP's in main residue(s)

-------------------------------------------------------------------------------

This test reports on non-positive definite (i.e. complex and unrealistic)

anisotropic displacement parameters in the main residue.

-------------------------------------------------------------------------------

_212 Test for NPD ADP's in small moieties (solvent/anions)

-------------------------------------------------------------------------------

This test reports on non-positive definite (i.e. complex and unrealistic)

anisotropic displacement parameters in small moieties (solvent/anion).

-------------------------------------------------------------------------------

_213 ADP maximum/minimum ratio test for main residue

-------------------------------------------------------------------------------

The maximum and minimum main axis ADP ratio (Angstrom Units) is tested

for the main residue. Large values may indicate unresolved disorder.

-------------------------------------------------------------------------------

_214 ADP maximum/minimum ratio test for small moiety

-------------------------------------------------------------------------------

The maximum and minimum main axis ADP ratio (Angstrom Units) is tested

for the minor residue(s). Large values may indicate unresolved disorder.

-------------------------------------------------------------------------------

_220 Test Ueq(max)/Ueq(Min) range for non-H  in main residue(s)

-------------------------------------------------------------------------------

This test reports on a larger than usual U(eq) range for the non-hydrogen

atoms. Too high or too low Ueq's may be an indication for falsely identified

atomic species (i.e. Br versus Ag)

-------------------------------------------------------------------------------

_221 Test Ueq(max)/Ueq(Min) range for non-H atoms in minor residue(s)

-------------------------------------------------------------------------------

This test reports on a larger than usual U(eq) range for the non-hydrogen

atoms. Too high or too low Ueq's may be an indication for falsely identified

atomic species (i.e. Br versus Ag)

-------------------------------------------------------------------------------

_222 Test Ueq(max)/Ueq(Min) range for H atoms in main residue(s)

-------------------------------------------------------------------------------

This test reports on large ranges in displacement parameters for hydrogen

atoms.

-------------------------------------------------------------------------------

_223 Test Ueq(max)/Ueq(Min) range for H atoms in minor residues(s)

------------------------------------------------------------------------------

This test reports on large ranges in displacement parameters for hydrogen

atoms.

-------------------------------------------------------------------------------

_230, _233 : Hirshfeld rigid bond test

-------------------------------------------------------------------------------

Components of anisotropic displacement parameters along chemical bonds

are assumed to be equal in magnitude. Large differences supposedly indicate

contamination of these parameters with other (unresolved) effects such

as disorder and/or overrefinement.

Atom sites assigned the wrong scattering type (e.g. Ag versus Br) should

generate 'problem signals' with this test.

Data sets corrected for absorption effects with DELREF techniques

(e.g. DIFABS, SHELXA, XABS2) often show large DELU values for bonds involving

the heaviest atom.

-------------------------------------------------------------------------------

_241, _242 Test for too high/low U(eq) as compaired to neighbours

-------------------------------------------------------------------------------

The U(eq) value of an atom is compaired with the average U(eq) for

to non-hydrogen atoms bonded to it. Large differences may indicate

that the wrong atom type was assigned (e.g. N instead of O).

-------------------------------------------------------------------------------

_301, _302 : Detection of Disorder

-------------------------------------------------------------------------------

Atom sites that are not fully occupied are counted. A large fraction of

disordered atoms may be both a signal for serious structure determination

problems or less reliable/interesting results. A distinction is made

for 'solvent molecules of less interest' (_302) and the

'main components' (_301)

-------------------------------------------------------------------------------

_305 Test for 'Isolated' Hydrogen atoms

-------------------------------------------------------------------------------

This test reports on hydrogen atoms that are not on bonding distance to

any atom.

-------------------------------------------------------------------------------

_306 & _311 Test for 'Isolated' Oxygen atoms

-------------------------------------------------------------------------------

This test reports on oxygen atoms that are not within bonding distance to

any other atom in the structure.

A common reason may be that no hydrogen atoms are given for a water molecule.

Alert _306 is issued for full weight atoms, _311 otherwise.

-------------------------------------------------------------------------------

_307 Test for 'Isolated' Metal atoms

-------------------------------------------------------------------------------

This test reports on metal atoms that are not bonded or at coordination

distance of other atoms.

Isolated ions are very unusual (or non-existent ?)

-------------------------------------------------------------------------------

_308 Test for single bonded metal atoms

-------------------------------------------------------------------------------

This test reports on single boned (coordinated) metal atoms/ions.

This represents a very unusual situation. There are literature examples

where such a 'single bonded metal' was shown to be a halogen.

-------------------------------------------------------------------------------

_310 Test for 'Too close' (symmetry related) full weight atoms.

-------------------------------------------------------------------------------

This test identifies (very) short contacts between atoms that only becomes

apparent after the application of symmetry on the primary coordinate set.

-------------------------------------------------------------------------------

_340, _341, _342 Test on Bond-Precision

-------------------------------------------------------------------------------

The average su for X-Y bonds is tested (named bond-precision).

X-Y will generally be C-C bonds, unless there are none. In the last

case the su's of the lowest element numbers are considered (excluding

hydrogen).

There are three test ranges: one for structures with the largest element

Z < 20, one for the largest Z in the range 20 to 39 and one for

structures with Z(max) 40 or higher (_340, _341 and _342 respectively)

-------------------------------------------------------------------------------

_320, _321 : Checks on Hybridisation of C atoms

-------------------------------------------------------------------------------

This test is split up in two sections: the main molecule (_320) and

solvent molecules (_321).

The test attempts to assign one of three hybridisations to C atoms:

sp, sp2 or sp3 on the basis of the angles around C. In this way, missing

H atoms or too many H-atoms on a carbon atom should be detected.

-------------------------------------------------------------------------------

_350 : _373 : Test for Short/Long Bonds as compaired with standard values

-------------------------------------------------------------------------------

Large deviations from generally accepted values may indicate model

problems, overrefinement etc.

Default (X-Ray) values are from SHELXL (X-H) and

Ladd & Palmer, Structure Determination by Xray Crystallography (1985).

Note: C4-C3 indicates a bond between an atom with 4 bonds and one with 3 bonds.

Conjugated systems may cause some 'false alarm' messages.

_350 Short C  - H                         Reference Xray : 0.96 Ang

_351 Long  C  - H                                          0.96

_352 Short N  - H                                          0.87 Ang.

_353 Long  N  - H                                          0.87

_354 Short O  - H                                          0.82

_360 Short C4 - C4                                         1.54

_361 Long  C4 - C4                                         1.54

_362 Short C4 - C3                                         1.52

_363 Long  C4 - C3                                         1.52

_364 Short C4 - C2                                         1.46

_365 Long  C4 - C2                                         1.46

_367 Long  C? - C?                                         1.50

_368 Short C3 - C3                                         1.34

_369 Long  C3 - C3                                         1.34

_370 Short C3 - C2                                         1.31

_371 Long  C3 - C2                                         1.31

_372 Short C2 - C2                                         1.25

_373 Long  C2 - C2                                         1.25

-------------------------------------------------------------------------------

_380 Check for Methyl Moiety refinement

-------------------------------------------------------------------------------

This test alerts for possible unrefined CH3 moieties.

(E.g. AFIX 33 instead of AFIX 137 etc. within the SHELXL realm)

-------------------------------------------------------------------------------

_390, _391 Test for unusual CH3 geometry.

-------------------------------------------------------------------------------

_390 Reports on unusual X-C-H Angles

_391 Reports on unusual H-C-H Angles

Methyl moieties easily refine to poor geometries when refined in a

poorly defined electron densityi or due to orientational disorder.

If so, refinement with relevant con-/restraints should be considered.

-------------------------------------------------------------------------------

_410, _413 : Tests for Short Intra and Intermolecular H...H Contacts

-------------------------------------------------------------------------------

Short intermolecular H..H contacts may indicate incorrectly determined

structures (i.e. wrong symmetry, missed translation symmetry, wrong

position with reference to the symmetry elements, hydrogen atoms on

atoms where there should not be any  etc..)

Short intramolecular contacts may arise when H-atoms are in (false)

calculated positions.

Short intramolecular contacts may also be a sign for a false structure

with the molecule sitting on a site with improper site symmetry (e.g.

'2' instead of '-1') which may happen when a lattice translation is missed.

Short contacts are defined using a van der Waals radius of 1.2 Angstrom.

For intermolecular contacts, an alert is generated for contacts less than

2.4 Angstrom. For intramolecular contacts alerts are generated for contacts

less than 2.0 Angstrom.

Shorter intramolecular contacts may make sense for crowded configurations,

in particular when hydrogen atoms are calculated in idealised positions.

_412 & _413 report on short H .. H contact involving CH3 H-atoms. Those

positions are often optimal when in calculated position.

-------------------------------------------------------------------------------

_416 : Test for short intra D-H .. H-D contacts

-------------------------------------------------------------------------------

Such contacts may be related to disordered of misplaced H-atoms.

-------------------------------------------------------------------------------

_417 : Test for short inter D-H .. H-D contacts

-------------------------------------------------------------------------------

Such contacts may be related to disordered of misplaced H-atoms.

-------------------------------------------------------------------------------

_420 : Test for D-H without acceptor

-------------------------------------------------------------------------------

Potential hydrogen bond donors are checked for the presence of suitable

acceptors using commonly used (Jeffrey) H-bond criteria. As a general rule

there should be an acceptor for each donor. Exceptions are very rare for

O-H and more common for -NH and -NH2. A common error is an -OH on a

calculated position pointing in the wrong direction.

-------------------------------------------------------------------------------

_430 : Test for short intermolecular D..A distances

-------------------------------------------------------------------------------

This test alerts for possibly missed Hydrogen bonds as indicated

by short (i.e. shorter than sum of the van der Waals radii - 0.2)

Donor - Acceptor distances.

-------------------------------------------------------------------------------

_431 : Test for short intermolecular Hl...D distances

-------------------------------------------------------------------------------

This test reports on short intermolecular Halogen .. Donor/Acceptor

atom-type distances.

-------------------------------------------------------------------------------

_432 : Test for short intermolecular distances

-------------------------------------------------------------------------------

This test reports on short intermolecular atom-type distances falling

in anather class than alerts _430, _431.

-------------------------------------------------------------------------------

_601, _602 : Solvent Accessible Void(s) Test(s)

-------------------------------------------------------------------------------

Crystal structures in general do not contain large solvent accessible

voids in the lattice. Most structures loose their long-range ordening

when solvent molecules leave the crystal. Only when the remaining network

is strongly bonded (e.g. zeolites and some hydrogen bonded networks) the

crystal structure may survive.

Residual voids in a structure may indicate the omission of (disordered)

density from the model. Disordered density may go undetected when smeared

since peak search programs are not designed to locate maxima on density

ridges. The presence or absence of residual density in the void may be

veryfied on a printed/plotted difference Fourier map or with PLATON/SQUEEZE.

Voids of 40 Ang**3 may accommodate H2O. Small molecules such as Tetrahydro

Furane have typical volumes in the 100 to 200 Ang**3 range.

_601 : This test reports the volume of the largest solvent accessible void

in the structure.

_602 : This test reports on a void, too large for the current PLATON version

for more detailed analysis.

----------------------------------------------------------------------------

_701, _702, _703 Check for consistency of derived geometry and coordinates

----------------------------------------------------------------------------

The Bond distances, Angles and Torsion angles given in the CIF are

cross-checked with corresponding values calculated from the coordinates.

Alerts are set at 1,2 and 3 sigma deviation levels.

Note: Default s.u.'s are used where no su given (e.g. for C-H)

In general, all differences should be within the associated su. Small

differences may arise from rounding.

Very large deviation (or zero distance) normally indicate incorrectly

specified symmetry operations on the associated atoms, or

'cut-and-pasting' of incompatible CIF's.

----------------------------------------------------------------------------

_704 Check for consistency of contact distance geometry and coordinates

----------------------------------------------------------------------------

Intermolecular contacts listed in the CIF are checked against the

coordinates in the CIF.

Alerts are set at 1,2 and 3 sigma deviation levels.

----------------------------------------------------------------------------

_705, _706, _707, _708 Check of H-Bond geometry against coordinates

----------------------------------------------------------------------------

Hydrogen-Bond data (D-H, H..D, D..A, D-H..A) listed in the CIF are checked.

Alerts are set at 1,2 and 3 sigma deviation levels.

----------------------------------------------------------------------------

_710 Check for linear torsions

----------------------------------------------------------------------------

Torsion angles specified in the CIF are checked for the 'linear variety'

where one or both of the 1-2-3 and 2-3-4 bond angles are close to 180 Deg.

SHELXL97 will generate those 'torsions' for molecules containing

linear moieties (E.g. Metal-C=O).

----------------------------------------------------------------------------

_711 to _718 Inconstent labels

----------------------------------------------------------------------------

When labels are found on geometry items (bonds, angles etc.) that are

not in the coordinate list, and alert _71n is issued, related to alert

_70n.

----------------------------------------------------------------------------

_721, _722, _723, _724 Check for consistency of derived geom. and coordinates

----------------------------------------------------------------------------

Same as _701, _702, _703, _704, but for distances, angles and torsions

without su (esd). Difference is tested in terms of Angstroms and

Degrees.

----------------------------------------------------------------------------

_725, _726, _727, _728 Check of H-Bond geometry against coordinates

----------------------------------------------------------------------------

Same as _705, _706, _707, _708, but for distances and angles without

su (esd). Differences are tested in terms of Angstrom and Degrees.

----------------------------------------------------------------------------

_731 to _738 Check on 'Comparable' reported and calculated s.u.

----------------------------------------------------------------------------

Same as _701 to _708 but for reported and calculated s.u.'s (Ratio)

This range of checks should warn for erroneous rounding:

E.g.         105.5(19) to 105.5(2)

105.0(5)  to 105(5)

etc.

Note: Large differences are possible when certain constraints/restrains

were applied in the refinement (e.g. the FLAT option in SHELXL97)

----------------------------------------------------------------------------

_741 to _748 Check for missing s.u.'s on Bonds, Angles, Torsions

----------------------------------------------------------------------------

An Alert in this series generally indicates a missing s.u. on the

reported Bond, Angle, Torsion.

----------------------------------------------------------------------------

_751 to _758 Check for senseless s.u.'s on Bonds, Angles, Torsions

----------------------------------------------------------------------------

Alerts are issued for constrained bonds, angles, torsions (i.e. with

the R flag in the CIF or of the type X-Y-X' = 180 where Y is on an inversion

Center.

----------------------------------------------------------------------------

_761, _762 Check for presence of X-H bonds and X-Y-H or H-Y-H angles

----------------------------------------------------------------------------

The CIF should also contain those data.

----------------------------------------------------------------------------

_763, _764 Check for too few or too many non X-H Bonds in CIF

----------------------------------------------------------------------------

----------------------------------------------------------------------------

_780 Check for Connected Atom Set

----------------------------------------------------------------------------

Atoms given in a CIF should form a 'connected set', i.e. no symmetry

operations are needed to get atoms in a bonding position.

A connected set of atoms is not needed for the refinement (unless

hydrogen atoms are to be added at calculated positions).

Geometry listings (bonds,angles,torsions & H-bonds) become unwieldy

for non-connected atom sets.

----------------------------------------------------------------------------

_790 Check for Residue Centres to be within the Unitcell bounds

----------------------------------------------------------------------------

Unless for a good reason, molecular species should be transformed

(by symmetry and/or translation) sothat their Centres of gravity are close

to or within the unitcell bounds.

This is a strickt rule for the main species.

Deviations from this general rule are smaller additional species when

relevant for intermolecular interactions with the main species.

============================================================================