PROGRAM FOR CALCULATION OF TDS-CORRECTIONS OF INTEGRATED

INTENSITIES FOR SINGLE CRYSTALS

VERSION OF FEBRUARY 1996

INSTITUT LAUE-LANDEVIN

PROGRAM NAME: TDS

PURPOSE: CORRECTION OF BRAGG INTENSITIES FOR THE EFFECT OF ANISOTROPIC THERMAL DIFFUSE SCATTERING (OMEGA-AND OMEGA-“THETA SCANS, CIRCULAR AND RECTANGULAR APERTURE, ALL CRYSTAL SYSTEMS EXCEPT TRICLINIC). THE PRESENT VERSION FORMS PART OF THE COLL5SYSTEM.

AUTHORS: J.KURITTU& M.MERISALO HELSINKI, FINLAND.

WITH CONTRIBUTIONS FROM S.WILKINS, J.TIBBBALLS AND W.F.KUHS.

LOCAL CONTACT: W.F.KUHS ILL19

HOW TO USE: PREPARE INPUT FILE ?????.DAT ACCORDING TO THE MANUAL(FOUND AS    TDS. MAN ON (10,25). HAVE YOUR DATAP OUTPUT FILE READY AND DO RUN TDS (10,25)GIVE FILENAMES AND THEN WAIT OR BETTER SUBMIT THE JOB IN BATCH!

MANUAL: AS I SAID IS ON (10,25) AS TDS.MAN.

ABSTRACT

THE PROGRAM CALCULATES THE CORRECTION FACTORS ALPHA FOR

ANISOTROPIC THERMAL DIFFUSE SCATERING IN THE LONG

WAVELENGTH, CONTIUUM ELASTICITY APPROXIMATION AND APPLIES

THESE CORRECTIONS TO THE INTEGRATED INTENSITIES FOR SINGLE

CRYSTALS OF CUBIC , TETRAGONAL, ORTHORHOMBIC,

MONOCLINIC, HEXAGONAL,AND TRIGONAL SYMMETRIES. THE PROGRAM

IS RESTRICTED TO OMEGA OR OMEGA-2THETA SCANS AND DEALS WITH

RECTANGULAR AND SPHERICAL RECEIVING SLITS. THE PROGRAM USES

THE FACT THAT THE THREEDIMENSIONAL SUMMATION OVER THE

WAVEVECTORS CAN BE REDUCED TO A TWODIMENSIONAL NUMERICAL

INTEGRATION OVER THE WAVE-VECTORS ENDING AT THE FACES OF THE

ACTIVE VOLUME. FOR A MORE DETAILED DESCRIPTION SEE

J.KURITTU& M.MERISALO J.APPL.CRYST. 11,179 (1978). A

CORRECTION FOR TDS CONTRIBUTIONS FROM TWO-PHONON PROCESSES

MAY BE INCLUDED OPTIONALLY, THE SECOND ORDER TDS CORRECTION

FOLLOWS E.D.STEVENS, ACTA CRYST.A30,184 (1974).THE

EXTENSIONS TO THE ORGINAL VERSION WERE PROGRAMMED BY

J.TIBBALLS (2-PHONON TDS) AND W.F.KUHS (2-PHONON TDS AND

CIRCULAR APERTURE). S.WILKENS ADAPTED THE PROGRAM TO THE

COLL5-SYSTEM.

THE ESTIMATED CPU-TIME FOR ONE REFLECTION WITH A GRID

10´10´10 IN THE ONE-PHONON APPROXIMATION IS 1.2 SEC.

INPUT

THE INPUT OF THE RUN CONTROL FILE IS IN FREE FORMAT. THIS

FILE CONTAIN YOUR CHOICE OF OPTIONS AND PARAMETERS. THE

REFLECTION DATA ARE ASSUMED TO BE ON A SECOND FILE.

NORMALLY THIS FILE WILL BE A FILE CREATED BY THE PROGRAM

DATAP CONTAINING THE UNIT VECTORS WHICH DEFINE THE

DIRECTION OF THE PRIMARY AND REFLECTED BEAM. BOTH, THE

CONTROL AND THE REFLECTION DATA FILE MUST HAVE THE EXTENSION

DAT.

CONTROL DATA

THE FILE CONTROLLING THE PROGRAM CONTAINS ALWAYS THE

FOLLOWING 13 CARDS:

1

FORMAT(16A5)

TITLE: FEEL FREE TO TYPE ANYTHING YOU WANT

2

FORMAT(A5)

CUBIC	   FOR THE CUBIC SYSTEM

TETRA	   FOR THE TETRAGONAL SYSTEM

HEXAG	   FOR THE HEXAGONAL SYSTEM

TRIGO	   FOR THE TRIGONAL SYSTEM

ORTHO	   FOR THE ORTHORHOMBIC SYSTEM

MONOC	   FOR THE MONOCLINIC SYSTEM

THIS PARAMETER HAS TO BE TYPED IN UPPER CASE!

3

FORMAT(6I)

CONTROL PARAMETERS CALLED ICN(I), I=1,6

FIRST:    SCAN MODE PARAMETER

= 1 FOR OMEGA-2THETA SCANS

= 2 FOR OMEGA SCANS

SECOND:   APERTURE PARAMETER

= 0 FOR CIRCULAR APERTURE

= 1 FOR RECTANGULAR APERTURE

THIRD:   SCAN WITH PARAMETER

= 1 SCAN WITH CALCULATED FROM EXPRESSION

A1*TAN(THETA)+A2 (S.CARD    4    )

= 2 TAKEN FROMTHE REFLECTION INPUT FILE

FOURTH:  = 0 NORMALLY

= 1 RHUMBOHEDRAL SYSTEM WITH HEXAGONAL SETTING

FIFTH:   VECTOR DEFINITION PARAMETER

= 0 KEEP DATAP VECTORS (XN PERP. SCATT. PLANE, XNSO,

DIFRACTED BEAM)

= 1 USE TDS VECTORS (XN AS ABOVE, XNSO PERP. PRIMARY

BEAM IN THE SCATTERING PLANE)

FOR THE OUTPUT COLL-5FILE.

SIXTH:   = 0 ONE-PHONON TDS CORRECTION ONLY

= 1 TWO-PHONON TDS CORRECTION INCLUDED.

4

FORMAT(7F)

FIRST:    A LATTICE CONSTANT

SECOND:   B LATTICE CONSTANT

THIRD:    C LATTICE CONSTANT; A B C IN ANGSTROM UNITS

FOURTH:   MONOCLINIC ANGLE BETA (GIVE 90.0 FOR ALL OTHER SYSTEMS!)

FIFTH:    WAVELENGTH IN ANGSTROM

SIXTH:    DENSITY IN G/CM3

SEVENTH:  ABSOLUTE TEMPERATURE IN K

NOTE: THE HEXAGONAL SETTING HAS TO BE USED FOR ALL

RHOMBOHEDRAL SYMMETRIES (EVEN IN THE CASE OF ICN(4) =0!).

NOTE: THE PROGRAM TRANSFORM THE MONOCLINIC CELL

INTO AN ORTHOGONAL CELL. IT ASSUMES:

A) THE UNIQUE AXIS IS THE B-AXIS.

B) THE COORDINATE SYSREM OF YOUR ELASTIC

CONSTANTS IS DEFINED AS E(1)//A,

E(2)//B AND E(3) = E(1)´E(2).

IF THIS DOES NOT APPLY TO YOUR DATA, RE-

ARRANGE YOUR HKLS AND YOUR LATTICE CONSTANTS.

5

FORMAT(4F OR 3F)

FIRST:    FULL WIDTH OF RECEIVING SLIT IN RADIAN

SECOND:   FULL HEIGHT OF RECEIVING SLIT(RECTANGULAR CASE

ONLY!); OMIT IN CASE OF CIRCULAR APERTURE.

THIRD:    A1 FOR DETERMINING THE

FOURTH:   A2 RESOLUTION FUNCTION (S.ICN(3))

FIFTH:    A3

6

FORMAT(4I)

FIRST:        LMAX      FOR DEFINING THE

SECOND:       MMAX     DENSITY OF THE MASH

THIRD:        NMAX      FOR NUMERICAL INTEGRATION.

NOTE: MMAX´NMAX, LMAX´MMAX AND LMAX´NMAX IS THE

NUMBER OF SURFACE ELEMENTS OF EACH SIDE OF

THE ACTIVE VOLUME. NORMALLY 10´10´10

GIVES A SUFFICIENT PRECISION OF THE

NUMERICAL INTEGRATION. IF YOU ARE NOT SURE,

TRY TO ESTABLISH A BETTER GRID BY TESTING A

SMALL DATA SET(USE HIGH ANGLE REFLECTIONS

FOR THIS CHECK)

FOURTH:   = 0 THE BACKGROUND HAS BEEN SUBSTRACTED FROM THE

INTEGRATED INTENSITIES (NORMAL CASE).

= 1 THE BACKGROUND HAS NOT BEEN SUBSTRACTED.

7

FORMAT(1I)

FIRST:      = 0 INCLUDE ALL PHONONS (USUALLY X-RAY CASE)

= 1 INCLUDE ONLY PHONONS SLOWER THAN THE NEUTRON

VELOCITY(NORMAL NEUTRON CASE)

8-13

EACH FORMAT(6F)

HERE GOES THE MATRIX OF ELASTIC CONSTANTS IN UNITS

OF 1012 DYNE/CM2. THE FIRST CARD GIVES THE

FIRST ROW(C11,C12,C13,C14,C15,C16)OF THE MATRIX,

THE SECOND CARD GIVES THE SECOND ROW AND SO ON.

NOTE: THE FULL MATRIX HAS TO BE GIVEN, BUT ONLY

THE UPPER RIGHT CORNER IS USED FOR THE

CALCULATION. 1NM2 = 10DYNECM2

DATAP

Program for calculation of absorption correction  and  quantities  for

extinction and TDS corrections.

Version of January 1992

Institut Laue-langevin

Uni Karlsruhe

INTRODUCTION AND HISTORY

The program calculates absorption corrections using  a  Gaussian  grid

integration.   For further reference see for example P.  Coppens:  The

Evaluation of Absorption and Extinction in  Single  Crystal  Structure

Analysis   in   'Crystallographic   Computing',   ed.    F.R.   Ahmed,

Munksgaard, 1970.

In addition the vectors needed in anisotropic  extinction  corrections

and for the TDS calculation can be obtained.

The vectors in question are:

XN(I),I=1,3:  Components, in angstroms, of the unit vector  normal  to

the  plane  containing  the incident and diffracted beams, given in

the real cell axis A,B and C.

XNSO(I),I=1,3:  Components of unit vector parallel to the diffracted

beam given in the real cell axis A,B and C.

Other quantity of interest is TBAR , which is  the  mean  path  for  a

reflection , weigthed by absorption.

For more detailed definitions  of  directions  see  Coppens,  P.   and

Hamilton  ,  W.C.  (1970) Acta Cryst.  A26 , 71-83 and Becker, P.  and

Coppens, P.  (1974) Acta Cryst.  A30 , 129-153.  To learn  more  about

the formalisms see Nelmes, R.

The program has been tested using  literature  data  by  Flack,  H.D.,

Vincent,  M.G.   and  Alcock, N.W.  Acta Cryst.  (1980) A36 , 682-686.

The usual disclaimers apply.  We can take no resposabilities  for  any

erroneous calculation produced by this program.

The program comes to the ILL from S.U.N.Y., Buffalo (P.Coppens) ,  via

the   University   of   Aarhus   (F.Z.S.Pedersen  and  F.K.   Larsen).

Modifications were done in Berlin (H.  Dietrich), and at the ILL (S.W.

Wilkins).

In case of problems contact M.S.  Lehmann ,ext.  7382, room  210,  ILL

20.

INPUT

Input is in free format (format is indicated for each kind  of  data).

All numbers must be given.

In principle two input data files are needed.  One file  controls  the

running of the program (choice of options, unit cell etc.).  The other

holds the Bragg reflection data as produced by the program COLL5N  (or

others in the same format).

CONTROL DATA

The file controlling the program contains:

1

FORMAT(20A5)

Title :  anything you might want to write

2

FORMAT(7F)

Cell constants and wavelength.  Cell can be real or reciprocal, angles

or  cosines.   If  a  value  is  less  than 1.0, it is assumed to be a

cosine.

3

FORMAT(3(7F/))

Three reflections

h,k,l,theta,omega,chi,phi

exactly as found on COLL5N output or similar.  Note NOT 2theta.

4

FORMAT(6I)

Six control parameters called ICN(I),I=1,6

first:  =0 no absorption correction

=1 absorption correction: crystal boundary planes

described by h,k,l and d in cm. Preferred option.

=2 absorption correction: crystal boundary planes

described by observed setting angles nescessary

to bring plane into diffraction condition plus

d in cm.

=3 absorption correction: crystal boundary planes

described by equation of planes in Seattle

coordinate system; a,b,c and d in cm.

second: =0 no quantities for extinction correction

(only if first number is zero; elsewhere

quantities are always calculated)

=1 quantities nescessary for extinction correction

as used in LINUS. Likewise used in TDS

calculations

=-1 only vector components for extinction, but no

TBAR.

third:  =0 normally

=1 calculate distance between pairs of vertices.

vertex cards follow boundary plane cards

fourth: =0 normally, four-circle with observed setting

angles given on data file with reflections

=-1 no angles are given, so four-circle bisecting

geomtry is assumed

=1 normal beam (experimental part of program)

=2 for data from STOE-PICKER:  Needs angles on

input data file and STOE-PICKER UB on

card *** 3 *** for orientation . This option  works

o.k. also for non-bisecting orientations.

(NB: STOE-PICKER-UB  transformed into BUSING&LEVY-UB

by signs changed in line 2 and 3)

fifth:  =0 normally. Output of corrected data on a file

=-1 no output of reflection data

sixth:  =N for setting angles check. If this (ICN(6)) is N

then below must follow N cards with h,k,l.

The program will give the angles assuming

bisecting geometry.

5 OPTIONAL

FORMAT(3F)

ICN(6) cards for angles check.  Each card holds h,k,l.

6

FORMAT(2I,2F)

first:  first reflection number to be treated

second: last reflection number

third:  theta minimum to be considered

fourth: theta maximum to be considered

7 OPTIONAL

FORMAT(4I)

Supply only if ICN(1) > 0, or ICN(2) > 0

Specifications for grid for absorption or  extinction  correction.   A

Gaussian  integration formula is used.  The first three numbers define

the number of grid points  along  the  three  axial  directions.   The

product of the three numbers must be less than 4097, and they must all

be even.  The fourth number is number of bounding planes.  As  a  rule

of thumb use N = 15 * mu * T, where mu is absorption coefficient and T

is thickness in the direction in question.  If you do not believe this

try various grids.

first:  grid along A

second: grid along B

third:  grid along C

fourth: number of planes

8 OPTIONAL

FORMAT(4F)

Supply only if 7 is supplied

One card for each boundary plane, option depending upon the  value  of

ICN(1)  ( card 4, first number).  The number of cards is given in card

7.

ICN(1)=1: The boundary planes are given in terms

of crystallgraphic planes and the perpendi-

cular distances in cm. of these planes from

an arbitrary origin within the crystal.

' h,k,l,d ' is input

ICN(1)=2: The unique observed setting angles omega

-teta, chi and phi nescessary to bring the

bounding plane normal into the diffracting

condition are used, with distance d in cm.

' omega-teta,chi,phi,d ' is input

ICN(1)=3: The coefficients of the equation of

plane:  a*x + b*y + c*z = d

are used

' a,b,c,d ' is input

9 OPTIONAL

FORMAT(2I)

Supply only if ICN(3) > 0

Vertex numbers for two  vertices.   The  program  will  calculate  the

distance  between the two vertices.  Can only be done in second run of

program.  Is intended as check.  Two numbers per line.  If the are the

same, stop reading 9.

10

FORMAT(2F)

first:  absorption coefficient. Unit is reciprocal cm.

second: scale factor to apply to F squared. Usually 1 will do.

11 OPTIONAL

FORMAT(3(3F/))

Transformation Tij of indices and vectors on  output  data,  given  in

three lines.  If the vector (or indices) is x1,x2,x3 then output is

x'1        T11  T12  T13     x1

x'2   =    T21  T22  T23  *  x2

x'3        T31  T32  T33     x3

REFLECTION DATA

The file containing structure factors is  assumed  to  come  from  the

COLL5N  program.   If  you have other sources, conform with the format

This data file might contain several card images per reflection.   The

various types are:

Type1:  h,k,l,Fsquared,sigma, angles, etc.

Type2:  other experimental information.

Type3:  absorption and TDS corrections.

Type4:  the counts, and other junk.

All cards have the reflection number in the first 6 columns.

The DATAP program finds out which kind of  cards  are  available.   It

then adds a card of type 3 if none is there.

OUTPUT

Output is on lineprinter and a file with card images.

The program copies all reflection information, and adds one line  with

the absorption correction.

This line has FORMAT(I6,F9.5,F8.4,F9.5,6F7.4,'ABTDS')

and contains:

Reflection number

Absorption correction factor

TBAR

Alpha, the TDS correction (none=-99.0)

XN(I),I=1,3 see page 2

XNO(I),I=1,3 see page 2

NOTE:  if a card with 'ABTDS' is present, then the program  will  stop

if the absorption correction found is between 0 and 1.

G E N E R A L

THE DIFFERENT PROGRAMS OF THE  PROMETHEUS SYSTEM  ARE CALLED BY A MONITORING PROGRAM. TO TELL THIS  MONITOR  WHICH PROGRAM HAS TO BE CALLED EACH RUN MUST START WITH A  -PROGRAM CALLING CARD- . THE CARDS DESCRIBED IN THE CARD ORDER OF THE CALLED PROGRAM MUST FOLLOW IMMEDIATELY BEHIND THIS  CALLING CARD.

AFTER THE LAST CARD DEMANDED BY THE CARD ORDER, THERE CAN FOLLOW A SECOND  PROGRAM CALLING CARD  IN ORDER TO RUN ANOTHER (OR AGAIN THE SAME) PROGRAM.

THE END OF A RUN MUST BE INDICATED BY A  -FINISH CARD-  TO TELL THE MONITORING PROGRAM THAT NO FURTHER PROGRAM CALL IS WHISHED.

REMARK

THE  PLOT FILE (OUTPUT OF PROGRAM PLOTTE) IS ONLY FINISHED BY AN END OF FILE MARK IF THE MONITORING PROGRAM READS THE  FINISH CARD. SO IF SEVERAL CALLS OF THE PROGRAM PLOTTE ARE EXECUTED DURING ONE RUN THESE PLOTS ARE WRITTEN BEHIND EACH OTHER ON THE SAME  PLOT FILE. (E.G. IT IS LIKE ONE BIG PLOT JOB).

PROGRAM CALLING CARDS

FORMAT (A4,A2)

THE FOLLOWING  PROGRAM CALLING CARDS  CAN BE RECOGNICED BY THE PROMETHEUS SYSTEM VERSION OF DECEMBER 1982

ARBSEC

AVERAG

BONVIB

FOURIE

LISTFC

PDFMAP

PLOTTE

POTCAL

PREDES

REFINE

SPHERE

TDSCOR

THE LABELS MUST BE WRITTEN IN COLUMNS l-6.

THE PROGRAMS AND THEIR INPUT CARDS ARE DESCRIBED UNDER THESE LABELS IN THE  DESCRIPTION  AND THE CARD ORDER PART.

FINISH CARD

FORMAT (A4,A2)

THE FINISH CARD CONTAINING THE WORD  FINISH  IN COLUMNS 1-6

INDICATES THE END OF A RUN OF THE PROMETHEUS SYSTEM.

A R B S E C

CARD ORDER SUMMARY OF THE PROGRAM  ARBSEC

TITLE                     CARD

CONTROL                   CARD

INPUT-OUTPUT              CARD

SECTION                   CARD

* CELL                    CARD

*   THIS CARD IS OPTIONAL

FORMAT (18A4)

COLUMN NAME    FUNCTION

1-72  TITEL   ANY SEQUENCE OF SYMBOLS USED AS TITLE OF THE PROGRAM.

CONTROL CARD

FORMAT (4Il,7I3)

COLUMN NAME    FUNCTION ANO P055IBLE VALUE5

1 IO           SWITCH FOR SAVE THE CALCULATED MAP FOR LATER PLOTTING

(WITH THE PROGRAM PLOTTE ).

0 DO NOT SAVE THE MAP.

NOT 0 SAVE THE MAP.

2  NP    SWITCH FOR NO PRINTING OF THE CALCULATED POINT ARRAY

0      PRINT THE POINT ARRAY.

1      DO NOT PRINT THE POINT ARRAY.

3  I5PC     SWITCH FOR SINGLE- OR DOUBLE SPACED PRINTED OUTPUT.

0      DOUBLE SPACED LINES.

1      SINGLE SPACED LINES.

4  MRK      SWITCH FOR READ NEW LIMITS FOR THE AXIS OF THE MAP

TO BE CALCULATED.

0      THE LIMITS OF THE AXIS PRINTED DOWN THE PAGE IN

THE INPUT SECTIONS WILL BE THE LIMITS OF THE

AXIS PRINTED ACROSS THE PAGE (TRANSFORMED X)

IN THE SELECTED ARBITRARY SECTION.

THE LIMITS OF THE AXIS GOING FROM SECTION TO

SECTION OF THE INPUT SECTIONS WILL BE THE

LIMITS OF THE AXIS PRINTED DOWN THE PAGE

TRANSFORMED Y) IN THE SELECTED ARBITRARY

SECTION.

NOT 0  READ NEW LIMITS FOR  TRANSFORMED X  AND

TRANSFORMED  Y  FROM THIS CARD.

(SEE THE FOLLOWING VARIABLES IYI,IYF,IZI,IZF).

5 - 7 IZO        NUMBER OF THE SECTION

(NEEDED TO RECOGNIZE THE SECTION WHEN A CONTOUR MAP

IS PLOTTED (PROGRAM  PLOTTE ).

THE FOLLOWING FOUR VALUES ARE ONLY READ IF  MRK  IS NOT  0

8-10  IYI     INITIAL VALUE OF  TRANSFORMED X .

11-13  IYF     FINAL   VALUE OF  TRANSFORMED X .

14-16  IZI     INITIAL VALUE OF  TRANSFORMED Y .

17-19  IZF     FINAL   VALUE OF  TRANSFORMED Y .

20-22  MINI    IN THE PRINTED POINT ARRAY ALL VALUES BETWEEN  MINI

23-25  MAXI   AND  MAXI  ARE SUPPRESSED.

INPUT-OUTPUT CARD

FORMAT (4I3)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

l - 3  IALP    SHEER ANGLE OF THE PRINTED POINT ARRAY.

0     DO NOT SHEER THE OUTPUT

l     THE REAL ANGLE BETWEEN THE AXIS CALCULATED BY

THE PROGRAMME WILL BE APPROXIMATED BY THE SHEER

ANGLE.

GREATER

THAN 30  THE VALUE OF  IALP  WILL BE THE APPROXIMATED

BY THE SHEER ANGLE.

(THE VALUE MUST BE LESS THAN 151).

4 - 6  LESE    SWITCH FOR READ THE  CELL CARD.

(READING NEW CELL PARAMETERS CAN BE NECESSARY IF IN

THE PROGRAMME REFINE NOT THE EXACT CELL CONSTANTS

HAVE BEEN USED).

0      TAKE THE DATA OF THE UNIT CELL FROM THE

DATA FILE.

1      READ THE DATA OF THE UNIT CELL FROM THE  CELL

CARD.

7 - 9  IEIN    LOGICAL NUMBER OF THE INPUT FILE.

0      THE DEFAULT VALUE  ll  WILL BE USED.

NOT 0  IEIN IS THE LOGICAL NUMBER OF THE INPUT FILE.

10 -12 IAUS    LOGICAL NUMBER OF THE OUTPUT FILE.

0       THE DEFAULT VALUE 12 WILL BE USED.

NOT 0   IAUS IS THE LOGICAL NUMBER OF THE OUTPUT FILE.

SECTION CARD

FORMAT (9F7.0I5)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1-63   POINTS  COORDINATES OF THE POINTS DEFINING THE SECTION TO BE CALCULATED.

1-7            X

8-14           Y                     FOR THE FIRST POINT.

15-21          Z

22-28          X

29-35          Y                     FOR THE SECOND POINT.

36-42          Z

43-49          X

50-56          Y                     FOR THE THIRD POINT.

57-63          Z

64-68          INDICATOR FR FRACTIONAL COORDINATES

0        THE COORDINATES OF THE THREE POINTS ARE GIVEN

IN THE GRID COORDINATES DEFINED IN THE

PROGRAMME WHICH HAS WRITTEN IN THE INPUT FILE.

NOT 0    THE COORDINATES ARE GIVEN IN FRACTIONAL

COORDINATES OF THE UNIT CELL.

CELL CARD

(ONLY NEEDED IF THE PARAMETER  LESE  ON THE INPUT-OUTPUT CARD

IS  1 . IT MAY BE NECESSARY TO READ THE CELL CARDS HERE, IF

INACCURATE VALUES HAVE BEEN GIVEN TO THE PROGRAM  REFINE ,

BECAUSE ACCURATE VALUES WERE NOT NEEDED.)

FORMAT (6F7.0)

COLUMN NAME    FUNCTION

1- 7 	A	LENGTH OF THE A-AXIS OF THE UNIT CELL IN ANGSTROMS.

8-14 	B	LENGTH OF THE B-AXIS OF THE UNIT CELL IN ANGSTROMS.

15-21 	C	LENTH OF THE C-AXIS OF THE UNIT CELL IN ANGSTROMS.

22-28  ALPHA ANGLE BETWEEN  THE  B-  AND  THE  C-AXIS IN DEGREES.

29-35  BETA  ANGLE BETWEEN  THE  A-  AND  THE  C-AXIS IN DEGREES.

36-42  GAMMA ANGLE BETWEEN  THE  A-  AND  THE  B-AXIS IN DEGREES.

CARD ORDER SUMMARY OF THE PROGRAMME AVERAG

LABEL           CARD

TITLE           CARD

CONTROL         CARD  ++

* PARAMETER       CARD  ++

* INPUT FORMAT    CARD  ++

* OUTPUT FORMAT   CARD  ++

* SCALE FACTOR    CARD  ++

* MATRIX          CARD  ++

* CELL            CARD  ++

* HKL             CARDS

END             CARD

*   THESE CARDS ARE OPTIONAL

++  THE SEQUENCE OF THESE CARDS IS ARBITRARY

LABEL CARD

FORMAT (A4,A2,1X,I1)

COLUMN NAME    FUNCTION

1- 6  LABEL   LABEL TO BE WRITTEN ON THE OUTPUT CARDS

8  ICV     SET TO NON-ZERO IF HKL-CARDS ARE NOT COLL5 TYPE

TITLE CARD

FORMAT (18A4)

COLUMN NAME    FUNCTION

1-72  TITEL   ANY SEQUENCE OF SYMBOLS TO BE PRINTED AS TITLE ON

THE OUTPUT.

CONTROL CARD

FORMAT (A4,1X,A4,A1,12I3)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 4  IBUFF   LABEL FOR IDENTIFYING THE CARD AS TO BE THE CONTROL

CARD.

CNTR   LABEL OF THE CONTROL CARD

6-10  ICHAR   NAME OF THE CRYSTAL CLASS TO BE USED IN THE AVERAGING

PROCESS. (LIST OF THE NAMES AT END OF FILE)

11-13  IKART   SWITCH FOR HKL-CARDS OR FOUIER-FILE TO BE READ.

0     READ HKL-CARDS

1     READ OBSERVED F-VALUES FROM A FOURIER-FILE

-1     READ CALCULATED F-VALUES FROM A FOURIER-FILE

IF IKART = 1 THE INTENSITIES ARE CORRECTED FOR

EXTINCTION USING THE CORRECTION FACTORS READ FROM THE

FOURIER FILE.

14-16  IFHKL   VERSION NUMBER OF THE HKL-CARDS TO BE READ (SEE TABLE)

NOT USED FOR FOURIER OR COLL5 TYPE HKL-CARD FILES

17-19  IGNO    SWITCH FOR CHANGE THE IGNORANCE FACTOR (SEE

DESCRIPTION PART).

0      NO CHANGE

NOT 0  CHANGE THE IGNORANCE FACTOR ACCORDING TO THE

VALUES OF DIGN1 AND DIGN2 ON THE  PARAMETER

CARD.

20-22  ISYS    SWITCH FOR CALCULATE A SYSTEMATIC ERROR SIGMA(S)

0      NO SYSTEMATIC ERROR TO BE CALCULATED

NOT 0  CACULATE A SYSTEMATIC ERROR USING THE VALUE OF

SYSDEV  ON THE  PARAMETER CARD.

23-25  ISST    SWITCH FOR DIFFERENT STANDARD DEVIATIONS TO BE WRITTEN

ONTO THE OUTPUT FILE.

0      WRITE  SIGMA(T) (DEFINITION OF THE DIFFERENT

SIGMA SEE DESCRIPTION PART OF THE PROGRAMME

AVERAG) ONTO THE OUTPUT FILE.

-N     IF THERE ARE MORE THAN OR EQUAL TO N

SYMMETRICALLY EQUIVALENT REFLECTIONS WRITE

SIGMA(D)  ONTO THE OUTPUT FILE, IF LESS THAN N

REFLECTIONS WRITE SIGMA(T)

+N     LIKE +N  BUT TAKE  SIGMA(D) ONLY IF IT IS

GREATER THAN  SIGMA(T).

26-28  MODU    SWITCH FOR SELECTION OF PREAVERAGING POSSIBILITIES

0      NO PREAVERAGING

1      AVERAGE FIRST ONLY IDENTICAL REFLECTIONS THEN

THE SYMMETRY OF THE CRYSTAL CLASS .

2      AVERAGE FIRST IDENTICAL REFLECTIONS, THEN

FRIEDEL-REFLECTIONS AND AT LAST THE SYMMETRY

OF THE CRYSTAL CLASS .

3      AVERAGE FIRST IDENTICAL- PLUS FRIEDEL-REFLEC-

TIONS AND THEN THE SYMMETRY OF THE CRYSTAL

CLASS .

29-31  IREJEC  SWITCH FOR DIFFERENT REJECT POSSIBILITIES

0      DO NOT REJECT ANY REFLECTION

+1     A REFLECTION WILL BE REJECTED IF THEIR INTEN-

SITY DEVIATES MORE THAN  REJE*SIGMA(T) (SEE

DESCRIPTION PART) FROM THE MEAN VALUE OF THE

GROUP.

(REJE MUST BE GIVEN ON THE  PARAMETER CARD

-1     A REFLECTIONS WILL BE REJECTED IF THEIR INTEN-

SITY IS MORE THAN  REJE*SIGMA(C)  LESS THAN

THE HIGHEST INTENSITY WITHIN THE GROUP.

(SIGMA(C) IN THIS CASE IS THE STANDARD

DEVIATION OF THE STRONGEST REFLECTION OF THIS

GROUP).

+2,-2  LIKE +1 OR -1 RESPECTIVELY BUT REFLECTIONS

WILL ONLY BE REJECTED IN THE SELECTED PRE-

AVERAGING PROCESSES.

+3,-3  LIKE +1 OR -1 RESPECITVELY BUT REFLECTIONS WILL

NOT BE REJECTED IN PREAVERAGING PROCESSES BUT

ONLY WHEN AVERAGING THE SYMMETRY OF THE CRYSTAL

CLASS .

32-34  NOR     SWITCH FOR WHAT TO DO, IF WITHIN A GROUP OF REFLEC-

TIONS ALL BUT ONE ARE REJECTED.

0      THE MEAN VALUE WILL BE REJECTED TOO.

1      IN A SELECTED PREAVERAGING PROCESS THE MEAN

VALUE WILL BE REJECTED TOO, BUT WHEN AVERAGING

THE SYMMETRY OF THE CRYSTAL CLASS THE REMAINING

REFLECTION WILL BE TREATED AS MEAN VALUE.

2      WHEN PREAVERAGING THE REMAINING REFLECTION WILL

BE TREATED AS MEAN VALUE, BUT WHEN AVERGING

THE SYMMETRY OF THE CRYSTAL CLASS THE MEAN

VALUE WILL BE REJECTED .

3      THE REMAINING REFLECTION WILL ALWAYS BE TREATED

AS MEAN VALUE.

35-37  NULL    SWITCH FOR SET NEGATIVE INTENSITIES TO ZERO

0      AFTER AVERGING NEGATIVE INTENSITIES WILL BE SET

TO ZERO.

NOT 0  THEY WILL NOT BE SET TO ZERO.

38-40  IABSO   SWITCH FOR CORRECT THE INTENSITIES FOR ABSORPTION

0      NO ABSORPTION CORRECTION

> 0    INTENSITIES AND STANDARD DEVIATIONS WILL BE

MULTIPLIED BY THE ABSORPTION FACTORS READ FROM

THE HKL-CARDS.

(IF THERE IS NO TBAR ON CARDS SEE  MU  ON THE

PARAMETER CARD).

< 0   PSI-DATA TREATMENT

NO ABSORPTION CORRECTION. PSI-VALUES FOUND

ON THE INPUT-FILE ARE USED ON THE HKL-FILE

(NOTE. NO SORTING ON SIN(THETA)/LAMBA IS

POSSIBLE)

41-43  IPSO    SWITCH FOR THE USE OF THE QUICKSORT PROCESS AND

FOR AVERAGING OR ONLY SORTING.

(THE USE OF QUICKSORT SAVES CPU-TIME BUT CHANGES THE

SEQUENCE OF THE REFLECTIONS. ONLY SORTING MEANS THAT

THE REFLECTIONS WILL BE WRITTEN ONTO THE OUPUT FILE

WITHOUT AVERAGING BUT IN GROUPS OF SYMMETRICALLY

EQUIVALENT REFLECTIONS).

(ABSORPTION OR EXTINCTION CORRECTION IS MADE ALSO

WHEN  ONLY SORTING  IS SELECTED).

IPSO        QUICKSORT USED          ONLY SORTING

0               YES                     NO

1                NO                     NO

2               YES                    YES

3                NO                    YES

44-46  ILIST   SWITCH FOR DIFFERENT OUPUT VERSIONS.

0      COMPLETE OUTPUT TO BE PRINTED

1      NO LIST OF REFLECTIONS

2      NO LIST OF THE DATA READ BUT LIST OF THE GROUPS

OF SYMMETRICALLY EQUIVALENT REFLECTIONS.

3      PRINT ONLY THE AVERAGED REFLECTIONS

PARAMETER CARD

FORMAT (A4,1X,8F8.3)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1 -4  IBUFF   LABEL FOR IDENTIFYING THE CARD TO BE THE PARAMETER

CARD.

PARA   LABEL OF THE PARAMETER CARD.

6-13  DIGN1   IGNORANCE FACTOR INCLUDED IN THE SIGMA(C) READ

(ONLY NEEDED IF THE PARAMETER  IGNO  ON THE CONTROL

CARD IS NOT 0 ).

(FOR IGNORANCE FACTOR AND SIGMA(C) SEE DESCRIPTION

PART).

14-21  DIGN2   NEW VALUE FOR THE IGNORANCE FACTOR

(ONLY NEEDED IF  IGNO  IS NOT  0 ).

22-29  SYSDEV  ESTIMATED SYSTEMATIC ERROR IN PERCENT OF INTENSITY.

(ONLY NEEDED IF THE PARAMETER  ISYS  ON THE CONTROL

CARD IS NOT 0 ).

30-37  REJE    MULTIPLE OF SIGMA(T) OR SIGMA(C) RESP. TO BE USED FOR

REJECTING REFLECTIONS.

(ONLY NEEDED IF THE PARAMETER  IREJEC  ON THE CONTROL

CARD IS NOT 0 ).

38-45  MU      ABSORPTION COEFFICIENT

(ONLY NEEDED IF THE PARAMETER  IABSO  ON THE CONTROL

CARD IS NOT 0 AND TBAR IS NOT READ FROM CARDS).

46-53  SIGLT   MULTIPLE OF THE STANDARD DEVIATION BELOW WHICH AN

INTENSITY SHOULD BE MARKED AS  LESS THAN .

0      THE DEFAULT VALUE 3.0 IS USED

NOT 0  SIGLT IS THE MULTIPLE TO BE USED

FOR THIS MARKING THE STANDARD DEVIATION WHICH IS

WRITTEN ONTO THE OUTPUT FILE IS USED.

54-61  OFLOW   WITH THIS VALUE ALL THE INTENSITIES AND SIGMA(C)

(SEE DESCRIPTION PART) ARE MULTIPLIED JUST AFTER

READING THE DATA.

(HELPFUL FOR EXAMPLE IF THERE ARE VERY HIGH

ABSORPTION FACTORS WHICH LEAD TO AN OVERFLOW OF THE

OUPUT FORMAT.)

62-69  FINTCU  AVERAGE ONLY THE REFLECTIONS WITH INTENSITIES LESS

THAN FINTCU * SIGMA(INTENSITY).  IF FINTCU=0, THEN

ALL REFLECTIONS ARE AVERAGED.

INPUT FORMAT CARD

(THIS CARD IS ONLY NEEDED IF THE HKL-CARDS CAN NOT BE READ USING

STANDARD FORMAT (SEE HKL-CARD DESCRIPTION) ).

FORMAT (A4,1X,16A4,A3)

COLUMN NAME    FUNCTION

1- 4  IBUFF   LABEL FOR IDENTIFYING THE CARD TO BE THE INPUT FORMAT

CARD.

FORI   LABEL OF THE INPUT FORMAT CARD.

6-72  IPFOR   STANDARD FORTRAN NOTATION OF THE FORMAT TO BE USED

READING THE HKL-CARDS.

(THE FORMAT MUST START WITH  (A3,  IN ORDER TO

RECOGNIZE THE END CARD (SEE BELOW) ).

(THE FORMAT MUST CORRESPOND WITH THE VERSION OF HKL-

CARDS SELECTED BY THE PARAMETER  IFHKL  ON THE

CONTROL CARD).

OUTPUT FORMAT CARD

(ONLY NECESSARY IF THE HKL-CARDS ON THE OUPUT FILE SHOULD NOT HAVE

STANDARD FORMAT).

FORMAT (A4,1X,4A1,1X,4A1)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 4  IBUFF   LABEL FOR IDENTIFYING THE CARD TO BE THE OUPUT FORMAT

CARD.

FORO   LABEL OF THE OUTPUT FORMAT CARD.

6- 9  IRMTI   NUMBER PART OF THE F-FORMAT FOR THE INTENSITY OUTPUT.

(THE  .  MUST BE IN COLUMN7 OR 8)

11-14  IRMTS   NUMBER PART OF THE F-FORMAT FOR THE STANDARD DEVIA-

TION OUTPUT.

(THE  .  MUST BE IN COLUMN12 OR 13)

EXAMPLE

FORO 10.2 7.3    MEANS THAT THE OUTPUT FORMAT IS

F10.2  FOR THE INTENSITY AND

F7.3   FOR THE STANDARD DEVIATION.

ATTENTION

IF THIS CARD IS READ ALWAYS BOTH FORMATS MUST BE GIVEN, IRMTI AND

IRMTS.

SCALE FACTOR CARD

(ONLY NECESSARY IF THERE ARE MORE THAN ONE SCALE GROUP IN THE

DATA SET).

FORMAT (A4,1X,8F8.3)

COLUMN NAME    FUNCTION

1- 4  IBUFF   LABEL FOR IDENTIFYING THE CARD TO BE THE SCALE FACTOR

CARD.

SCAL   LABEL OF THE SCALE FACTOR CARD.

6-69  SCLFAK  SCALE FACTORS FOR THE DIFFERENT SCALE GROUPS IN THE

SEQUENCE OF INCREASING SCALE GROUP NUMBERS USING

EIGHT COLUMNS FOR EACH FACTOR.

(THE INTENSITIES WILL BE CHANGED SO THAT THE SCALE

FACTOR OF THE FIRST REFLECTION READ IS VALUABLE FOR

THE WHOLE DATA SET.)

MATRIX CARD

(ONLY NECESSARY IF THE INDICEES OF THE REFLECTIONS SHOULD BE

TRANSFORMED).

FORMAT (A4,1X,9F8.5)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 4  IBUFF   LABEL FOR IDENTIFYING THE CARD TO BE THE SCALE FACTOR

CARD.

MATR   LABEL OF THE MATRIX CARD

6-53  TRAFO   ELEMENTS OF A 3*3 MATRIX LINE BY LINE USING EIGHT

COLUMNS FOR EACH ELEMENT.

EACH HKL-VECTOR WILL BE MULTIPLIED WITH THIS

MATRIX BEFORE AVEARGING. (IF A NON INTEGER VALUE

FOR ONE OF THE INDICES RESULTS THE REFLECTION

WILL BE REJECTED WITH A WARNING).

CELL CARD

(ONLY NECESSARY IF AFTER AVERAGING THE DATA SHOULD BE SORTED

FOR INCREASING  SIN(THETA) / LAMBDA).

FORMAT (A4,1X,6F8.5)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 4  IBUFF   LABEL FOR IDENTIFYING THE CARD TO BE THE CELL CARD.

CELL   LABEL OF THE CELL CARD.

6-53  CELL    CELL PARAMETERS  A, B, C, ALPHA, BETA, AND GAMMA

USING EIGHT COLUMNS FOR EACH VALUE.

(A,B,C IN ANGSTROMS AND THE ANGLES IN DEGREES).

HKL CARDS

(ONLY NECESSARY HERE IF IKART ON THE CONTROL CARD IS 0 AND THEY

ARE NOT READ FROM A FILE (PARAMETER INUNIT ON THE LABEL CARD) ).

STANDARD FORMAT (ALTERNATIVE VALUES BELOW DEPENDING ON THE SELECTED

VERSION).

(A3,10X,3I3,3X,I3,I3,F9.0,F9.5,F5.0,7X,F8.4)

3X      9X           8X

TABEL OF VERSIONS ACCEPTED BY THE PROGRAMME

VERSION  LABEL  H,K,L  LESS THAN  SCALE  INTEN-  TBAR  SIGMA  ASTAR

NUMBER                  MARKING   GROUP   SITY

1        X      X        X               X             X

2        X      X        X               X      X      X

3        X      X        X        X      X             X

4        X      X        X        X      X      X      X

5        X      X        X               X             X     X

6        X      X        X               X      X      X     X

7        X      X        X        X      X             X     X

8        X      X        X        X      X      X      X     X

EXPLANATION OF THE TABLE

LABEL        ONLY FOR IDENTIFICATION OF THE  END  CARD.

(IF THE LABEL OF THE CARD IS NOT  END  IT WILL BE TAKEN

AS A HKL CARD).

H,K,L        MILLER INDECES

LESS THAN    1  NO  LESS THAN  REFLECTION

MARKING      2  LESS THAN REFLECTION

SCALE GROUP  NUMBER OF THE SCALE GROUP TO WHICH THE REFLECTIONS

BELONGS.

INTENSITY    IF THE VERSION NUMBER IS POSITIVE   INTENSITY  (I(OBS))

IF THE VERSION NUMBER IS NEGATIVE   STRUCTUR FACTOR (F)

TBAR         MEAN PATH LENGTH

SIGMA        STANDARD DEVIATION

IF THE VERSION NUMBER IS POSITIVE   OF  I(OBS)

IF THE VERSION NUMBER IS NEGATIVE   OF  F

ASTAR        ABSORPTION FACTOR

REMARK

THE OUTPUT CARDS OF THE AVERAGING PROGRAMME ARE WRITTEN ACCORDING

TO THE VERSION NUMBER  4  RESPECTIVELY (IF ABSORPTION FACTORS ARE

READ BUT IABSO = 0 (SEE CONTROL CARD) ) ACCORDING TO VERSION NO. 8 .

END CARD

(THIS CARD MUST ALWAYS BE THE LAST CARD BEFORE THE  FINISH  CARD OR

THE CALLING CARD FOR THE NEXT PROGRAMME)

FORMAT (A3)

COLUMN NAME    FUNCTION AND POSIBLE VALUES

1- 3  IBUFF   LABEL FOR IDENTIFYING THE CARD TO BE THE END CARD.

END    LABEL OF THE END CARD.

THE END CARD IS NECESSARY FOR RECOGNIZING THE END OF THE HKL-CARDS IF

THESE ARE READ FROM THE STANDARD INPUT UNIT RESPECTIVELY FOR RECOG-

NIZING THE END OF THE INPUT CARDS IF THE HKL-CARDS ARE READ FROM A

FILE OR A FOURIER FILE IS READ.

NOTATION OF THE LAUE AND CRYSTAL CLASSES

TO BE WRITTEN LEFT JUSTIFIED IN THE FORMAT A4,A1 ON THE CONTROL

CARD.

TRICLINIC

SPACE GROUP       CRYSTAL CLASS      LAUE CLASS       NOTATION

P 1        (1)    1                                    5 BLANKS

P -1       (2)    -1                  -1               -1

MONOCLINIC

SPACE GROUP       CRYSTAL CLASS      LAUE CLASS       NOTATION

C-AXIS PERPENDICULAR

P 2        (3)

P 21       (4)

B 2        (5)    2                                    2/MZA

P M        (6)

P B        (7)

B M        (8)

B B        (9)    M                                    2/MZB

P 2/M     (10)

P 21/M    (11)

B 2/M     (12)

P 2/B     (13)

P 21/B    (14)

B 2/B     (15)    2/M                 2/M              2/MZ

B-AXIS PERPENDICULAR

P 2        (3)

P 21       (4)

C 2        (5)    2                                    2/MYA

P M        (6)

P C        (7)

C M        (8)

C C        (9)    M                                    2/MYB

P 2/M     (10)

P 21/M    (11)

C 2/M     (12)

P 2/C     (13)

P 21/C    (14)

C 2/C     (15)    2/M                 2/M              2/MY

ORTHORHOMBIC

SPACE GROUP       CRYSTAL CLASS      LAUE CLASS       NOTATION

P 222     (16)

P 221     (17)

P 21212   (18)

P 212121  (19)

C 2221    (20)

C 222     (21)

F 222     (22)

I 222     (23)

I 212121  (24)    222                                  MMM A

P MM2     (25)

P MC21    (26)

P CC2     (27)

P MA2     (28)

P CA21    (29)

P NC2     (30)

P MN21    (31)

P BA2     (32)

P NA21    (33)

P NN2     (34)

C MM2     (35)

C MC21    (36)

C CC2     (37)

A MM2     (38)

A BM2     (39)

A MA2     (40)

A BA2     (41)

F MM2     (42)

F DD2     (43)

I MM2     (44)

I BA2     (45)

I MA2     (46)    MM2                                  MMM B

P MMM     (47)

P NNN     (48)

P CCM     (49)

P BAN     (50)

P MMA     (51)

P NNA     (52)

P MNA     (53)

P CCA     (54)

P BAM     (55)

P CCN     (56)

P BCM     (57)

P NNM     (58)

P MMN     (59)

P BCN     (60)

P BCA     (61)

P NMA     (62)

C MCM     (63)

C MCA     (64)

C MMM     (65)

C CCM     (66)

C MMA     (67)

C CCA     (68)

F MMM     (69)

F DDD     (70)

I MMM     (71)

I BAM     (72)

I BCA     (73)

I MMA     (74)    MMM                 MMM              MMM

TETRAGONAL

SPACE GROUP       CRYSTAL CLASS      LAUE CLASS       NOTATION

P 4       (75)

P 41      (76)

P 42      (77)

P 43      (78)

I 4       (79)

I 41      (80)    4                                    4/M A

P -4      (81)

I -4      (82)    -4                                   4/M B

P 4/M     (83)

P 42/M    (84)

P 4/N     (85)

P 42/N    (86)

I 4/M     (87)

I 41/A    (88)    4/M                 4/M              4/M

P 422     (89)

P 4212    (90)

P 4122    (91)

P 41212   (92)

P 4222    (93)

P 42212   (94)

P 4322    (95)

P 43212   (96)

I 422     (97)

I 4122    (98)    422                                  4/MMA

P 4MM     (99)

P 4BM    (100)

P 42CM   (101)

P 42NM   (102)

P 4CC    (103)

P 4NC    (104)

P 42MC   (105)

P 42BC   (106)

I 4MM    (107)

I 4CM    (108)

I 41MD   (109)

I 41CD   (110)    4MM                                  4/MMB

P -42M   (111)

P -42C   (112)

P -421M  (113)

P -421C  (114)

I -42M   (121)

I -42D   (122)    -42M                                 4/MMC

P -4M2   (115)

P -4C2   (116)

P -4B2   (117)

P -4N2   (118)

I -4M2   (119)

I -4C2   (120)    -4M2                                 4/MMD

P 4/MMM  (123)

P 4/MCC  (124)

P 4/NBM  (125)

P 4/NNC  (126)

P 4/MBM  (127)

P 4/MNC  (128)

P 4/NMM  (129)

P 4/NCC  (130)

P 42/MMC (131)

P 42/MCM (132)

P 42/NBC (133)

P 42/NNM (134)

P 42/MBC (135)

P 42/MNM (136)

P 42/NMC (137)

P 42/NCM (138)

I 4/MMM  (139)

I 4/MCM  (140)

I 41/AMD (141)

I 41/ACD (142)    4/MMM               4/MMM            4/MMM

TRIGONAL

SPACE GROUP       CRYSTAL CLASS      LAUE CLASS       NOTATION

RHOMBOHEDRAL SETTING

R 3      (146)    3                                    -3RHA

R -3     (148)    -3                  -3               -3RHO

R 32     (155)    321                                  -3M A

R 3M     (160)

R 3C     (161)    3M1                                  -3M B

R -3M    (166)

R -3C    (167)    -3M1                -3M1             -3M

HEXAGONAL SETTING

P 3      (143)

P 31     (144)

P 32     (145)

R 3      (146)    3                                    -3HEA

P -3     (147)

R -3     (148)    -3                  -3               -3HEX

P 312    (149)

P 3112   (151)

P 3212   (153)    312                                  -31MA

P 31M    (157)

P 31C    (159)    31M                                  -31MB

P -31M   (162)

P -31C   (163)    -31M                -31M             -31M

P 321    (150)

P 3121   (152)

P 3221   (154)

R 32     (155)    321                                  -3M1A

P 3M1    (156)

P 3C1    (158)

R 3M     (160)

R 3C     (161)    3M1                                  -3M1B

P -3M1   (164)

P -3C1   (165)

R -3M    (166)

R -3C    (167)    -3M1                -3M1             -3M1

HEXAGONAL

SPACE GROUP       CRYSTAL CLASS      LAUE CLASS       NOTATION

P 6      (168)

P 61     (169)

P 65     (170)

P 62     (171)

P 64     (172)

P 63     (173)    6                                    6/M A

P -6     (174)    -6                                   6/M B

P 6/M    (175)

P 63/M   (176)    6/M                 6/M              6/M

P 622    (177)

P 6122   (178)

P 6522   (179)

P 6222   (180)

P 6422   (181)

P 6322   (182)    622                                  6/MMA

P 6MM    (183)

P 6CC    (184)

P 63CM   (185)

P 63MC   (186)    6MM                                  6/MMB

P -6M2   (187)

P -6C2   (188)    -6M2                                 6/MMC

P -62M   (189)

P -62C   (190)    -62M                                 6/MMD

P 6/MMM  (191)

P 6/MMC  (192)

P 63/MCM (193)

P 63/MMC (194)    6/MMM               6/MMM            6/MMM

CUBIC

SPACE GROUP       CRYSTAL CLASS      LAUE CLASS       NOTATION

P 23     (195)

F 23     (196)

I 23     (197)

P 213    (198)

I 213    (199)    23                                   M3  A

P M3     (200)

P N3     (201)

F M3     (202)

F D3     (203)

I M3     (204)

P A3     (205)

I A3     (206)    M3                  M3               M3

P 432    (207)

P 4232   (208)

F 432    (209)

F 4132   (210)

I 432    (211)

P 4332   (212)

P 4132   (213)

I 4132   (214)    432                                  M3M A

P -43M   (215)

F -43M   (216)

I 43M    (217)

P -43N   (218)

F -43C   (219)

I -43D   (220)    -43M                                 M3M B

P M3M    (221)

P N3N    (222)

P M3N    (223)

P N3M    (224)

F M3M    (225)

F M3C    (226)

F D3M    (227)

F D3C    (228)

I M3M    (229)

I A3D    (230)    M3M                 M3M              M3M

CARD ORDER SUMMARY OF THE PROGRAM  BONVIB

TITLE           CARD

CONTROL         CARD

CELL VAR.-COVAR.CARDS

NO CARD IS OPTIONAL

TITLE CARD

FORMAT (18A4)

COLUMN NAME    FUNCTION

1-72  TITEL   ANY SEQUENCE OF SYMBOLS USED AS TITLE OF THE PROGRAM.

CONTROL CARD

FORMAT (3I1,3F8.0)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1  MBODAN  BOND DISTANCE, ANGLE SELECTOR

0      NONE COMPUTED

1      DISTANCES ONLY

2      DISTANCES AND ANGLES COMPUTED

2  MELVIB  THERMAL ELLIPSOID SELECTOR

0      NO ELLIPSOIDS COMPUTED

1      THERMAL ELLIPSOIDS COMPUTED FOR ANISOTRIPIC

ATOMS

3  MTENSA  HIGHER ORDER ANALYSIS SELECTOR

0      NO ANALYSIS

1      ANALYSIS OF HIGHER ORDER THERMAL TENSORS

4-11  DLIMIT(1)      MINIMUM BOND DISTANCE OUTPUT

12-19  DLIMIT(2)      MAXIMUM BOND DISTANCE USED IN ANGLES OUTPUT

20-27  DLIMIT(3)      MAXIMUM BOND DISTANCE OUTPUT

CELL VAR.-COVAR.CARDS

(THIS 4 CARDS CONTAIN THE CELL VARIANCE-COVARIANCE MATRIX)

REMARK

THIS MATRIX CAN NOT BE CALCULATED BY THE PROMETHEUS SYSTEM UP TO NOW.

IT IS COMPUTED E.G. BY THE  PARAM PROGRAM  OF THE XRAY-SYSTEM

(STEWARD ET. AL. 1967).

YOU CAN GIVE 0 RESP. BLANKS ON THESE CARDS. THIS MEANS THAT THE

EFFECT OF ERRORS IN THE CELL CONSTANTS ON THE DISTANCES ARE

NEGLECTED.

CARD 1

FORMAT (6E12.6)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1-12          VARIANCE OF A (IN ANGSTROMS2)

13-24          COVARIANCE OF A WITH B

25-36          COVARIANCE OF A WITH C

37-48          COVARIANCE OF A WITH ALPHA (IN ANGSTROMS-RADIANS)

49-60          COVARIANCE OF A WITH BETA

61-72          COVARIANCE OF A WITH GAMMA

CARD 2

FORMAT (6E12.6)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1-12          VARIANCE OF B

13-24          COVARIANCE OF B WITH C

25-36          COVARIANCE OF B WITH ALPHA

37-48          COVARIANCE OF B WITH BETA

49-60          COVARIANCE OF B WITH GAMMA

61-72          VARIANCE OF C

CARD 3

FORMAT (6E12.6)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1-12          COVARIANCE OF C WITH ALPHA

13-24          COVARIANCE OF C WITH BETA

25-36          COVARIANCE OF C WITH GAMMA

37-48          VARIANCE OF ALPHA (IN RADIANS2)

49-60          COVARIANCE OF ALPHA WITH BETA

61-72          COVARIANCE OF ALPHA WITH GAMMA

CARD 4

FORMAT (6E12.6)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1-12          VARIANCE OF BETA

13-24          COVARIANCE OF BETA WITH GAMMA

25-36          VARIANCE OF GAMMA

CARD ORDER SUMMARY OF THE PROGRAMME  FOURIE

TITLE           CARD

INPUT-OUTPUT    CARD

* ANGLE SELECTION CARD(S)

* HKL SELECTION   CARDS

* PATTERSON TITLE CARD

* CELL            CARD

* CELL TYPE       CARD

* SYMMETRY        CARDS

* FORMAT          CARD

* HKL             CARDS

* END             CARD

CONTROL         CARD

* MULTIPLICITY    CARD

*   THESE CARDS ARE OPTIONAL

TITLE CARD

FORMAT (18A4)

COLUMN NAME    FUNCTION

1-72  TITEL   ANY SEQUENCE OF SYMBOLS USED AS TITLE OF THE PROGRAMME

INPUT-OUTPUT CARD

FORMAT (8I3)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 3  ISELEC  SHEER ANGLE OF THE PRINTED POINT ARRAY

0      THE REAL ANGLE OF THE AXES LAYING IN THE

PRINTED SECTION WILL BE THE SHEER ANGLE.

NOT 0  THE VALUE OF ISELEC WILL BE THE SHEER ANGLE.

4- 6  ISTAT   SWITCH FOR PRINT THE REFLECTIONS USED FOR CALCULATION.

0      DO NOT PRINT THE REFELCTIONS.

NOT 0  PRINT FOR ALL REFLECTIONS USED THE FOLLOWING

VALUES

H,K,L,F(OBS),F(CALC),A(CALC),B(CALC),PHASE.

(H,K,L    MILLER INDICES,

F(OBS)   OBSERVED   STRUCTURE FACTOR

F(CALC)  CALCULATED STRUCTURE FACTOR

A(CALC)  REAL     PART OF F(CALC)

B(CALC)  IMGINARY PART OF F(CALC)

PHASE    PHASE ANGLE OF F(CALC) IN DEGREES.

7- 9  IFILE   LOGICAL NUMBER OF THE INPUT FILE (FOURIER-FILE).

0      THE DEFAULT VALUE  13  WILL BE USED.

NOT 0  IFILE IS THE LOGICAL NUMBER OF THE INPUT FILE.

ATTENTION

THIS FILE IS ALSO NEEDED IF HKL-CARDS ARE READ (IPAT NOT 0). IN THIS

CASE THE FILE IS TEMPORARILY USED TO STORE THE DATA INFORMATION.

10-12  IAUSG   LOGICAL NUMBER OF THE OUTPUT FILE.

0      THE DEFAULT VALUE  11  WILL BE USED.

NOT 0  IAUSG IS THE LOGICAL NUMBER OF THE OUTPUT FILE.

13-15  IPAT    SWITCH FOR READ HKL-CARDS INSTEAD OF A FOURIER FILE

AND CALCULATE A PATTERSON SYNTHESIS.

(THE FOURIER FILE IN THIS CASE IS CREATED BY THE

FOURIE  PROGRAMME ITSELF).

0      THE DATA ARE READ FROM A FOURIER FILE.

NOT 0  READ HKL-CARDS AND CALCULATE A PATTERSON

SYNTHESIS.

ATTENTION

IF HKL-CARDS ARE READ NO ANGLE SELECTION IS POSSIBLE AND THE

PARAMETER IPART IS SET TO 0.

REMARK

A PATTERSON SYNTHESIS CAN ALSO BE CALCULATED FROM A FOURIER FILE,

WITH HKL-CARDS HOWEVER ONLY A PATTERSON SYNTHESIS CAN BE

CALCULATED.

16-18  IPART   SWITCH FOR READ ANGLE SELECTION CARDS OR HKL SELECTION

CARDS.

0      READ NO SELECTION CARDS.

NEG.   READ ANGLE SELECTION CARD(S).

POS.   READ HKL   SELECTION CARDS.

19-21  IMINI   IN THE PRINTED POINT ARRAY ALL VALUES BETWEEN IMINI

22-24  IMAXI   ANS IMAXI ARE SUPPRESSED.

ANGLE SELECTION CARD(S)

(THIS CARD IS ONLY READ IF THE PARAMETER  IPART  ON THE INPUT-OUTPUT

CARD IS LESS THAN 0 ).

FORMAT (2F8.5,I3)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 8  SINMIN  ONLY THOSE REFLECTIONS WILL BE USED FOR CALCULATION

9-16  SINMAX  FOR WHICH  SIN(THETA) / LAMBDA  IS GREATER THAN SINMIN

AND LESS THAN SINMAX.

17-19  IMORE   SWITCH FOR READ THE SECOND ANGLE SELECTION CARD.

0      THIS IS THE LAST ANGLE SELECTION CARD.

NOT 0  THE FOLLOWING CARD IS AGAIN AN ANGLE SELECTION

CARD.

(AT MOST THERE CAN BE 2 ANGLE SELECTION CARDS).

HKL SELECTION CARDS

(THIS CARDS ARE ONLY READ IF THE PARAMETER  IPART  ON THE INPUT-

OUTPUT CARD IS GREATER THAN 0 ).

FORMAT (A3,10X,3I3)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 3  IMEHR   SWITCH FOR READ ONE MORE HKL SELECTION CARD.

END     THIS IS THE LAST HKL SELECTION CARD.

NOT END THE FOLLOWING CARD IS AGAIN A HKL SELECTION

CARD.

14-16  IHKL    USE THE REFLECTION WITH THE

17-19  JHKL    MILLER INDICES IHKL,JHKL, AND KHKL

20-22  KHKL    FOR THE CALCULATION.

ATTENTION

IN THE PRESENT VERSION THE PROGRAMME CAN READ AT MOST  100  HKL

SELECTION CARDS.

PATTERSON TITEL CARD

(THIS CARD IS ONLY NECESSARY IF THE PARAMETER  IPAT  ON THE INPUT-

OUTPUT CARD IS NOT ZERO).

FORMAT (3A4)

COLUMN NAME    FUNCTION

1-12  LABEL   ANY SEQUENCE OF SYMBOLS USED AS LABEL OF THE FOURIER

FILE TO BE CREATED.

CELL CARD

(ONLY NECESSARY IF  IPAT  IS NOT ZERO)

FORMAT (6F8.0)

COLUMN NAME    FUNCTION

1- 8  A       LATTICE CONSTANTS

9-16  B       GIVEN IN

17-24  C       ANGSTROMS.

25-32  ALPHA   LATTICE ANGLES

33-40  BETA    GIVEN IN

41-48  GAMMA   DEGREES.

CELL TYPE CARD

(ONLY NECESSARY IF  IPAT  IS NOT ZERO)

FORMAT (2I3)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 4  ICENT   FLAG FOR CETRIC OR ACENTRIC CELL

0      CENTRIC  CELL

1      ACENTRIC CELL

4- 6  ICELL   INDICATOR FOR THE TYPE OF THE CELL

0      PRIMITIVE

1      A- CENTERED

2      B- CENTERED

3      C- CENTERED

4      I- CENTERED

5      F- CENTERED

6      RHOMBOHEDRAL

SYMMETRY CARDS

(ONLY NECESSARY IF  IPAT  IS NOT ZERO)

FORMAT (3(3A3,1X),I4)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 3  JTNS    TRANSLATIONAL      PART OF  X .

HERE CAN APPEAR ONE OF THE OPERATORS DESCRIBED BELOW

OR  END  TO INDICATE THAT ALL SYMMETRY CARDS  WERE

READ.

5- 6  IRMAT   FIRST  POSITIONAL  PART OF  X .

8- 9  IRMAT   SECOND POSITIONAL  PART OF  X .

11-13          TRANSLATIONAL      PART OF  Y .

15-16          FIRST  POSITIONAL  PART OF  Y .

18-19          SECOND POSITIONAL  PART OF  Y .

21-23

25-26          SAME FOR  Z .

28-29

30-33  IEF     END OF SYMMTERY CARDS INDICATOR

0       THE NEXT CARD IS A SYMMETRY CARD

NOT 0   THIS IS THE LAST SYMMETRY CARD (THE CARD

ITSELF MUST ALSO CONTAIN SYMMETRY INFORMATION)

THE TRANSLATIONAL OPERATORS MUST BE OF THE FOLLOWING FORM (B = BLANK)

BBB, 1/2, 1/3, 2/3, 1/4, 3/4, 1/6, OR 5/6 .

THE POSITIONAL OPERATOR MUST BE OF THE FORM (W = X, Y, OR Z)

+W, BW (=+W), OR -W.

ATTENTION

THE SYMMETRIY POSITION  X Y Z  MUST BE INCLUDED. THE MAXIMUM

NUMBER OF SYMMETRY CARDS IS 24. SYMMETRY OPERATIONS INCLUDING

THE INVERSION OR A TRANSLATION DUE TO LATTICE CENTERING ARE

CREATED BY THE PROGRAM AND MUST NOT BE INCLUDED HERE.

FORMAT CARD

(ONLY NECESSARY IF  IPAT  IS NOT ZERO)

FORMAT (18A4)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1-72  FRMT    INDICATOR FOR THE FORMAT OF THE HKL-CARDS.

1- 4          (A3,   THIS BEGINNING IS NECESSARY TO RECOGNIZE THE

END CARD AFTER THE LAST HKL-CARD

5-72                 REST OF THE FORMAT IN STANDARD FORTRAN NOTATION

THE DATA ON A HKL-CARD CAN ONLY BE READ IN THE

SEQUENCE DEFINED IN  HKL CARDS  BELOW.

HKL CARDS

(ONLY NECESSARY IF  IPAT  IS NOT ZERO).

FORMAT (A3, (REST DEFINED IN THE FORMAT CARD ABOVE)  )

THE DATA ARE READ FROM EACH CARD IN THE FOLLOWING SEQUENCE

FLAG,H,K,L,MREJ,F(OBS),SIGMA

FLAG   VARIABLE TO RECOGNIZE THE  END  CARD  (SEE DESCRIPTION BELOW)

H,K,L  MILLER INDECES

MREJ   MARK FOR  LESS THAN  REFLECTIONS (REJECTION FLAG)

1    NO LESS THAN REFLECTION

2    LESS THAN REFLECTION

F(OBS) INTENSITY

SIGMA  STANDARD DEVIATION OF THE INTENSITY

END CARD

(ONLY NECESSARY IF  IPAT  IS NOT ZERO, BUT THEN IS MUST FOLLOW AFTER

THE LAST HKL CARD).

FORMAT (A3)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 3 FLAG    HKL     THIS IS A HKL CARD

NOT HKL THIS IS THE END CARD AFTER THE LAST HKL CARD

CONTROL CARD

FORMAT (7I1,1X,9I4,2F8.0,A4)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1  ITYPE   MAP TYPE INDICATOR.

0      FOURIER MAP USING F-OBSERVED.

1      DIFFERENCE FOURIER MAP.

2      FOURIER MAP USING F-CALCULATED.

3      PATTERSON MAP.

4      PROJECTION ON A PLANE GIVEN BY ISECT  FO-MAP

5          "             "     "        "    DELTA

6          "             "     "        "    FC-MAP

REMARK

IF ON THE  INPUT,OUPUT CARD  THE PARAMETER  IPAT  IS NOT ZERO

ITYPE IS SET TO BE 3 .

2  ISECT   SELECTION FLAG FOR DIFFERENT ORIENTATIONS OF THE

SECTION(S) TO BE CALCULATED.

ACROSS     DOWN    SECTION(S)   UP  OR  DOWN

THE PAGE  THE PAGE                (SEE BELOW)

0         X         Y          Z            DOWN

1         X         Z          Y             UP

2         Y         X          Z             UP

3         Y         Z          X            DOWN

4         Z         Y          X             UP

5         Z         X          Y            DOWN

UP   = THE SECTION COORDINATE INCREASES WITH INCREASIN

SECTION NUMBER

DOWN   THE SECTION COORDINATE DECREASES WITH INCREASIN

SECTION NUMBER.

(THIS MEANS THAT THE SYSTEM IS ALWAYS RIGHT HANDED).

3  ICENT   CENTRIC INDICATOR

(FOR SUBSTANCES WITH CENTRIC CELL AND HIGH ANOMALOUS

DISPERSION IT IS NECESSARY HERE, TO DECLARE AN

ACENTRIC CELL, BECAUSE IN THE  FOURIE PROGRAMME FOR

CENTRIC CELLS THE IMAGINARY PART OF THE STRUCTURE

FACTOR IS ALWAYS NEGLECTED).

0      CENTRIC

1      ACENTRIC

4  KREJ    SWITCH FOR INCLUDE THE REFLECTIONS WITH REJECTION

FLAG SET IN THE CALCULATION.

(REJECTION FLAG  SEE HKL-CARD DESCRIPTION).

0      IGNORE REJECTION FLAG AND INCLUDE ALL

REFLECITONS.

1      DO NOT INCLUDE ANY REFLECTIONS WITH REFLECTION

FLAG SET.

5  IPLT    SWITCH FOR SAVE THE CALCULATED MAP FOR LATER PLOTTING

(PLOTTE) OF CALCULATION OF AN ARBITRARY SECTION

(ARBSEC).

0      DO NOT SAVE THE MAP.

NOT 0  SAVE THE MAP ON UNI

CARD).

6  LSPC    SWITCH FOR SINGLE- OR DOUBLE SPACED PRINTED OUPUT.

0      DOUBLE SPACED LINES.

1      SINGEL SPACED LINES.

7  NPNT    SWITCH FOR PRINT THE CALCULATED POINT ARRAY.

0      PRINT THE POINT ARRAY

1      DO NOT PRINT THE POINT ARRAY

9-12  IX      NUMBER OF DIVISIONS ALONG THE X-AXIS.

13-16  IXI     INITIAL POINT ALONG THE X-DIRECTION.

17-20  IXF     FINAL   POINT ALONG THE X-DIRECTION.

21-24  IY      NUMBER OF DIVISIONS ALONG THE Y-AXIS.

25-28  IYI     INITIAL POINT ALONG THE Y-DIRECTION.

29-32  IYF     FINAL   POINT ALING THE Y-DIRECTION.

33-36  IZ      NUMBER OF DIVISIONS ALONG THE Z-AXIS.

37-40  IZI     INITIAL POINT ALONG THE Z-DIRECTION.

41-44  IZF     FINAL   POINT ALONG THE Z-DIRECTION.

45-52  EXXAG   EXAGGERATION FACTOR (OUTPUT VALUES ARE MULTIPLIED

BY EXXAG).

53-60  F000    NUMBER OF ELECTRONS IN THE UNIT CELL, NEEDED FOR

FOURIER AND PATTERSON SYTHESES.

(CAN BE OMITTED DIFFERENCE FOURIER IS CALCULATED).

61-64  IWAHL   SWITCH FOR READ OR CALCULATE MULTIPLICITIES.

CALC      CALCULATE THE MULTIPLICITIES

NOT CALC  READ MULTIPLICITIES FROM THE FOLLOWING

MULTIPLICITY CARD

65-72  FMGNR   MULTIPLICITY OF GENERAL REFLECTION

MULTIPLICITY CARD

(ONLY NECESSARY IF THE PARAMETER  IWAHL  ON THE CONTROL CARD IS

NOT EQUAL CALC)

FORMAT (12F6.0)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 6  FMULT   MULTIPLICITY OF   H00  OR  0K0

7-12          MULTIPLICITY OF   00L

13-18          MULTIPLICITY OF   HK

19-24          MULTIPLICITY OF   H0L

25-30          MULTIPLICITY OF   0KL

31-36          MULTIPLICITY OF   HH

37-42          MULTIPLICITY OF   H0H

43-48          MULTIPLICITY OF   0KK

49-54          MULTIPLICITY OF   HHH

55-60          MULTIPLICITY OF   HHL

61-66          MULTIPLICITY OF   HKK  OR  HKH

67-72          MULTIPLICITY OF   HKL

L I S T F C

CARD ORDER SUMMARY OF THE PROGRAMME  LISTFC

LABEL               CARD

TITLE               CARD

CONTROL             CARD

* HKL                 CARD(S)

*   THESE CARDS ARE OPTIONAL

LABEL CARD

FORMAT (3A4,A3)

COLUMN    NAME            FUNCTION

1-12      LABEL           ANY SEQUENCE OF SYMBOLS USED AS PROGRAMME TITLE

13-15     INPUT           BLANK    INPUT IS VIA FOURIER-FILE

HKL      INPUT IS VIA STANDARD HKL-CARDS

TITLE CARD

FORMAT (10A4)

COLUMN       NAME           FUNCTION

1- 40        TITLE          ANY SEQUENCE OF SYMBOLS USED AS PROGRAM TITLE

CONTROL CARD

FORMAT (3Il,I4,Al,I3,5(1X,Il),3(1X,Al),9(lX,Il).2X.

2(1X,Il),2F8.0)

COLUMN      NAME          FUNCTION

1           ISORH         (1)/(2)/(3) FOR H INDEX VARIES (MOST)/(NEXT MOST)/(LEAST)

RAPIDLY

(CODE TO TELL HOW INPUT DATA ARE   SORTED)

2           ISORK           SORTING FREQUENCY FOR K INDEX

3           ISORL           SORTING FREQUENCY FOR L INDEX

4- 7        LINE            NUMBER OF LINES PER *LISTFC* PAGE

8           IAPAGE         (BLANK)/(A) FOR (DO)/(ABSOLUTELY DO NOT) INTERNALLY

CHANGE THE VALUE OF THE NUMBER OF LINES IN ORDER TO

MAKE THE BOTTOM OF THE LAST PAGE AS EVEN AS POSSIBLE

9-11        ICOLPP          NUMBER OF LIST COLUMNS PER *LISTFC* PAGE

N O T E   THE PRODUCT OF THE NUMBER OF LINES

AND THE NUMBER OF COLUMNS PER *LISTFC* PAGE

CANNOT

EXCEED 3000.  THAT IS, NO MORE THAN 3000 FOR THE TOTAL

OF

REFLECTIONS AND HEADINGS (INCLUDING ALL SPACES), PER

PAGE.

PREFATORY NOTICE PAY ATTENTION - ZERO (OR BLANK) IN ANY

OF THE NEXT FIVE FIELDS MEANS JUST THAT...

13     IBLNK         NUMBER OF BLANK PRINT COLUMNS BEFORE THE LISTFC COLUMN

15     IIN           NUMBER OF PRINT COLUMNS FOR THE MOST RAPIDLY

CHANGING INDEX

17     IFOC          NUMBER OF PRINT COLUMNS FOR FO TIMES 10

19     IFCC          NUMBER OF PRINT COLUMNS FOR FC TIME5 10

21     IPHC          NUMBER OF PRINT COLUMNS FOR SIGFO TIMES 10

23     IFLLT         SPECIAL FLAG FOR LESS THANS (BLANK=*)

25     IFLEX         SPECIAL FLAG FOR EXTINCT REFLECTIONS (BLANK=E)

27     IFLSPC        SPECIAL FLAG FOR SPECIAL REFLECTIONS

PREFATORY NOTICE THE NEXT NINE FIELDS ARE (BLANK)/(l) FOR {DO NOT)/(DO)

29     ISIGNO        PRINT SYMBOLS FOR LESS THAN OR EXTINCT REFLECTIONS

THIS SYMBOL SWITCH ADDS ONE MORE PRINT COLUMN

31     ISIGNC        FOR CENTRIC STRUCTURES ATTACH SIGN OF A TO FC

33     ISPACE        DOUBLE SPACE THE LINES

35     IRESTO        RESTORE EACH *LISTFC* PAGE TO BE THE TOP OF A

PRINTER PAGE

37     ITIT          PRINT CURRENT TITLE AT TOP OF EACH PRINTER PAGE

39     IPNCH         PUNCH A SET OF =FCARD= (HAS H,K,L,FO,FC,A,B,JCODE,

AND LEVEL INDICATORS) PUNCH UNIT AT PRESENT

41     NOT USED

43     ISIGS         PRINT MINUS SIGN ON THE FO OF THE REFLECTIONS

WHICH ARE LESS THANS

45     ITWOT         WRITE A SEPERATE COPY OF THE FC LIST ON UNIT

49     ITRY          NUMBER OF TIMES TO TRY TO GET HEADINGS AT THE TOP

CARD ORDER SUMMARY OF THE PROGRAMME  PDFMAP

TITLE           CARD

CONTROL         CARD

OUTPUT          CARD

* SPECIAL PLANE   CARDS

ATOM SELECTION  CARD

*   THESE CARDS ARE OPTIONAL

TITLE CARD

FORMAT (17A4)

COLUMN NAME    FUNCTION

1-68  TITEL   ANY SEQUENCE OF SYMBOLS USED AS PROGRAMME TITLE

CONTROL CARD

FORMAT (5I1,1X,6I1,10I5,F6.3,I1)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 5  IPDFF   SELECTION FLAGS FOR ORDERS OF THERMAL PARAMETERS

TO BE INCLUDED IN THE PDF- OR POTENTIAL-CALCULATION.

0      DO NOT INCLUDE THIS ORDER

1      INCLUDE THIS ORDER

1          SECOND ORDER  OF THERMAL PARAMETERS.

2          THIRD  ORDER

3          FOURTH ORDER

4          FIFTH  ORDER

5          SIXTH  ORDER

7  IPDF    SELECTION FLAG FOR CALCULATE A NORMAL OR AN EXTENDED

EDGEWORTH MAP.

(EXTENDED MAP IS ONLY MEANINGFUL FOR THE EDGEWORTH

EXPANSION).

0      CALCULATE A NORMAL MAP.

1      CALCULATE AN EXTENDED MAP.

8  ISECT   SELECTION FLAG FOR DIFFERENT ORIENTATIONS OF THE

SECTION(S) TO BE CALCULATED.

ACROSS     DOWN    SECTION(S)   UP  OR  DOWN

THE PAGE  THE PAGE                (SEE BELOW)

0         X         Y          Z            DOWN

1         X         Z          Y             UP

2         Y         X          Z             UP

3         Y         Z          X            DOWN

4         Z         Y          X             UP

5         Z         X          Y            DOWN

UP   = THE SECTION COORDINATE INCREASES WITH INCREA-

SING SECTION NUMBER

DOWN   THE SECTION COORDINATE DECREASES WITH INCREA-

SING SECTION NUMBER.

(THIS MEANS THE SYSTEM IS ALWAYS RIGHT HANDED).

9  IPLT    SWITCH FOR SAVE THE CALCULATED MAP FOR LATER PLOTTING

(PROGRAM  PLOTTE ).

0      DO NOT SAVE THE MAP

1      SAVE THE MAP

NB: THE VAX-VMS VERSION WILL CREATE SEVERAL

FILES FOR LATER PLOTTING:

XXXX_PD.S2  CONTAINS THE PDF

XXXX_PD.S4  CONTAINS THE ERROR-PDF

XXXX_PD.S6  CONTAINS THE SIGNIFICANCE-MAP

10  LSPC    SWITCH FOR SINGLE- OR DOUBLE SPACED PRINTED OUPUT.

0      DOUBLE SPACED LINES.

1      SINGLE SPACED LINES.

11  NPNT    SWITCH FOR PRINT THE CALCULATED POINT ARRAY.

0      PRINT THE POINT ARRAY.

1      DO NOT PRINT THE POINT ARRAY.

12  IOR     SWITCH FOR DIFFERENT POSTIONS OF THE ATOM FOR WHICH

THE PDFMAP IS CALCULATED WITHIN THE SECTION.

0      ATOM POSITION AT THE GRID COORDINATES  0,0,0 .

1      ORIGIN OF THE CELL AT THE GRID COORDINATES

0,0,0 .

13-17  IX      NUMBER OF DIVISIONS ALONG THE X-AXIS.

18-22  IXI     INITIAL POINT ALONG THE X-DIRECTION.

23-27  IXF     FINAL   POINT ALONG THE X-DIRECTION.

28-32  IY      NUMBER OF DIVISIONS ALONG THE Y-AXIS.

33-37  IYI     INITIAL POINT ALONG THE Y-DIRECTION.

38-42  IYF     FINAL   POINT ALONG THE Y-DIRECTION.

43-47  IZ      NUMBER OF DIVISIONS ALONG THE Z-AXIS.

(IF IZ=0  THE  SPECIAL PLANE CARDS  WILL BE READ AND

THE PDF (OR POTENTIAL) WILL BE CALCULATED ONLY IN THE

PLANE DEFINED BY THESE CARDS).

48-52  IZI     INITIAL POINT ALONG THE Z-DIRECTION.

53-57  IZF     FINAL   POINT ALONG THE Z-DIRECTION.

58-62  MC      NUMBER OF MONTE-CARLO LOOPS FOR ERROR CALCULATION

(IF MC = 0 NO ERRORS WILL BE CALCULATED;

IF MC > 0 ONLY ONE PLANE OR ONE SECTION MAY BE

CALCULATED)

A SAVE NUMBER FOR MC USUALLY IS 1000.

63-66  SIFA    SIGNIFICANCE LIMIT FOR MONTE-CARLO SIGNIFICANCE MAPS

(SEE EQ.A6 IN W.F.KUHS ACTA CRYST.A (1992).

67-69  ISQ     0   MEAN SQUARE DEVIATIONS ARE CALCULATED (DEFAULT)

1   POSITIVE AND NEGATIVE DEVIATIONS ARE CALCULATED

SEPARATELY

OUTPUT CARD

FORMAT (F9.0,3I5,I2,1X,I2,I5,F8.0,A1,F8.0)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES.

1- 9  EXXAG   EXAGGERATION FACTOR (OUTPUT VALUES ARE MULTIPLIED

BY  EXXAG ).

10-14  IMINI   IN THE PRINTED OUTPUT ALL VALUES BEWTEEN IMINI AND

15-19  IMAXI   IMAXI ARE SUPPRESSED.

20-24  ISELEC  SHEER ANGLE OF THE PRINTED POINT ARRAY.

0      APPROXIMATION OF THE REAL CRYSTAL ANGLE.

NOT 0  ISELEC IS THE VALUE OF THE ANGLE IN DEGREES TO

BE APROXIMATED.

25-26  IPLU    LOGICAL UNIT NUMBER OF THE PLOT-FILE TO BE WRITTEN

(DEFAULT VALUE  11).

28-29  MAIN    LOGICAL UNIT NUMBER OF THE MAIN-FILE TO BE READ

(DEFAULT VALUE  10).

30-34  ICCH    SELECTION FLAG FOR TREATEMENT OF THE MIXED PRODUCT OF

THE THIRD ORDER TENSOR IN THE EDGEWORTH EXPANSION.

(ONLY NEEDED IF THE EDGEWOTH EXPANSION IS SELECTED).

0      INCLUDE THE TERM

C(I,J,K) * C(P,Q,R) * H(I,J,K,P,Q,R)

(EINSTEIN SUMMATION CONVENTION USED).

1      DO NOT  INCLUDE  THIS TERM.

2      DO ONLY INCLUDE  THIS TERM.

35-42  T       TEMPERATURE AT WHICH THE DATA HAVE BEEN MEASURED.

0.     CALCULATE THE PROBABILITY DENSITY

FUNCTION (PDF).

NOT 0. INSTEAD OF THE PDF THE  POTENTIAL  WILL BE CAL-

CULATED. THE NORMALISATION IS DONE BY USING THE

MAXIMUM VALUE OF THE  PDF  OF THE FIRST CALCU-

LATED SECTION IF THERE IS NOT GIVEN A VALUE FOR

PNORM (PNORM  SEE BELOW).

43  KELVIN  UNIT OF THE TEMPERATURE  T (ONLY NEEDED IF T IS NOT

0.)

K      TEMPERATURE IS GIVEN IN  KELVIN .

NOT K  TEMPERATURE IS GIVEN IN  CELSIUS.

(0. CELSIUS MUST BE GIVEN AS  273.16 K  BECAUSE

FOR T = 0. THE PDF WOULD BE CALCULATED).

44-51  PNORM   VALUE OF THE  PROBABILITY DENSITY FUNCTION (PDF) TO BE

USED FOR THE NORMALISATION OF THE POTENTIAL (ONLY

NEEDED IF  T  IS NOT 0.)

0.     THE MAXIMUM VALUE OF THE FIRST CALCULATED

SECTION OF THE  PDF  WILL BE USED FOR NORMALI-

SATION.

NOT 0. PNORM WILL BE USED FOR NORMALISATION.

SPECIAL PLANE CARDS

(THESE CARDS ARE ONLY NECESSARY IF THE PARAMETER  IZ  ON THE CONTROL

CARD IS ZERO).

FORMAT (A4,2X,3F8.0)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 6  MTYP    SWITCH FOR DIFFERENT POSSIBILITIES OF DEFINITION OF A

PLANE.

CENTER THE POINT GIVEN ON THIS CARD SHOULD BE THE

CENTER OF THE PLANE CALCULATED.

CORNER THE POINT GIVEN ON THIS CARD SHOULD LAY ON AN

EDGE OF THE PLANE.

7-14  E(1)    X-COORDINATE OF THE POINT IN FRACTIONAL COORDINATES

15-22  E(2)    Y-COORDINATE OF THE POINT IN FRACTIONAL COORDINATES

23-30  E(3)    Z-COORDINATE OF THE POINT IN FRACTIONAL COORDINATES

THERE MUST BE EXACTY  THREE SUCH CARDS .

(ONLY ONE OF THE POINTS CAN BE THE  CENTER  OF THE PLANE).

REMARK

THE FOLLOWING PARAMETERS ARE MEANINGLESS WHEN SPECIAL PLANE CARDS

ARE GIVEN

ON THE CONTROL CARD   ISECT,IOR,IZI, AND IZF.

ON THE OUTPUT  CARD   ISELEC.

ATOM SELECTION CARD

FORMAT (60I1)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1  IPLOT   SWITCH FOR CALCUATION OF THE PDF (OR POTENTIAL)

FOR THE FIRST ATOM

0      DO NOT CALCULATE A PDF (RESP. POTENTIAL)

1      CALCULATE THE PDF (OR POTENTIAL) FOR THE

FIRST ATOM.

2          SAME FOR THE SECOND ATOM

3          SAME FOR THE THIRD  ATOM

4          AND SO ON .

REMARK

THERE CAN BE MORE THAN ONE PARAMETER SET TO 1 .

P L O T T E

CARD ORDER SUMMARY OF THE PROGRAMME  PLOTTE

INPUT-OUTPUT    CARD

TITLE           CARD

LAYOUT          CARD

CONTOUR-LINE    CARD(S)

* LINE DRAWING    CARD(S)

* CROSS           CARD(S)

* PDFMAP          CARD

SECTION         CARD(S)

* LABEL           CARD(S)

* CROSS CONTINUE  CARD(S)

*   THESE CARDS ARE OPTIONAL

INPUT-OUTPUT CARD

FORMAT (A4,2X,4I2)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 4  IWAEHL  INDICATOR FOR THE PROGRAMME WHICH HAS WRITTEN THE DATA

FILE TO BE READ (FILE GENERATING PROGRAMME).

FOUR

PDFM

ARBS

5- 6  INRFIL  LOGICAL NUMBER OF THE INPUT FILE.

0      DEPENDING ON THE VALUE OF IWAEHL THE FOLLOWING

DEFAULT VALUES ARE USED.

IWAEHL           DEFAULT VALUE

FOURIE                11

ARBSEC                12

*** Note for PDFMAP_Files :	  no default value

take  31 for density

33 for Errmaps

35 for Signif.-maps

NOT 0  INRFIL IS THE LOGICAL NUMBER OF THE INPUT FILE.

9-10  IFRAC   INDICATOR FOR THE COORDINATES USED ON THE  LINE

DRAWING CARD(S), THE  CROSS CARD(S), AND THE  LINE

LABEL CARD(S).

0      COORDINATES ARE GIVEN IN GRID COORDINATES.

(ACCORDING TO THE GRID SELECTION IN THE FILE

GENERATING PROGRAMME).

1      COORDINATES ARE GIVEN IN FRACTIONAL COORDINATES

OF THE UNIT CELL, IF SECTIONS PARALLEL TO THE

AXES ARE PLOTTED.

11-12  NRCROS  SWITCH FOR DIFFERENT POSSIBILITIES OF DRAWING CROSSES

ON THE PLOTS.

POS.   FOR EACH PLOT NEW COORINATES FOR THE CROSSES TO

BE DRAWN WILL BE READ FROM A  CROSS (CONTINUE)

CARD.

0      CROSSES WILL ONLY BE DRAWN ON THE FIRST PLOT

AT THE POINTS GIVEN ON THE  CROSS CARD.

NEG.   CROSSES WILL BE DRAWN ON THE NEXT  NRCROS

PLOTS AT THE POINTS GIVEN ON THE  CROSS CARD.

TITLE CARD

FORMAT (8A4)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1-32  ITITLE  ANY SEQUENCE OF SYMBOLS TO BE PRINTED AS A TITLE ON

THE PLOT(S).

*** note for POSTSCRIPT following Charaters difficult to use

"(" , ")" , "\" , "Angstroem"

*** look at the POSTSCRIPT-Manual

LAYOUT CARD

FORMAT (F5.0,I5,3F5.0,4I5,F5.0,I5)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 5  TLEHGT  HEIGHT OF THE LETTERS OF THE TITLE TO BE WRITTEN ON

THE PLOT(S).

NEG.   NO TITLE WILL BE WRITTEN ON THE PLOT(S).

0      THE DEFAULT VALUE OF 1. CM IS USED.

POS.   TLEHGT IS THE HEIGHT OF THE LETTERS IN CM.

REMARK

IF THE TITLE GIVEN ON THE  TITLE CARD  WOULD BECOME LONGER THAN

THE PAPER USING THE SELECTED DIMENSION OF THE LETTERS THIS DIMENSION

IS NOT DECREASED AUTOMATICALLY.

6- 10 NLABL   NUMBER OF  LABEL CARDS  TO BE READ.

11-15  A       THESE TWO VARIABLES ARE USED TO DETERMINE THE

16-20  B       DIMENSION OF THE PLOT(S).

(DEFINITION OF WIDTH AND LENGTH SEE PICTURE BELOW).

A=0, B=0.

FOR THE FIRST PLOT A SCALE FACTOR IS DETERMINED TO

MAKE A = 16. CM . THE SAME FACTOR IS USED FOR B.

FOR THE FOLLOWING PLOTS THE SCALE OF THE FIRST PLOT

WILL BE USED PROVIDED THAT  A  IS LESS THAN 16. CM.

IF  A  WOULD BECOME  GREATER THAN 16 CM A NEW SCALE

FACTOR IS DETERMINED USING THE SAME PROCEDURE AS FOR

THE FIRST PLOT.

A LESS THAN -1        , B ARBITRARY BUT NOT 0    OR

A ARBITRARY BUT NOT 0 , B LESS THAN -1.

A NEW SCALE FACTOR WILL BE CALCULATED USING THE SAME

PROCEDURE AS FOR THE FIRST PLOT IN THE CASE

A=0, B=0 . (USEFUL FOR CHANGING THE SCALE IF THE

CURRENT  LAYOUT CARD  IS NOT FOR THE FIRST PLOT).

A=0, B LESS THAN 0.

S = ABS(B) WILL BE USED AS SCALE FACTOR.

(ABS(B) IS THE SCALE OF THE PLOT IN CM/ANGSTROMS).

A GREATER THAN 0, B = 0              OR

A = 0           , B GREATER THAN 0.

A OR B WILL BE TAKEN AS THE WIDTH OR THE LENGTH OF THE

PLOT IN  CM  RESPECTIVELY AND THE OTHER AXIS WILL BE

PLOTTED IN THE SAME SCALE.

REMARK

WHENEVER THE INPUT LEADS TO A WIDTH (A) GREATER THAN 16. CM THE PLOT

WILL BE SCALED TO  A = 16. CM.

21-25  ALP     ANGLE BETWEEN THE AXES DEFINED IN THE FOLLOWING

PICTURE.

/I

/  I

/    I

X  /      I

/        I

/          I

/            I

/              I

I ALP          /

I            /

I          /

B I        /

I      /

I    /

I  /

I/

I------ A -----I

EXPLANATION OF THE SYMBOLS

X      AXIS WHICH WAS PRINTED ACROSS THE PAGE IN THE

POINT ARRAY OF THE FILE GENERATING PROGRAMME.

B      LENGTH OF THE AXIS PLOTTED IN VERTICAL DIREC-

TION.(IN THIS DESCRIPTION ONLY CALLED  LENGTH

OF THE PLOT).

A      WIDTH OF THE PLOT.

ALP    ANGLE BETWEEN  B  AND  X .

IF ALP IS  0  THE REAL ANGLE BETWEEN  B AND X  WILL BE

PLOTTED.

26-30  NP      NUMBER OF INTERPOLATION POINTS PER SPLINE.

(IF NP IS LESS THAN 6 THE DEFAULT VALUE NP=6 IS USED)

31-35  NCONT   NUMBER OF  CONTOUR-LINE CARDS  TO BE READ.

(THE POSSIBLE MAXIMUM VALUE OF NCONT IS 10)

36-40  NLINE   NUMBER OF  LINE DRAWING CARDS  TO BE READ AND SWITCH

FOR MAKE A FRAME AROUND THE PLOT.

0      MAKE A FRAME

POS.   AROUND THE PLOT.

NEG.   MAKE NO FRAME.

IABS(NLINE) IS THE NUMBER OF  LINE DRAWING CARDS TO BE

READ. (THE POSSIBLE MAXIMUM VALUE OF NLINE IS 20)

41-45  NCROSS  NUMBER OF  CROSS CARDS  TO BE READ. (SEE ALSO THE

PARAMETER  NRCROS  ON THE  INPUT-OUTPUT CARD).

(THE POSSIBLE MAXIMUM VALUE OF NCROSS IS 20).

46-50  SCROSS  DIMENSION OF THE CROSSES.

0      THE DEFAULT VALUE  0.75 CM  IS USED.

NOT 0  SCROSS IS THE DIMENSION IN CM.

51-55  NSECT   NUMBER OF SECTION CARDS TO BE READ

(THE POSSIBLE MAXIMUM VALUE OF NSECT IS 10).

CONTOUR-LINE CARD(S)

(THIS CARD MUST APPEAR  NCONT TIMES (SEE LAYOUT CARD) ).

FORMAT (3F5.0,I5)

COLUMN NAME    FUNCTION

1- 5  SBEG    HEIGHT OF THE INITIAL CONTOUR-LINE (OF THE GROUPE OF

OF LINES DEFINED ON THIS CARD).

6-10  CSTEP   STEP BETWEEN THE CONTOUR-LINES (DEF. ON THIS CARD).

11-15  SLIM    HEIGHT OF THE FINAL CONTOUR LINE (OF THE GROUP OF

LINES DEFINED ON THIS CARD).

16-20  NSIG    NUMBER OF DIGITS AFTER DECIMAL POINT USED FOR WRITING

THE HEIGHT AT EACH CONTOUR LINE.

NEG.   DO NOT WRITE THE HEIGHTS ON THE PLOT.

0      ONLY INTEGER VALUES

POS.   WRITE THE HEIGHTS WITH NSIG DIGITS.

LINE DRAWING CARD(S)

(THIS CARD MUST APPEAR  ABS(NLINE)  TIMES (SEE LAYOUT CARD) ).

FORMAT (2F5.0)

COLUMN NAME    FUNCTION AND POSSSIBLE VALUES

1- 5  XLINE   COORDINATES OF A POINT. LINES ARE DRAWN FROM THE POINT

6-10  YLINE   GIVEN IN THE LAST LINE DRAWING CARD TO THIS ONE (IF

THIS IS NOT THE FIRST CARD) AND FROM THIS POINT TO THE

ONE GIVEN IN THE NEXT LINE DRAWING CARD (IF THIS WAS

NOT THE LAST CARD)

CROSS CARD(S)

(THIS CARD MUST APPEAR  NCROSS  TIMES (SEE LAYOUT CARD) ).

FORMAT (2F5.0)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 5  BX      COORDINATES OF THE POINT, WHERE A CROSS SHOULD BE

6-10  BY      PLOTTED.

(THE SECTION(S) ON WHICH A CROSS WILL BE DRAWN IS

DEFINED BY THE PARAMETER  NRCROS  ON THE INPUT-OUTPUT

CARD. THE COORDINATES MUST BE GIVEN IN THE SYSTEM

DEFINED BY THE PARAMETER  IFRAC ON THE SAME CARD)

SECTION CARD(S)

(THIS CARD MUST APPEAR  NSECT  TIMES (SEE LAYOUT CARD) ).

FORMAT (3I5)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 5  INITZ   FOR THE SECTIONS FROM SECTION NUMBER  INITZ  TO THE

6-10  ISTEPZ  SECTION NUMBER  IFINZ  IN STEPS OF  ISTEPZ  CONTOUR

11-15  IFINZ   MAPS WILL BE PLOTTED.

(THE SECTIONS WERE NUMBERED BY THE FILE GENERATING

PROGRAMME).

LABEL CARD(S)

(THIS CARD MUST APPEAR  NLABL  TIMES (SEE LAYOUT CARD) ).

FORMAT (5F5.0,A6)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 5  WX      COORDINTES OF THE POINT WHICH SHOULD BE MARKED BY

6-10  WY      THE LABEL  LABL  GIVEN ON THIS CARD.

(WX AND WY MUST BE GIVEN IN GRID COORDINATES. WX

IS TAKEN ALONG THE X-AXIS AND WY ALONG THE B-AXIS

DESCRIBED IN THE PICTURE UNDER THE  LAYOUT CARD).

11-15  DX      PLOTTER OFFSET FROM THE POINT DEFINED BY WX AND WY

16-20  DY      FOR STARRING THE LABEL.

(DX AND DY MUST BE GIVEN IN CM. DX IS TAKEN ALONG THE

X-AXIS AND DY ALONG THE B-AXIS DESCRIBED IN THE

PICTURE UNDER THE  LAYOUT CARD).

(IF DX=0. AND DY=0. THE DEFAULT VALUES  DX=0.5 CM  AND

DY=0. CM  ARE USEC).

21-25  HGT     HEIGHT OF THE SYMBOLS OF THE LABEL TO BE PLOTTED.

(IF HGT=0. THE DEFAULT VALUE HGT=0.75 CM  IS USED).

26-31  LABL    ANY SEQENCE OF SYMBOLS TO BE PLOTTED AT THE POINT

AND IN THE DIMENSION DEFINED ON THIS CARD.

CROSS CONTINUE CARD(S)

(THIS CARD IS ONLY NECESSARY IF THE PARAMETER  NRCROS  ON THE

INPUT-OUTPUT CARD IS POSITIV, BUT THEN IT MUST APPEAR

(NRCROS * NCROSS)  TIMES (NCROSS SEE LAYOUT CARD) ).

THIS CARD HAS EXACTLY THE SAME FORM AS THE  CROSS CARD.

P O T C A L

CARD ORDER SUMMARY OF THE PROGRAM  POTCAL

TITLE                       CARD

CONTROL                     CARD

DATA                        CARD

DIRECTION                   CARD

ATOM SELECTION              CARD

TITLE CARD

FORMAT (17A4)

COLUMN   NAME       FUNCTION

1-68     TITLE      ANY HOLLERITH TEXT AS PROGRAM TITLE

CONTROL CARD

FORMAT (10Il)

COLUMN   NAME      FUNCTION AND POSSIBLE VALUES

1        IDIR      0      DIRECTION GIVEN BY FRACTIONAL COORDINATES OF

THE LAST POINT TO BE CALCULATED.  CALCULATION

IS REFERRED EITHER TO THE REFINED ATOM POSITION

OR THE MAXIMUM IN PDF (S.VARIABLE ISRT)

1      AS 0 BUT DIRECTION GIVEN BY A VECTOR (UVW).

2      DIRECTION GIVEN BY A VECTOR (UVW). CALCULATION

STARTS AT A GIVEN POINT (S.VARIABLE

VE ON DIRECTION CARD).

2        ISRT      0      CALCULATION STARTS AT THE REFINED ATOM POSITION,

AT THE MAXIMUM OF PDF OR AT A GIVEN POINT.

1      CALCULATION STARTS AT A GIVEN POINT AND STOPS

AT AN OTHER GIVEN POINT.

2      CALCULATION STARTS IN THE OPPOSITE OF THE GIVEN

DIRECTION, GOES THROUGH THE REFERENCE POINT AND

STOPS AT THE GIVEN LAST POINT.

3        IRNM      0      NO SEARCH FOR LOCAL MAXIMA IN PDF

1       ( SEARCH FOR ) + NO CALCULATION OF BONDS

2       ( LOCAL MAX- ) + CALCULATION OF BOND DISTANCES

3       ( IMA IN PDF ) + CALCUL. OF BONDS AND ANGLES

4       AS 1  ( ADDITIONAL REPLACE THE )

5       AS 2  ( REFINED ATOM POSITION)

6       AS 3  ( BY THE MAXIMUM IN PDF)

7       READ MAXIMUM IN PDF AND NORMALIZE TO THIS VALUE

(S. VARIABLE RNORM ON DATA CARD).

4       IPLT       0       NO PLOTTING OF THE CALCULATED POTENTIAL

1       PLOT AND PRINT THE CALCULATED POTENTIAL

2       ONLY PLOTTING OF THE CALCULATED POTENTIAL

5       IMOM       0       DO NOT CALCULATE MOMENTS AND CUMULANTS

1       CALCULATE MOMENTS AND CUMULANTS NUMERICALLY

ALONG A GIVEN DIRECTION (SEE DIRECTION CARD)

OR

ALONG THE SHIFT VECTOR IF DIRECTION IS NOT

SPECIFIED (VA AND VE EQUAL TO ZERO).

6      IEXT        0       NORMAL EDGEWORTH-SERIES (AND GRAM-CHARLIER)

1       EXTENDED EDGEWORTH-SERIES TO BE CALCULATED

(PREREQUISITE IS THE PRESENCE OF THE HARMONIC DATA ON

UNIT IHARMO).

7      IPRI        0       NO PRINT OF POINTS WITH ZERO OR NEGATIVE PDF

1       PRINT ALL CALCULATED POINTS

2       NO PRINT AT ALL

8      ISGL        0       SIGNIFICANCE LIMIT AUTOMATICALLY CALCULATED AT

THE 1 P.C. POINT.

1       SIGNIFICANCE LIMIT GIVEN BY THE USER (S. VARIABLE SIGLN

ON DATA CARD).

9      IPLO        0       PLOTSCALES GIVEN BY THE USER, PLOTSIZE

ACCORDINGLY (S. VARIABLES GRIDX AND GRIDY ON

DATA CARD).

1       PLOTSIZE GIVEN BY THE USER, PLOTSCALES

ACCORDINGLY (S. VARIABLES GRIDX AND GRIDY ON

DATA CARD).

10     IWHL        0       ENERGY IN THE PLOT IS GIVEN IN EV

1       ENERGY IN THE PLOT IS GIVEN IN 10**-19 J.

DATA CARD

FORMAT (6F8.3)

COLUMN  NAME       FUNCTION AND POSSIBLE VALUES

1- 8    TMPT       TEMPERATURE OF THE DIFFRACTION EXPERIMENT IN K

9-16    GRID       INTERVAL FOR PRINTED OUTPUT OF POTENTIAL IN AE

(DEFAULT = 0.05 AE).

17-24   GRIDX      SCALE FOR DISPLACEMENT (X-AXIS) IN THE PLOT

IN CM/AE (DEFAULT = 10.0 CM/AE)  OR

PLOTSIZE (X-AXIS) IN CM (DEFAULT = 10.0 CM).

25-32   GRIDY      SCALE FOR ENERGY (Y-AXIS) IN THE PLOT IN CM/EV

OR CM/10**-19 J (DEFAULT = 50 CM/EV OR

50 CM/10**-19 J) OR

PLOTSIZE (Y- AXIS) IN CM (DEFAULT = 10.0 CM).

33-40   SIGLN      SIGNIFICANCE LIMIT IN EV OR 10**-19 J (ACCORDING

TO VARIABLE IWHL ON CONTROL CARD). ALL POINTS

ABOVE THIS LIMIT WILL NOT BE PLOTTED.

41-48   RNORM      PDF AT THE MAXIMUM FOR NORMALIZATION OF THE

POTENTIAL (TO SAVE THE TIME FOR REPEATED

SEARCHING OF THE LOCAL MAXIMUM).

DIRECTION CARD

FORMAT (6F8.5)

COLUMN   NAME      FUNCTION AND POSSIBLE VALUES

1- 8     VE(l)     ( COORDINATES OF THE LAST  )  FOR A-DIRECTION

9-16     VE(2)     ( POINT TO BE CALCULATED   )  FOR B-DIRECTION

17-24    VE(3)     ( OR COMPONENT OF (UVW)    )  FOR C-DIRECTION

25-32    VA(l)     ( COORDINATES OF THE FIRST )  FOR A-DIRECTION

33-40    VA(2)     ( POINT TO BE CALCULATED   )  FOR B-DIRECTION

41-48    VA(3)     ( ONLY IF IDIR=2 OR ISRT=l )  FOR C-DIRECTION

ATOM SELECTION CARD

FORMAT (60I.l)

COLUMN   NAME      FUNCTION AND POSSIBLE VALUES

1        ICALC     0     DO NOT CALCULATE POTENTIAL OF THIS ATOM

1     CALCULATE POTENTIAL OF THIS ATOM

ETC                     (SEQUENCE ACCORDING TO THE ORDER IN THE

MAIN UNIT).

P R E D E S

CARD ORDER SUMMARY OF THE PROGRAMME  PREDES

TITLE            	CARD

TEXT            	CARDS

END             	CARD

NO CARD IS OPTIONAL

TITLE CARD

FORMAT (5Al,I3,lX,Al,lX,6Al)

COLUMN   NAME      FUNCTION AND POSSIBLE VALUES

1- 5     IT        THIS COLUMNS MUST CONTAIN THE WORD  TITLE  FOR RECOG-

NITION OF THIS CARD AS A TITLE CARD.

6- 8     NPAGE     STARTING NUMBER FOR NUMERATING THE PRINTED PAGES.

10     IPART     INDICATOR FOR THE PART OF THE SYSTEM DESCRIPTION TO

WHICH THE PRINTED PAGES BELONG.

BLANK    GENERAL (ONLY THE SYMBOLS GIVEN IN COLUMN 12-17

FOR KENN ARE PRINTED IN THE PAGE HEADER.

1        DESCRIPTION (DESCRIPTION OF THE FORMALISMS

USED IN THE PROGRAMM. THE WORD DESCRIPTION IS

INCLUDED IN THE PAGE HEADER).

2        CARD ORDER (CARD ORDER SUMM

(THE WORD CARD ORDER IS INCLUDED IN THE PAGE

HEADER).

12-17    KENN      ANY SEQUENCE OF SYMBOLS TO BE PRINTED ON EACH PAGE AS

A PAGE HEADER.

REMARK

ALTHOUGH THE TITLE CARD ALWAYS MUST BE THE FIRST CARD IT CAN BE

REPEATED BETWEEN THE TEXT CARDS AS OFTEN AS THE PAGE HEADER MUST BE

CHANGED.

TEXT CARDS

FORMAT (Al,71Al)

COLUMN   NAME      FUNCTION AND POSSIBLE VALUES

1        IT        THE SIGN IN THE FIRST COLUMN IS ALWAYS INTERPRETED

AS A MONITORING SIGN AND NOT PRINTED IN THE OUTPUT.

BLANK      NORMAL PRINTING OF THE LINE (COLUMN 2 - 72).

0          LIKE BLANK

1          START A NEW PAGE WITH THIS LINE.

+          PRINT THIS LINE ON THE SAME LINE AS THE ONE

BEFORE. (NO LINEFEED)

L          LEAVE ONE BLANK LINE BEFORE PRINTING THIS LINE.

R          PRINT THIS LINE TWO TIMES WITHOUT LINEFEED.

(IT THEN LOOKS DARKER THAN THE NORMAL TEXT).

2           THE NEXT 4 LINES SHOULD NOT BE SEPARATED BY

STARTING A NEW PAGE. IF THERE ARE LESS THAN 4

LINES LEFT ON THE CURRENT PAGE A NEW PAGE WILL

BE STARTED WITH THIS LINE.

3           SAME AS 2 FOR THE NEXT 6 LINES

4           SAME AS 2 FOR THE NEXT 8 LINES

AND SO

ON

9           SAME AS 2 FOR THE NEXT 18 LINES

A           SAME FOR 20 LINES

B           SAME FOR 22 LINES

C           SAME FOR 24 LINES

D           SAME FOR 26 LINES

END CARD

FORMAT (3Al)

COLUMN   NAME      FUNCTION AND POSSIBLE VALUES

1- 3     IT        IF IN THE FIRST THREE COLUMNS THE WORD  END  IS READ

THIS MEANS THAT ALL THE TEXT CARDS ARE READ AND THE

PRINTING IS NOW FINISHED.

CARD ORDER SUMMARY OF THE PROGRAMME  REFINE

LABEL           CARD

CYCLE START     CARD

MAIN CONTROL    CARD

* CELL            CARD

* BOND LIMITS     CARD

* CELL TYPE       CARD

* SYMMETRY        CARD(S)

* SCAT.CURVE TYPE CARD

* EXPONENT. FORM  CARD

* TABULATION FORM CARDS

* ANOMALOUS DISP. CARD

* SCALE FACTOR    CARD

* EXTINCTION      CARD

* ATOM SELECTION  CARD

* ATOM PARAMETER  CARD(S)

* SPECIAL PARAM.  CARD

* SPECIAL VALUE   CARD(S)

* ALPHA PARAMETER CARD(S)

* T-FACTOR TYPE   CARD

* SCALE REFINE    CARD

* EXTINC. REFINE  CARD

* ALPHA REFINE    CARD

* SPECIAL REFINE  CARD

* ATOM REFINE     CARD(S)

* NEW VALUE       CARD(S)

* DEPENDENCY      CARD(S)

* DIFFRACTOMETER  CARD

* ORIENTATION     CARDS

* FORMAT          CARD

* DATA DEFINING   CARD

* MEASUREMENT     CARD

* MONOCHROMATOR   CARD

* PARTIAL         CARDS

* HKL             CARDS

CONTINUE        CARD

*   THESE CARDS ARE OPTIONAL

LABEL CARD

FORMAT (3A4,2X,I2)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1-12  LABEL   ANY SEQUENCE OF SYMBOLS TO BE USED AS LABEL OF THE

PRINTED OUTPUT AND THE BINARY DATA FILE FOR ALL RUNS

OF THE PROGRAMME REFINE AND ALL FOLLOWING RUNS OF

PROGRAMMES READING A BINARY DATA FILE (FOURIE, PDFMAP,

ETC.).

(IF THE CURRENT CYCLE IS NOT AN -A PRIORI- CYCLE THE

LABEL READ FROM THE -MAINFILE- FILE OVERWRITES THE

LABEL GIVEN HERE).

15-16  ICFILE  LOGICAL NUMBER OF THE -MAINFILE- FROM WHICH RESULTS OF

EARLIER RUNS HAVE TO BE READ IF THIS IS A -CONTINUE-

CYCLE.

0      THE DEFAULT VALUE 10 IS USED.

NOT 0  ICFILE IS THE LOGICAL NUMBER OF THE MAINFILE.

CYCLE START CARD

FORMAT (18A4)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1-72  TITLE   ANY SEQUENCE OF SYMBOLS TO BE USED AS TITLE OF THE

CURRENT CYCLE. IF OTHER PROGRAMMES WILL USE THE

RESULTS OF THIS CYCLE, THIS TITLE IS USED AS A LABEL.

MAIN CONTROL CARD

FORMAT (A1,3I1,1X,5I1,1X,8I1,3F8.0,1X,I1,1X,3I1,1X,I1)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1  IEEF    SWITCH FOR PRINTED OUTPUT

P      PRINT THE OUTPUT ON THE STANDARD OUTPUT UNIT.

T      PRINT ALL INFORMATIONS ONLY ON A TEMPORARY-

FILE (LOGICAL NUMBER 15 ).

2  MFSEL   STRUCTURE FACTOR EQUATION SELECTOR.

0      ONLY STRUCTURE FACTOR AND R-VALUE CALCULATION

FOR STRUCTURE FACTOR EQUATION (SF-EQ.) NO. 1 .

1      LEAST-SQUARES REFINEMENT FOR SF-EQ. NO. 1 .

2      SAME FOR

3      SF-EQ. NO. 2

4      SAME FOR

5      SF-EQ. NO. 3

6      SAME FOR

7      SF-EQ. NO. 4

THE STRUCTURE FACTOR EQUATIONS ARE

NO. 1  FOURIERTRANSFORM OF THE EDGEWORTH EXPANSION.

NO. 2  FOURIERTRANSFORM OF THE GRAM-CHARLIER EXPANSION.

NO. 3  MODIFIED EDGEWORTH EXPANSION.

NO. 4  ALPHA FORMALISM  (MODIFIED GRAM-CHARLIER EXPANSION).

REMARK

A CHANGE OF THE STRUCTURE FACTOR EQUATION USED DURING THE REFINEMENT

IS ONLY POSSIBLE BY SETTING  IOVER  (SEE BELOW) NOT EQUAL 0 .

THEREFORE IN -CONTINUE- CYCLES USING THE SAME SF-EQ. YOU MUST ONLY

CHOSE BETWEEN  MFSEL=0  FOR  STRUCTURE FACTOR CALCULATION  AND

MFSEL=1  FOR  REFINEMENT.

3  MREF    SWITCH FOR DIFFERENT REFINEMENT TYPES.

0      DO NOT CHANGE THE CURRENT REFINEMENT TYPE.

(IN AN -A PRIORI- CYCLE 0 IS OVERWRITTEN BY 1).

1      REFINE R(W) (WEIGHTED R-VALUE) ON F.

(F = STRUCTURE FACTOR)

2      REFINE R(U) (WEIGHTED R-VALUE BUT USING UNIT

WEIGHTS) ON F.

3      REFINE R(W) ON F**2.

4      REFINE R(U) ON F**2.

4  IREJ    SWITCH FOR DIFFERENT REJECT POSSIBILITIES.

0      DO NOT CHANGE THE FORMALISM USED FOR REJECTING

REFLECTIONS.

(IN AN -A PRIORI- RUN  IREJ=0 IS INTERPRETED TO

BE 1 ).

1      INCLUDE ALL REFLECTIONS.

2      REJECT REFLECTIONS MARKED ON THE HKL-CARDS

AS  LESS THAN.

IF DELMAX (THIS CARD COL.36-43) IS NOT EQUAL

0, ALL REFLECTIONS WITH A RATIO I/SIGMA(I)

(OR F/SIGMA(F)) SMALLER THAN DELMAX WILL BE

REJECTED.

3      REJECT ALL REFLECTIONS MARKED AS LESS THAN AND

IN ADDITION ALL REFLECTIONS WITH

ABS(F(OBS)-F(CALC)) GREATER THAN (DELMAX*SIGMA)

EXPLANATION

F(OBS)  = OBSERVED STRUCTURE FACTOR.

F(CALC) = CALCULATED STRUCTURE FACTOR.

DELMAX  = FACTOR WHICH CAN BE GIVEN ON THIS

CARD (SEE BELOW).

IF THIS IS NOT AN -A PRIORI- CYCLE

AND IF IN THIS CYCLE A FOURIER FILE

HAS BEEN CREATED, DELMAX IS COMPUTED

TO BE 9 TIMES THE VALUE FOR WHICH

EXACTLY HALF OF THE REFLECTIONS WOULD

BE REJECTED.

SIGMA   = STANDARD DEVIATION OF F(OBS) IF THE

THE PARAMETER  IWT  ON THE

DATA DEFINING CARD IS NOT EQUAL 2 .

(FOR IWT = 2 SIGMA IS CALCULATED FROM

THE WEIGHTS).

***REMARK***

WHEN REFINING ON  F**2, NEVERTHELESS THE

FUNCTION ABOVE IS CALCULATED USING F AND THE

STANDARD DEVIATION OF F.

4      LIKE 3 BUT NO SPECIAL CONDITION FOR

LESS THAN REFLECTIONS.

5      LIKE 3 BUT NO SPECIAL COMPUTATION OF  DELMAX .

(DELMAX MUST BE GIVEN ON THIS CARD).

6      LIKE 4 BUT NO SPECIAL COMPUTATION OF  DELMAX .

(DELMAX MUST BE GIVEN ON THIS CARD).

7      REJECT ALL REFLECTIONS MARKED AS LESS THAN AND

IN ADDITION THOSE LAYING OUTSIDE THE INTERVAL

OF SIN(THETA)/LAMBDA DEFINED BY THE PARAMETERS

UNTEN  AND  OBEN  (SEE BELOW).

8      REJECT ALL REFLECTIONS MARKED AS LESS THAN BUT

THOSE FOR WITH

F(CALC)  LESS THAN  F(OBS) .

9      LIKE 8 BUT IN ADDITION REJECT ALL REFLECTIONS

LAYING OUTSIDE THE INTERVALL OF

SIN(THETA)/LAMBDA  DEFINED BY THE PARAMETERS

UNTEN  AND  OBEN  (SEE BELOW).

(COLUMN 5 IS EMPTY)

6  MSF     STRUCTURE FACTOR OUPUT SELECTOR.

0      NO STRUCTUR FACTOR OUTPUT.

1      PRINTED LIST OF STRUCTURE FACTORS CONTAINING

THE FOLLOWING INFORMATION

H,K,L        = MILLER INDICES

F(OBS)       = OBSERVED STRUCTURE FACTOR

(REFLECTIONS MARKED AS LESS THAN

SQUAREROOT(Y * (A*A + B*B))

ON THE INPUT CARDS ARE MARKED

HERE WITH  LT  PRINTED BEHIND

F(OBS)

F(CALC)      = CALCULATED STRUCTURE FACTOR

A(CALC)      = REAL PART OF F(CALC)

B(CALC)      = IMAGINARY PART OF F(CALC)

DELTA F      = F(OBS) - F(CALC)

DELT/WEIGHT  = (DELTA F) / WEIGHT

EXT. FACTOR  = EXTINCTION FACTOR Y DEFINED BY

F(CALC) =

SINT/LAM     = SIN(THETA) / LAMBDA

(REJECTED REFLECTIONS ARE MARKED

BY  RJ  PRINTED BEHIND

SINT/LAM ).

2      STRUCTURE FACTORS PRINTED ON OUTPUT AND WRITTEN

ON THE FOURIER FILE . THIS FILE IS THE DATA

FILE FOR THE PROGRAMMES FOURIE, ERRMAP, LISTFC,

OR AVERAG.

3      STRUCTURE FACTORS WRITTEN ON FOURIER FILE ONLY.

4/5    LIKE 1/2 BUT SHORT STRUCTURE FACTOR LIST.

7  MCORR   CORRELATION MATRIX OUTPUT SELECTOR

0      CORRELATION MATRIX NOT PRINTED.

(THE VALUES OF THE 20 LARGEST OFF-DIAGONAL

CORRELATION COEFFICIENTS ARE PRINTED FOR

EVERY LEAST-SQUARES CYCLE).

1      ENTIRE CORRELATION MATRIX PRINTED.

2/3    LIKE 0/1 BUT ALSO OUTPUT TO A LEVERAGE FILE.

ATTENTION

The leveredge file is only created if this cycle is a

refinement cycle or a calc. of structure factors.

8  MBODAN  BOND DISTANCES AND ANGLES SELECTOR.

0      NO DISTANCES AND ANGLES COMPUTED.

1      ONLY DISTANCES BETWEEN DMIN AND DMAX COMPUTED.

(DMIN AND DMAX SEE  BOND LIMITS CARD ).

2      DISTANCES BETWEEN DMIN AND DMAX AND CORRESPON-

DING ANGLES COMPUTED.

(DMIN AND DMAX SEE  BOND LIMITS CARD ).

REMARKS

THE PROGRAMME REFINE CALCULATES DISTANCES AND ANGLES WITHOUT STANDARD

DEVIATIONS. FOR CALCULATION WITH STANDARD DEVIATIONS CALL PROGRAMME

BONVIB .

THE BOND LIMITS (DMIN,DMAX) MAY NOT BE READ IN THE CURRENT CYCLE IF

DONE BEFORE (THEY MUST BE READ IN AN -A PRIORI- CYCLE INDEPENDENT

FROM THE VALUE GIVEN FOR  MBODAN ).

9  MELVIB  SELECTOR FOR COMPUTATION OF ELLIPSOIDS OF VIBRATION.

0      NO ELLIPSOIDS COMPUTED.

1      THERMAL ELLIPSOIDS COMPUTED FOR ATOMS WITH AT

LEAST ANISOTROPIC TEMPERATURE FACTORS.

(ONLY IN THE CASE OF HARMONIC REFINEMENT THE

THERMAL ELLIPSOIDS REPRESENT THE MEAN SQUARE

DISPLACEMENTS OF THE ATOMS ALONG THE PRINCIPAL

AXES.

WHEN REFINING ANHARMONIC TEMPERATURE FACTORS

THE ELLIPSOIDS COULD BE CALCULATED AS A TEST

FOR THE POSITIVITY OF THE SECOND ORDER TENSOR.

THIS POSITIVITY IS A PREREQUISITE FOR THE

EXISTENCE OF THE PROBABILITY DENSITY FUNCTION)

2       LIKE 1 AND IN ADDITION THERMAL CORRECTIONS

OF THE BOND LENGTHS COMPUTED.

10  IFILE   READ AND REWRITE DATA SET INDICATOR.

0      DATA SET REWRITTEN ON THE MAINFILE IF THIS IS

THE LAST CYCLE OF THE CURRENT RUN.

1      DATA SET NOT REWRITTEN.

2      LIKE  0  BUT IN ADDITION (DURING INPUT) READ

THE STANDARD DEVIATIONS OF PARAMETERS REFINED

IN PREVIOUS CYCLES FROM THE  MAINFILE  (SEE

REMARK BELOW).

3      LIKE  1  BUT IN ADDITION (DURING INPUT) READ

THE STANDARD DEVIATIONS OF PARAMETERS REFINED

IN PREVIOUS CYCLES FROM THE  MAINFILE  (SEE

REMARK BELOW).

REMARK

2 OR 3 ARE ONLY NECESSARY, IF CALCULATION IS NOT A FULL MATRIX LEAST

SQUARES FIT.

(COLUMN 11 IS EMPTY)

ATTENTION

IN AN -A PRIORI- CYCLE ALL THE FOLLOWING VARIABLES  NEW(1)-NEW(8)

MUST BE  NOT EQUAL 0  .

12  NEW(1)  READ NEW CELL PARAMETERS (CELL CARD).

0      DO NOT.

1      DO.

13  NEW(2)  READ NEW BOND DISTANCE LIMITS USED IN CALCULATION OF

BONDS (BOND LIMITS CARD).

0      DO NOT.

1      DO.

14  NEW(3)  READ NEW SYMMETRY INFORMATION (CELL TYPE CARD  AND

SYMMETRY CARDS).

0      DO NOT.

1      DO.

15  NEW(4)  READ NEW SCATTERING FACTORS (SCAT.CURVE TYPE CARD(S)

AND  EXPONENT. FORM CARD(S) OR  TABULATION FORM

CARD(S) PLUS ANOMALOUS DISP. CARD(S) IF NEEDED).

0      DO NOT.

1      DO.

16  NEW(5)  SELECTOR FOR READ NEW PARAMETER VALUES (SCALE FACTOR

CARD,  EXTINCTION CARD,  ATOM PARAMETER CARD(S),

SPECIAL PARAM. CARD, AND  ALPHA PARAMETER CARD(S) ).

0      DO NOT READ ANY NEW PARAMETER VALUE.

1      READ ENTIRE PARAMETER LIST OF PARAMETER VALUES

(SEE LIST OF CARDS IN BRACKETS ABOVE).

2      READ ONLY NEW SCALE FACTORS (SCALE FACTOR CARD)

3      READ ONLY NEW EXTINCTION PARAMETERS (EXTINCTION

CARD).

4      READ ONLY NEW ATOM PARAMETERS (ATOM PARAMETER

CARD(S) ) SELECTED BY  ATOM SELECTION CARD(S

5      READ ONLY NEW SPECIAL PARAMETERS (SPECIAL

PARAM. CARD).

6      READ ONLY NEW ALPHA PARAMETER(S) (ALPHA PARA-

METER CARD(S) ).

17  NEW(6)  READ NEW TEMPERATURE FACTOR TYPES (T-FACTOR TYPE CARD)

0      DO NOT.

1      DO.

18  NEW(7)  READ NEW PARAMETER SELECTIONS AND DEPENDENCIES (SCALE

REFINE CARD,  EXTINC. REFINE CARD,  ALPHA REFINE CARD

SPECIAL REFINE CARD,  ATOM REFINE CARD(S), NEW VALUE

CARD(S), AND DEPENDENCY CARD(S) ).

REMARK

THE  EXTINC. REFINE CARD  IS ONLY NEEDED IF EXTINCTION CORRECTION IS

SELECTED, AS WELL THE  ALPHA-  AND THE  SPECIAL REFINE CARD CAN BE

OMITTED. THE  NEW VALUE CARD(S)  ARE ONLY NEEDED FOR SPECIAL VALUES

OF THE PARAMETERS GIVEN ON THE  ATOM REFINE CARD(S).

19  NEW(8)  INDICATOR FOR DIFFERENT TYPES OF HKL-CARDS TO BE READ

(FORMAT SPECIFIED BY THE  FORMAT CARD ) AND SWITCH FOR

READ  DIFFRACTOMETER CARD  AND ORIENTATION CARDS.

0      DO NOT READ HKL-CARDS. (I.E. READ MAIN UNIT)

NOT 0  READ HKL CARDS. THE VALUES 1-9 CORRESPOND TO

VARIOUS OPTIONS FOR THE ORDER OF THE LIST,

AS EXPLAINED BELOW. IF THE VALUES IN BRACKETS

ARE USED (6-9) IN ADDITION THE  DIFFRACTOMETER

CARD  AND THE  ORIENTATION CARDS  ARE READ.

TABEL OF VERSIONS OF HKL-CARDS ACCEPTED BY THE PROGRAMME

NEW(8)   LABEL    IH     MREJ     ISG    FOBS   TBAR   SIGMA

1 (6)      X       X       X               X             X

2 (7)      X       X       X               X      X      X

3 (8)      X       X       X       X       X             X

4 (9)      X       X       X       X       X      X      X

5        X       X

EXPLANATION OF THE TABLE

LABEL        HKL CARD DESIGNATOR. IT IS ONLY NEEDED TO INDICATE

THE END OF THE LIST.

END      THE LAST HKL-CARD WAS READ. (NO FURTHER

INFORMATION IS READ FROM THE CARD WITH THE

LABEL = END).

NOT END  THIS CARD IS AN HKL=CARD.

IH           MILLER INDICES.

MREJ         DESIGNATOR FOR REFLECTIONS USED AS  OBSERVED  OR  LESS

THAN  REFLECTIONS.

1  OBSERVED REFLECTION

2  LESS THAN REFLECTION

ISG          NUMBER OF THE SCALE GROUP TO WHICH THE REFLECTION

BELONGS.

FOBS         INTENSITY OR STRUCTURE FACTOR (SEE PARAMETER  IFOBS  ON

THE  DATA DEFINING CARD).

TBAR         ABSORPTION WEIGHTED MEAN PATH LENGTH OF THE BEAM THROUGH

THE CRYSTAL.

SIGMA        STANDARD DEVIATION OR WEIGHT OF THE PARAMETER FOBS

DEPENDING ON THE VALUE OF  IWT  ON THE  DATA DEFINING

CARD.

REMARK

NEW(8)=5 CAN BE USED FOR READING ONLY MILLER INDICES FOR STRUCTURE

FACTOR CALCULATION.

20-27  UNTEN   LOWER AND UPPER LIMIT OF THE INTERVAL OF

28-35  OBEN    SIN(THETA)/LAMBDA NEEDED FOR THE REJECTION OF CERTAIN

REFLECTIONS IF THE PARAMETER  IREJ  ON THIS CARD

(SEE ABOVE) IS  7  OR  9 .

36-43  DELMAX  FACTOR NEEDED FOR THE REJECTION OF CERTAIN REFLECTIONS

IF THE PARAMETER  IREJ  ON THIS CARD (SEE ABOVE) IS

2 (IN SOME CASES), 3, 4, 5, OR 6 .

(COLUMN 44 IS EMPTY)

45  IPART   PARTIAL STRUCTURE FACTOR CALCULATION SELECTOR

0      CALCULATE STRUCTURE FACTORS FOR THE COMPLETE

STRUCTURE.

ATTENTION

THE VALUES IPART = 1 AND 2  ARE ONLY POSSIBLE IF THE

PARAMETER NEW(8) ON THIS CARD IS NOT EQUAL 0.

1      CALCULATE STRUCTURE FACTORS ONLY FOR A PARTIAL

STRUCTURE BY DELETING ATOM(S) SELECTED BY

PARTIAL CARD(S).

(SEE REMARK BELOW).

2      LIKE  1  BUT NOW F(OBS) IS OVERWRITTEN BY

F(CALC) OF THE COMPLETE STRUCTURE.

(USING THIS PROCEDURE ONE CAN CALCULATE THE

MAXIMUM INFLUENCE OF THE SELECTED ATOM(S)

ON THE R-VALUE).

REMARK

IF IPART=1 OR 2 NO REFINEMENT IS POSSIBLE AND THE DATA SET CANNOT BE

REWRITTEN.

WHEN THE CALCULATION OF PARTIAL STRUCTURE FACTORS IS SELECTED BY

SETTING IPART=1 OR 2 THE  PHASE  WILL BE CALCULATED USING THE

COMPLETE STRUCTURE. IF THE PHASE SHOULD BE CALCULATED TOO ONLY USING

THE PARITAL STRUCTURE TOO, THE ATOMS MUST BE DELETED BY SETTING THE

OCCUPANCIES TO ZERO. (THEN THE PARAMETER  IPART  IS NOT NEEDED).

(COLUMN 46 IS EMPTY)

47  IHARM   SELECTOR FOR WRITE A HARMONIC-FILE FOR CALCULATION OF

AN EXTENDED EDGEWORTH MAP.

(ONLY NEEDED IF AN EXTENDED EDGEWORTH MAP SHOULD BE

CALCULATED WITH THE PROGRAMME  PDFMAP ).

0      DO NOT WRITE A HARMONIC-FILE.

1      WRITE A HARMONIC-FILE.

(LOGICAL NUMBER OF THE HARMONIC-FILE = 9 ).

48  JXPAR   INDICATOR FOR THE USE OF SPECIAL PARAMETERS.

0      DO NOT USE SPECIAL PARAMETERS.

1      USE SPECIAL PARAMETERS.

(FOR SPECIAL PARAMETERS SEE DESCRIPTION PART).

49  IFSGEN  ORIENTATION DETERMINATION SELECTOR.

(ORIENTATION  MEANS THE TWO DIFFERENT ORIENTATIONS OF

ENANTIOMORPHIC STRUCTURES)

0      DO NOT DETERMINE THE ORIENTATION.

1      CHANGE TO THE ORIENTATION WITH LOWER  R(W) .

(COLUMN 50 IS EMPTY)

51  IOVER   INDICATOR FOR CHANGE OF THE STRUCTURE FACTOR EQUATION.

0      DO NOT ALLOW A CHANGE OF THE STRUCTURE FACTOR

EQUATION IF THE CURRENT CYCLE IS NOT AN

-A PRIORI- CYCLE.

1      ALLOW THE CHANGE OF THE STRUCTURE FACTOR

EQUATION ALTHOUGH THE CURRENT CYCLE IS NOT AN

-A PRIORI- CYCLE.

CELL CARD

(ONLY NEEDED IF NEW(1) NOT EQUAL 0).

FORMAT (6F8.0)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 8  AG      LATTICE CONSTANTS

9-16  BG      A, B, AND C

17-24  CG      GIVEN IN ANGSTROMS.

25-32  WINKEL  LATTICE ANGLES

33-40  WINKEL  ALPHA, BETA, AND GAMMA

41-48  WINKEL  GIVEN IN DEGREES.

BOND LIMITS CARD

(ONLY NEEDED IF NEW(2) IS NOT EQUAL 0 ).

FORMAT (2F8.0)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 8  DMIN    MINIMUM BOND DISTANCE IN ANGSTROMS.

9-16  DMAX    MAXIMUM BOND DISTANCE IN ANGSTROMS.

(DMIN AND DMAX ARE ONLY USED FOR THE CALCULATION OF

BOND DISTANCES AND ANGELS IF THE PARAMETER  MBODAN

ON THE  MAIN CONTROL CARD  IS 1 OR 2 ).

CELL TYPE CARD

(ONLY NEEDED IF NEW(3) IS NOT EQUAL 0 ).

FORMAT (A4,25X,2A1)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 4  ICENT   FLAG FOR CENTRIC OR ACENTRIC CELL

CENT   CENTRIC  CELL

ACEN   ACENTRIC CELL

22,23 ICELL   INDICATOR FOR THE TYPE OF THE CELL (CAN BE WRITTEN

IN COLUMN 22 OR 23).

P      PRIMITIVE

A      A- CENTERED

B      B- CENTERED

C      C- CENTERED

I      I- CENTERED

F      F- CENTERED

R      RHOMBOHEDRAL

REMARK

BECAUSE OF THE SPECIAL FORMAT THIS CARD CAN BE WRITTEN IN THE FORM

CENTRIC CELL OF TYPE I

OR

ACENTRIC CELL OF TYPE P

AND SO ON.

SYMMETRY CARDS

(ONLY NEEDED IF NEW(3) IS NOT EQUAL 0 ).

FORMAT (3A3,1X,3A3,1X,3A3)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 3  JTNS    TRANSLATIONAL      PART OF  X .

HERE ONE OF THE OPERATORS DESCRIBED BELOW OR THE WORD

END  TO INDICATE THAT ALL SYMMETRY CARDS  WERE READ

MUST APPEAR.

5- 6  IRMAT   FIRST  POSITIONAL  PART OF  X .

8- 9  IRMAT   SECOND POSITIONAL  PART OF  X .

11-13          TRANSLATIONAL      PART OF  Y .

15-16          FIRST  POSITIONAL  PART OF  Y .

18-19          SECOND POSITIONAL  PART OF  Y .

21-23

25-26          SAME FOR  Z .

28-29

THE SYMMETRY OPERATIONS MUST BE WRITTEN IN THE FORM USED IN THE

INTERNATIONAL TABLES OF CRYSTALLOGRAPHY. THIS MEANS THAT

THE TRANSLATIONAL OPERATORS ARE OF THE FOLLOWING FORM  (B = BLANK)

BBB, 1/2, 1/3, 2/3, 1/4, 3/4, 1/6, OR 5/6 .

THE POSITIONAL OPERATOR ARE OF THE FORM  (W = X, Y, OR Z)

+W, BW (=+W), OR -W.

ATTENTION

THE SYMMETRY POSITION  X Y Z  MUST BE INCLUDED.

REMARK

THERE CAN BE A MAXIMUM OF 24 SYMMETRY CARDS, BECAUSE POSITIONS

RELATED BY A CENTER OF INVERSION OR LATTICE CENTERING MAY NOT

BE INCLUDED.

SCAT.CURVE TYPE CARD

(ONLY NEEDED IF NEW(4) IS NOT EQUAL 0 ).

FORMAT (A1,3A4)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1  IFLAG   ANOMALOUS DISPERSION INDICATOR AND END FLAG

BLANK  NO ANOMALOUS DISPERSION CORRECTION TO BE USED.

A      ANOMALOUS DISPERSION CORRECTION TO BE READ FOR

THIS SCATTERING CURVE (ANOMALOUS DISP. CARD)

E      END FLAG  INDICATING THAT ALL SCATTERING CURVES

WERE READ.

2- 9  L       ANY SEQUENCE OF SYMBOLS TO BE USED AS A LABEL FOR

THIS SCATTERING CURVE.

10-13  ITABLE  INDICATOR FOR THE REPRESENTATION OF THE SCATTERING

CURVE TO BE READ.

BLANKS ON THE FOLLOWING CARD THE COEFFICIENTS FOR THE

EXPONENTIAL REPRESENTATION OF THE SCATTERING

CURVE ARE GIVEN.

(EXPONENT. FORM CARD).

TABLE  ON THE FOLLOWING CARDS THE SCATTERING CURVE

IS GIVEN IN TABULATION FORM.

(TABULATION FORM CARDS).

REMARK

FOR EACH SCATTERING CURVE THERE MUST BE A  SCAT.CURVE TYPE CARD

FOLLOWED BY AN  EXPONENT. FORM CARD  OR BY  TABULATION FORM CARDS

PLUS (IF IFLAG=A) AN  ANOMALOUS DISP. CARD.

THE END OF THE LIST MUST BE INDICATED BY AN  END CARD  CONTAINING

ONLY THE WORD  END  TYPED IN COLUMNS 1-3 .

EXPONENT. FORM CARD

(ONLY NEEDED IF FOR THE PARAMETER  ITABLE  ON THE  SCAT.CURVE TYPE

CARD  JUST WRITTEN ONLY BLANKS WERE TYPED).

FORMAT (9F8.0)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 8  A(1)    IF X-RAY DATA ARE REFINED THESE COEFFICIENTS MUST

9-16  B(1)    SATISFY THE FOLLOWING EQUATION

17-24  A(2)                4

25-32  B(2)    F(X) = C + SUM ( A(I) * EXP(-B(I) * X * X ) )

33-40  A(3)               I=1

41-48  B(3)    WITH  X = SIN(THETA) / LAMBDA

49-56  A(4)

57-64  B(4)    IF NEUTRON DATA ARE REFINED  C  MUST BE THE SCATTERING

65-72  C       LENGTH. (A(I) AND B(I) ARE MEANINGLESS).

TABULATION FORM CARDS

(ONLY NEEDED IF THE PARAMETER  ITABLE  ON THE  SCAT.CURVE CARD

JUST WRITTEN IS EQUAL  TABLE ).

FORMAT (2F8.0)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 8  X       SIN(THETA) / LAMBDA

9-16  Y       CORRESPONDING VALUE OF THE SCATTERING CURVE.

REMARKS

FOR ONE SCATTERING CURVE THERE MAY BE A MINIMUM OF 10 AND CAN BE A

MAXIMUM OF 35 TABULATION CARDS.

THE END OF THE LIST IS INDICATED BY X=0. AND Y=0. .

THE VALUES OF X MUST INCREASE MONOTONICALLY WHILE THE VALUES OF  Y

SHOULD DECREASE MONOTONICALLY. A 5 PERCENT DEVIATION FROM THE

MONOTONY OF Y IS ALLOWED.

ANOMALOUS DISP. CARD

(ONLY NEEDED IF THE PARAMETER  IFLAG  ON THE LAST  SCAT.CURVE TYPE

CARD IS EQUAL  A ).

FORMAT (2F8.0)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 8  ANOM1   REAL      PART OF ANOMALOUS DISPERSION CORRECTION

9-16  ANOM2   IMAGINARY PART OF ANOMALOUS DISPERSION CORRECTION

SCALE FACTOR CARD

(ONLY NEEDED IF NEW(5)=1 OR 2 ).

FORMAT (10F7.0)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 7  SCALE(1)

8-14    .

15-21    .            A MAXIMUM OF  10  SCALE FACTORS

22-28    .            CAN BE GIVEN.

29-35                 (IF MORE THAN ONE SCALE FACTOR IS GIVEN ON THIS

36-42                  CARD, EACH HKL CARD MUST CONTAIN THE NUMBER OF

43-49                  THE SKALE FACTOR TO BE USED FOR THE REFLEC-

50-56                  TION (NEW(8) = 3(8) OR 4(9) ) ).

57-63

64-70  SCALE(10)

EXTINCTION CARD

(ONLY NEEDED IF NEW(5)=1 OR 3 ).

FORMAT (2I1,7F8.4)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1  NIEXT   INDICATOR FOR THE EXTINCTION MODEL TO BE USED.

0-6    EXTINCTION CORRECTION BASED ON THE CALCULATIONS

OF BECKER AND COPPENS.

0    NO          EXTINCTION CORRECTION

1    ISOTROPIC   EXTINCTION CORRECTION  TYPE 1

2    ISOTROPIC   EXTINCTION CORRECTION  TYPE 2

3    ISOTROPIC   EXTINCTION CORRECTION  GENERAL CASE

4    ANISOTROPIC EXTINCTION CORRECTION  TYPE 1

5    ANISOTROPIC EXTINCTION CORRECTION  TYPE 2

6    ANISOTROPIC ECTINCTION CORRECTION  GENERAL CASE

8      ISOTROPIC ZACHARIASEN CORRECTION OR ANISOTROPIC

COPPENS AND HAMILTON CORRECTION. (SEE PARAMETER

NDISTR ).

2  NDISTR  DISTRIBUTION AND EXTINCTION CHANGE INDICATOR.

(THIS PARAMETER HAS DIFFERENT MEANINGS DEPENDING ON

THE EXTINCTION MODEL INDICATOR  NIEXT ).

FOR  NIEXT  LESS EQUAL  6

(BECKER AND COPPENS EXTINCTION)

0 (2)  DISTRIBUTION OF MOSAIC SPREAD IS GAUSSIAN.

1 (3)  DISTRIBUTION OF MOSAIC SPREAD IS LORENTZIAN.

(IF NDISTR=0 OR 1 AN AUTOMATIC DAMPING ROUTINE

IS USED IF PARAMETERS BECOME UNPHYSICAL AND IF

NO FIXED DAMPING FACTORS ARE SELECTED ON THE

EXTINC. REFINE CARD. IF NDISTR=2 OR 3 NO

AUTOMATIC DAMPING IS USED).

FOR  NIEXT  =  8

(ZACHARIASEN OR HAMILTON AND COPPENS EXTINCTION)

0   ISOTROPIC          ,  DO NOT CHANGE.

1   ANISOTROPIC TYPE I ,  DO NOT CHANGE.

2   ANISOTROPIC TYPE II,  DO NOT CHANGE.

3   ISOTROPIC          , CHANGE TO ANISOTROPIC TYPE I

4   ISOTROPIC          , CHANGE TO ANISOTROPIC TYPE II

5   ANISOTROPIC TYPE I , CHANGE TO ISOTROPIC

6   ANISOTROPIC TYPE II, CHANGE TO ISOTROPIC

7   ANISOTROPIC TYPE I , CHANGE TO ANISOTROPIC TYPE II

8   ANIXOTROPIC TYPE II, CHANGE TO ANISOTROPIC TYPE I

9   DO NOT CHANGE TYPE BUT RESET PARAMETER TO ZERO.

3-58  RNEUEX  INITIAL VALUES FOR THE EXTINCTION PARAMETERS TO BE

USED MULTIPLIED WITH A FACTOR OF 10000 .(SO THE

VALUES GIVEN HERE ARE IDENTICAL WITH THOSE IN THE

OUTPUT BECAUSE THERE THEY ARE PRINTED WITH THE SAME

FACTOR).

3-10  PARAMETER 1

11-18      .

19-26      .

27-34      .

35-42

43-5

51-58  PARAMETER 7

WHEN BECKER AND COPPENS EXTINCTION IS USED THESE PARAMETERS HAVE THE

FOLLOWING MEANING

.....I S O T R O P I C....      ***A N I S O T R O P I C***

TYPE 1   TYPE 2    GENERAL      TYPE 1   TYPE 2    GENERAL

PARAM.                       CASE                            CASE

NO.

1        G        R          R          Z(11)    Z(11)     Z(11)

2                            G          Z(22)    Z(22)     Z(22)

3                                       Z(33)    Z(33)     Z(33)

4                                       Z(12)    Z(12)     Z(12)

5                                       Z(13)    Z(13)     Z(13)

6                                       Z(23)    Z(23)     Z(23)

7                                                            R

REMARK

IF THE TYPE OF THE EXTINCTION IS CHANGED, THE CYCLE IS NOT AN

-A PRIORI- CYCLE, AND THERE ARE NO VALUES GIVEN FOR  RNEUEX

(RNEUEX = 0.), THE PROGRAMME CONVERTS THE PARAMETER(S) REFINED BEFORE

INTO THE NEW ONE(S). A CONVERSION FROM ISOTROPIC TO ANISOTROPIC IS

ONLY POSSIBLE IF THE ORIENTATION MATRIX WAS READ OR WILL BE READ.

(SEE NEW(8) ON THE  MAIN CONTROL CARD).

ATTENTION

A CONVERSION FROM THE ZACHARIASEN (RESP. COPPENS AND HAMILTON) MODEL

TO THE BECKER AND COPPENS MODEL OR VICE VERSA IS NOT POSSIBLE. FOR

CHANGING THE MODEL AN -A PRIORI- CYCLE MUST BE STARTED.

ATOM SELECTION CARD(S)

PREFATORY NOTICE     (CONCERNING THIS AND THE ATOM PARAMETER

CARDS).

THERE ARE TWO POSSIBILITIES OF READING  ATOM PARAMETER CARDS. IN AN

-A PRIORI- CYCLE THE ENTIRE ATOM PARAMETER LIST FOR ALL ATOMS HAS TO

BE READ AND NO  ATOM SELECTION CARD CAN BE GIVEN. THE END OF THE LIST

MUST BE INDICATED BY AN  END CARD  WHICH CONTAINS ONLY THE WORD  END

IN COLUMNS 1-3 .

IF IN A -CONTINUE CYCLE- YOU WANT TO CHANGE PARAMETERS OF CERTAIN

ATOMS OR ADD SOME ATOMS TO THE CURRENT LIST, YOU MUST USE THE

ATOM SELECTION CARD(S).

(THE ATOM SELECTION CARD IS ONLY NEEDED IF  NEW(5) = 4).

FORMAT (I2)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 2  I       NUMBER OF THE ATOM FOR WHICH NEW ATOM PARAMETERS

SHOULD BE READ (ATOM PARAMETER CARD(S) ).

(THE ATOMS ARE NUMBERED IN THE -A PRIORI- RUN

FROM  1  TO  NATOM  ACCORDING TO THE SEQUENCE THEY

ARE LISTED ON THE  ATOM PARAMETER CARDS. THE NUMBER

GIVEN FOR  I  HERE CAN BE GREATER THAN  NATOM , BUT

THEN  I  MUST INCREASE MONOTONICALLY STARTING WITH

I = NATOM+1 ).

(AS MENTIONED ABOVE THE END OF THE LIST OF ATOM

SELECTION CARDS MUST BE INDICATED BY  I = 0 ).

ATOM PARAMETER CARD(S)

(ONLY NEEDED IF NEW(5) = 1 OR 4 ).

PREFATORY NOTICE

THE ATOM PARAMETER CARD FOR ONE ATOM IN REALITY CONSISTS OF AT LEAST

2 CARDS AND AT MOST OF 11 CARDS DEPENDING ON THE ORDER OF THERMAL

TENSORS TO BE READ.

CARD 1

FORMAT (A1,2A3,I4,2X,2F6.4)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1  IFLAG   INDENTIFIER FOR THE END OF ATOM PARAMETER LIST.

BLANK     THERE ARE MORE ATOM PARAMETER CARDS TO BE

READ.

NOT BLANK THE ATOM PARAMETER CARDS WERE READ FOR ALL

ATOMS. (ATTENTION.. THIS CARD IDENTIFIING

THE END OF THE ATOM PARAMETER CARDS IS AN

ADDITIONAL CARD. IT S  CONTENS IS NO MORE

READ.)

2- 7  TAG     ANY SEQUENCE OF SYMBOLS USED AS IDENTIFIER FOR THE

ATOM.

8-11  SITE    MULTIPLICITY OF THE POSITION OCCUPIED BY THE ATOM.

14-19  OCCUP   TOTAL OCCUPANCY OF THE SITE.

20-25  OCCA    OCCUPANCY OF SPECIES  A  (SEE CARD 2) IN THIS SITE.

(OCCA IS ONLY NEEDED IF MULTIPLE OCCUPANCY IS USED).

CARD 2

FORMAT (2I2,1X,A2,1X,9F7.5,I2)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 2  ISCAT1  NUMBER OF THE SCATTERING CURVE FOR SPECIES  A .

(THE SCATTERING CURVES ARE NUMBERED ACCORDING TO THE

SEQUENCE THEY WERE READ).

3- 4  ISCAT2  NUMBER OF THE SCATTERING CURVE FOR SPECIES  B .

(ONLY NEEDED IF THERE IS A MULTIPLE OCCUPANCY OF TWO

DIFFERENT ATOMS IN THIS POSITION).

6- 7  BTYP    TEMPERATUR FACTOR TYPE

B   BETA NOTATION

U   CRUICKSHANK NOTATION (MULTIPLIED BY 100)

BU  INPUT BETA, OUTPUT U

UB  INPUT U, OUTPUT BETA

DEFAULT IS B.

9-15  XYZ(1)  X COORDINATE     FOR THE ATOM

16-22  XYZ(2)  Y COORDINATE     GIVEN IN

23-29  XYZ(3)  Z COORDINATE     FRACTIONAL COORDINATES.

30-71  BETA    VALUES OF THE ELEMENTS FO THE THERMAL TENSOR OF

SECOND ORDER (ANISOTROPIC TEMPERATURE FACTOR) RESP.

ISOTROPIC  B-VALUE.

30-36          BETA(11) OR THE  B-VALUE  IF ONLY ISOTROPIC THERMAL

VIBRATONS ARE REFINED.   )

37-43          BETA(22)                          )

44-50          BETA(33)                          ) OR U(IJ)*100 DE-

51-57          BETA(12)                          ) PENDING ON BTYP

58-64          BETA(13)                          )

65-71          BETA(23)                          )

72-73  IEF     INDICATOR FOR THIRD ORDER CUMULANTS TO BE READ FROM

THE FOLLOWING CARD.

0      DO NOT READ THIRD ORDER CUMULANTS, THIS IS THE

LAST PARAMETER CARD FOR THIS ATOM.

1      READ THIRD ORDER CUMULANTS (CARD 3).

CARD 3

FORMAT (10F7.0,I1)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 70 CR      VALUES OF THE ELEMENTS OF THE THIRD ORDER CUMULANT

MULTIPLIED WITH 1000. (THIS MEANS THAT THE VALUES

GIVEN HERE ARE IDENTICAL WITH THOSE PRINTED IN THE

OUTPUT).

1- 7          C(111)

8-14          C(222)

15-21          C(333)

22-28          C(112)

29-35          C(122)

36-42          C(113)

43-49          C(133)

50-56          C(223)

57-63          C(233)

64-70          C(123)

71  IEF     INDICATOR FOR FOURTH ORDER CUMULANTS TO BE READ FROM

THE FOLLOWING TWO CARDS.

0      DO NOT READ FOURTH ORDER CUMULANTS. THIS IS THE

LAST PARAMETER CARD FOR THIS ATOM.

1      READ FOURTH ORDER CUMULANTS.

CARD 4  AND  CARD 5

FORMAT  CARD 4  (10F7.0)

CARD 5  (5F7.0,35X,I1)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1-70  DR      VALUES OF THE ELEMENTS OF THE FOURTH ORDER TENSOR

MULTIPLIED WITH 10 000 LIKE ON THE OUTPUT.

CARD 4        CARD 5

1- 7          D(1111)       D(1233)

8-14          D(2222)       D(1333)

15-21          D(3333)       D(2223)

22-28          D(1112)       D(2233)

29-35          D(1113)       D(2333)

36-42          D(1122)

43-49          D(1123)

50-56          D(1133)

57-63          D(1222)

64-70          D(1223)

71  IEF     (CARD 5 ONLY) INDICATOR FOR FIFTH ORDER CUMULANTS TO

BE READ FROM THE NEXT THREE CARDS.

0      DO NOT READ FIFTH ORDER CUMULANTS

THIS IS THE LAST PARAMETER CARD

FOR THIS ATOM.

1      READ FIFTH ORDER CUMULANTS.

CARD 6, CARD 7, AND CARD 8

FORMAT  CARD 6  (10F7.0)

CARD 7  (10F7.0)

CARD 8  (F7.0,63X,I1)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1-70  ER      VALUES OF THE ELEMENTS OF THE FIFTH OREDER TENSOR

MULTIPLIED WITH 100 000  LIKE ON THE OUTPUT.

CARD 6        CARD 7        CARD 8

1- 7          E(11111)      E(11233)      E(23333)

8-14          E(22222)      E(11333)

15-21          E(33333)      E(12222)

22-28          E(11112)      E(12223)

29-35          E(11113)      E(12233)

36-42          E(11122)      E(12333)

43-49          E(11123)      E(13333)

50-56          E(11133)      E(22223)

57-63          E(11222)      E(22233)

64-70          E(11223)      E(22333)

71  IEF     INDICATOR FOR SIXTH ORDER CUMULANTS TO BE READ FROM

THE NEXT THREE CARDS (IEF MAY APPEAR ONLY ON CARD 8).

0      DO NOT READ SIXTH ORDER CUMULANTS. THIS IS THE

LAST PARAMETER CARD FOR THIS ATOM.

1      READ SIXTH ORDER CUMULANTS.

CARD 9, CARD 10, AND CARD 11

FORMAT  CARD  9  (10F7.0)

CARD 10  (10F7.0)

CARD 11  (8F7.0)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1-70  FR      VALUES OF THE ELEMENTS OF THE SIXTH ORDER TENSOR

MULTIPLIED WITH 1000 000  LIKE ON THE OUTPUT.

CARD 9        CARD 10       CARD 11

1- 7          F(111111)     F(111233)     F(122333)

8-14          F(222222)     F(111333)     F(123333)

15-21          F(333333)     F(112222)     F(133333)

22-28          F(111112)     F(112223)     F(222223)

29-35          F(111113)     F(112233)     F(222233)

36-42          F(111122)     F(112333)     F(222333)

43-49          F(111123)     F(112222)     F(223333)

50-56          F(111133)     F(122222)     F(233333)

57-63          F(111222)     F(122223)

64-70          F(111223)     F(122233)

REMARK

IF THERE ARE MORE ATOMS TO BE READ, START AGAIN WITH AN  ATOM

SELECTION CARD  RESP. WITH  CARD 1  OF THE ATOM PARAMETER CARDS.

ATTENTION

IF THIS WAS THE LAST ATOM OF THE LIST THIS MUST BE INDICATED BY AN

END CARD  CONTAINING ONLY THE WORD  END  IN COLUMNS 1-3 .

SPECIAL PARAM. CARD

(ONLY NEEDED IF ON THE MAIN CONTROL CARD  JXPAR  NOT EQUAL  0  OR

NEW(5) = 5 ).

FORMAT (2X,I2)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

3- 4  NPAR    NUMBER OF SPECIAL PARAMETERS TO BE USED.

SPECIAL VALUE CARD(S)

(THERE MUST BE  NPAR  SPECIAL VALUE CARDS  CONTAINING THE NAMES,

INITIAL VALUES, AND IDENTIFIERS OF THE SPECIAL PARAMETERS).

FORMAT (2A4,3I2,F10.4)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 8  PARNAM  NAME OF SPECIAL PARAMETER, UP TO 8 ALPHANUMERIC

CHARACTERS.

9-10  ITEM(1) TYPE OF SPECIAL PARAMETER

0      LINEAR FUNCTION WITH COEFFICIENT GIVEN ON

DEPENDENCY CARD.

1      RIGID BODY SYSTEM.

2      DEFINED BY USER IN SUBROUTINES  SPVAL  AND

SPDERI (SEE DESCRIPTION PART).

11-12  ITEM(2) INTEGER INDICATING WHICH RIGID-BODY-SYSTEM, IF MORE

THAN ONE, THIS PARAMETER REFERS TO.

13-14  ITEM(3) PARAMETER DESIGNATOR. IF THE TYPE IS 1 THE CODE IS AS

FOLLOWS

1      X COORDINATE OF ORIGIN (IN CRYSTAL AXES SYSTEM)

2      Y COORDINATE OF ORIGIN

3      Z COORDINATE OF ORIGIN

4      OMEGA ANGLE FOR TRANSFORMATION TO STANDARD,

ORTHONORMAL COORDINATE SYSTEM.

5      CHI ANGLE FOR TRANSFORMATION.

6      PHI ANGLE FOR TRANSFORMATION.

7      X COORDINATE OF AN ATOM IN THE SPECIAL SYSTEM.

8      Y COORDINATE OF AN ATOM.

9      Z COORDINATE OF AN ATOM.

10     T11

11     T22

12     T33

13     T12

14     T13

15     T23

16     L11

17     L22

18     L33

19     L12

20     L13

21     L23

22     S11

23     S22

24     S33

25     S12

26     S13

27     S23

28     S21

29     S31

30     S32

15-24  PAR     INITIAL VALUE OF THIS PARAMETER.

ALPHA PARAMETER CARD(S)

(ONLY NEEDED IF  NEW(5)=1 OR 6  AND THE STRUCTURE FACTOR EQUATION

NO. 3 OR 4 IS USED).

FORMAT (9F8.0)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 8          VALUE OF ALPHA FOR THE FIRST ATOM (ATOM NUMBER ACCOR-

DING TO THE SEQUENCE OF ATOMS ON THE ATOM PARAMETER

CARDS).

ATTENTION

ALPHA VALUES ARE READ FOR  ALL  ATOMS, NOT ONLY FOR THOSE WHICH ARE

REFINED WITH ANHARMONIC TEMPERATURE FACTORS. YOU HAVE TO TYPE 0. OR

LEAVE EIGHT BLANKS FOR ATOMS WHICH DO NOT NEED AN ALPHA.

9-16          VALUE OF ALPHA FOR THE SECOND ATOM.

17-24            .                    THIRD  ATOM.

25-32            .

33-4

41-48

49-56

57-64

65-72          VALUE OF ALPHA OF THE NINETH ATOM.

REMARKS

IF THE NUMBER OF INITIAL VALUES (= NUMBER OF ATOMS) IS GREATER THAN

9, ADD AS MANY OF ALPHA PARAMETER CARDS AS YOU NEED.

IN THE CURRENT VERSION OF THE PROGRAMME NO DEPENDENCIES BETWEEN

ALPHA-PARAMETERS ARE POSSIBLE.

T-FACTOR TYPE CARD

(ONLY NEEDED IF  NEW(6) NOT EQUQL 0 ).

FORMAT (60I1)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1  ISOT    INDICATOR FOR THE TEMPERATURE FACTOR TYPE TO BE

USED FOR ATOM NUMBER 1  (NUMBER ACCORDING TO THE

SEQUENCE OF THE ATOMS ON THE ATOM PARAMETER CARDS).

0      NOW ISOTROPIC        , DO NOT CHANGE.

1      NOW ANISOTROPIC      , DO NOT CHANGE.

2      NOW ISOTROPIC        , CHANGE TO ANISOTROPIC.

3      NOW ANISOTROPIC      , CHANGE TO ISOTROPIC.

4      NOW 3RD ORDER        , DO NOT CHANGE.

5      NOW ANISOTROPIC      , CHANGE TO 3RD ORDER.

6      NOW 3RD ORDER        , CHANGE TO ANISOTROPIC

7      NOW 4TH ORDER        , DO NOT CHANGE.

8      NOW 3RD ORDER        , CHANGE TO 4TH ORDER.

9      NOW 4TH ORDER        , CHANGE TO 3RD ORDER.

A      NOW 5TH ORDER        , DO NOT CHANGE.

B      NOW 4TH ORDER        , CHANGE TO 5TH ORDER.

C      NOW 5TH ORDER        , CHANGE TO 4TH ORDER.

D      NOW 6TH ORDER        , DO NOT CHANGE.

E      NOW 5TH ORDER        , CHANGE TO 6TH ORDER.

F      NOW 6TH ORDER        , CHANGE TO 5TH ORDER.

2          SAME FOR ATOM NUMBER 2 .

3          SAME FOR ATOM NUMBER 3 .

4          AND SO ON .

ATTENTION

60 ATOMS IN THE ASSYMMETRIC UNIT ARE ALLOWED AS LONG AS ONLY TEMPERA-

TURE FACTORS UP TO 4TH ORDER ARE REFINED.

IF TEMPERATURE FACTORS ARE REFINED IN HIGHER THAN 4TH ORDER THE

SITUATION IS AS FOLLOWS

ONLY THE FIRST SIX ATOMS GIVEN ON THE ATOM PARAMETER CARDS

CAN BE REFINED WITH 5TH OR (AND) 6TH ORDER.

THE ATOMS WITH THE NUMBERS 7 TO 40 CAN BE REFINED WITH

TEMPERATURE FACTORS UP TO 4TH ORDER.

THE ATOMS WITH THE NUMBERS 41 TO 60 MUST BE RESTRICTED TO

ANISOTROPIC TEMPERATURE FACTORS.

SCALE REFINE CARD

(ONLY NEEDED IF  NEW(7) NOT EQUAL 0 ).

FORMAT (10I1)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1  IBUFF   REFINEMENT SWITCH FOR SCALE FACTOR NUMBER 1 .

0      PARAMETER IS FIXED, DO NOT REFINE.

1      REFINE THE PARAMETER.

5      REFINE PARAMETER, BUT DAMP THE SHIFT BY MULTI-

MULTIPLYING THE CALCULATED VALUE WITH      0.1

6      REFINE PARAMETER, BUT DAMP WITH THE FACTOR 0.25

7      REFINE PARAMETER, BUT DAMP WITH THE FACTOR 0.5

8      REFINE PARAMETER, BUT DAMP WITH THE FACTOR 0.75

9      REFINE PARAMETER, BUT DAMP WITH THE FACTOR 0.9

2          SAME FOR SCALE FACTOR NUMBER 2 .

3          SAME FOR SCALE FACTOR NUMBER 3 .

4          AND SO ON .

EXTINC. REFINE CARD

(ONLY NEEDED IF  NEW(7) NOT EQUAL 0  AND EXTINCTION IS SELCTED)

FORMAT (A1,6I1)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1  IBUFF   REFINEMENT SWITCH FOR THE FIRST EXTINCTION PARAMETER.

FOR THE FUNCTION OF THE VALUES SEE  SCALE REFINE CARD.

(JUST ABOVE).

2          SAME FOR THE SECOND EXTINCTION PARAMETER (IF NEEDED).

3          AND SO ON .

REMARK

IF IN CASE OF BECKER AND COPPENS EXTINCTION THE REFINEMENT SWITCH(ES)

ARE SET TO 1 AN AUTOMATIC DAMPING CAN BE SELECTED (SEE PARAMETER

NDISTR  ON THE  EXTINCTION CARD).

ALPHA REFINE CARD

(ONLY NEEDED IF  NEW(7) NOT EQUAL 0 AND THE STRUCTURE FACTOR EQUATION

NUMBER 3 OR 4 IS USED AND AT LEAST ONE ATOM IS REFINED WITH

ANHARMONIC TEMPERATURE FACTORS).

FORMAT (65I1)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1  IBUFF   REFINEMENT SWITCH FOR THE ALPHA PARAMETER OF THE ATOM

NUMBER 1 .

FOR THE MEANING OF THE VLAUES SEE  SCALE REFINE CARD .

2          SAME FOR ATOM NUMBER 2 .

3          AND SO ON .

(THERE MUST BE AS MANY REFINEMENT SWITCHES AS ATOMS

WERE GIVEN ON THE ATOM PARAMETER CARDS. THE ATOM

NUMBERS CORRESPOND TO THE SEQUENCE OF THE ATOMS ON

THESE CARDS).

SPECIAL REFINE CARD

(ONLY NEEDED IF NEW(7) NOT EQUAL 0 AND SPECIAL PARAMETERS ARE USED).

FORMAT (65I1)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1  IRPAR   REFINEMENT SWITCH FOR THE FIRST SPECIAL PARAMETER.

MEANING OF THE VALUES SEE  SCALE REFINE CARD .

2          SAME FOR THE SECOND SPECIAL PARAMETER.

3          AND SO ON.

(THERE MUST BE  NPAR (SEE SPECIAL PARAM. CARD) REFINE-

MENT SWITCHES).

ATOM REFINE CARD(S)

(ONLY NEEDED IF  NEW(7) NOT EQUAL 0 ).

PREFATORY NOTICE

LIKE THE  ATOM PARAMETER CARD  THE  ATOM REFINE CARD  CONSISTS OF

SEVERAL CARDS (UP TO 5) FOR EACH ATOM.

CARD 1

FORMAT (10I1)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1  IPARA   REFINEMENT SWITCH FOR THE TOTAL OCCUPANCY OF THE

SITE RESP. FOR THE OCCUPANCY OF SPECIES  A  IF

MULTIPLE OCCUPANCY IS USED (SEE  ATOM PARAMETER CARD).

0      PARAMETER IS FIXED, DO NOT REFINE.

1      REFINE THE PARAMETER.

2      DO NOT REFINE AND SET BACK THE PARAMETER TO  0.

3      DO NOT REFINE BUT READ NEW PARAMETER VALUE

FROM A NEW VALUE CARD (THE NEW VALUE CARD(S)

FOLLOW AFTER THE ENTIRE LIST OF  ATOM REFINE

CARDS).

4      LIKE 3 BUT NOW REFINE THE NEW PARAMETER.

5      REFINE THE PARAMETER, BUT CAMP THE SHIFT BY

MULTIPLYING THE CALCULATED VALUE WITH      0.1

6      REFINE PARAMETER, BUT DAMP WITH THE FACTOR 0.25

7      REFINE PARAMETER, BUT DAMP WITH THE FACTOR 0.5

8      REFINE PARAMETER, BUT DAMP WITH THE FACTOR 0.75

9      REFINE PARAMETER, BUT DAMP WITH THE FACTOR 0.9

REMARK

THIS POSSIBLE PARAMETER VALUES HAVE THE SAME MEANING IN ALL

REFINEMENT SWITCHES ON THE  ATOM REFINE CARDS .

2  X       REFINEMENT SWITCH FOR THE X-COORDINATE OF THE ATOM.

3  Y       REFINEMENT SWITCH FOR THE Y-COORDINATE.

4  Z       REFINEMENT SWITCH FOR THE Z-COORDINATE.

5-10          REFINEMENT SWITCHES FOR THE ELEMENTS OF THE

SECOND ORDER TENSOR (ANISORPIC TEMPERATURE FACTOR)

RESP. FOR THE ISOTROPIC  B-VALUE.

5          B-VALUE RESP. BETA(11)

6          BETA(22)

7          BETA(33)

8          BETA(12)

9          BETA(13)

10          BETA(23)

CARD 2

(ONLY NEEDED IF FOR THIS ATOM AT LEAST 3RD ORDER CUMULANTS ARE

SELECTED (SEE  T-FACTOR TYPE CARD).

FORMAT (10I1)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1-10  IPARA   REFINEMENT SWITCHES FOR THE 10 REFINABLE ELEMENTS OF

THE 3RD ORDER THERMAL TENSOR.

SEQUENCE OF THE ELEMENTS SEE  ATOM PARAMETER CARDS  OR

DEPENDENCY CARDS.

POSSIBLES VALUES OF THE SWITCHES SEE  CARD 1  OF THE

ATOM REFINE CARDS.

CARD 3

(ONLY NEEDED IF FOR THIS ATOM AT LEAST 4TH ORDER CUMULANTS ARE

SELECTED (SEE  T-FACTOR TYPE CARD).

FORMAT (15I1)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1-15  IPARA   REFINEMENT SWITCHES FOR THE 15 REFINABLE ELEMENTS OF

THE 4TH ORDER THERMAL TENSOR.

SEQUENCE OF THE ELEMENTS SEE  ATOM PARAMETER CARDS  OR

DEPENDENCY CARDS.

POSSIBLES VALUES OF THE SWITCHES SEE  CARD 1  OF THE

ATOM REFINE CARDS.

CARD 4

(ONLY NEEDED IF FOR THIS ATOM AT LEAST 5TH ORDER CUMULANTS ARE

SELECTED (SEE  T-FACTOR TYPE CARD).

FORMAT (21I1)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1-21  IPARA   REFINEMENT SWITCHES FOR THE 21 REFINABLE ELEMENTS OF

THE 5TH ORDER THERMAL TENSOR.

SEQUENCE OF THE ELEMENTS SEE  ATOM PARAMETER CARDS  OR

DEPENDENCY CARDS.

POSSIBLES VALUES OF THE SWITCHES SEE  CARD 1  OF THE

ATOM REFINE CARDS.

CARD 5

(ONLY NEEDED IF FOR THIS ATOM 6TH ORDER CUMULANTS ARE

SELECTED (SEE  T-FACTOR TYPE CARD).

FORMAT (28I1)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1-28  IPARA   REFINEMENT SWITCHES FOR THE 28 REFINABLE ELEMENTS OF

THE 6TH ORDER THERMAL TENSOR.

SEQUENCE OF THE ELEMENTS SEE  ATOM PARAMETER CARDS  OR

DEPENDENCY CARDS.

POSSIBLES VALUES OF THE SWITCHES SEE  CARD 1  OF THE

ATOM REFINE CARDS.

NEW VALUE CARD(S)

(ONLY NEEDED IF  NEW(7) NOT EQUAL 0  AND AT LEAST ONE OF THE

REFINEMENT SWITCHES ON THE  ATOM REFINE CARDS IS 3 OR 4).

FORMAT (F8.0)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 8  ---     NEW VALUE FOR THE PARAMETER FOR WHICH THE REFINEMENT

SWITCH ON THE  ATOM REFINE CARDS  WAS SET TO 3 OR 4.

--- THE NAME DEPENDS ON THE SELECTED PARAMETER.

REMARK

THERE MUST BE ONE  NEW VALUE CARD  FOR EACH REFINEMENT SWTICH SET TO

3 OR 4 . THE SEQUENCE OF THE PARAMETER VALUES  ON THE  NEW VALUE

CARDS  MUST CORRESPOND WITH THE SEQUENCE OF THE SELECTED PARAMETERS

ON THE ATOM REFINE CARDS.

DEPENDENCY CARD(S)

(ONLY NEEDED IF  NEW(7) NOT EQUAL 0 ).

FORMAT (A1,I2,I3,5(2I3,F8.0)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1  LABEL   DEPENDENCY CHANGE SELECTOR

N      THIS IS A  NEW  LIST OF DEPENDENCIES. IF DEPEN-

DENCIES WERE READ IN AN EARLIER CYCLE THEY ARE

OVERWRITTEN BY THIS NEW LIST.

IF THE LABEL=N  IS USED IT MUST APPEAR ON THE

FIRST DEPENDENCY CARD. THE END OF THE LIST MUST

BE INDICATED BY LABEL=E (SEE BELOW). ALL CARDS

BETWEEN THE  LABEL=N  AND THE  LABEL=E  CARD

MUST START WITH LABEL=BLANK.

O      USE AGAIN THE  OLD  LIST OF DEPENDENCIES READ

IN EARLIER CYCLES.

IF LABEL=O IS USED NO FURTHER DEPENDENCY CARDS

ARE ALLOWED.

(O IS ONLY SENSEFUL IF THE CURRENT CYCLE IS NOT

AN -A PRIORI- CYCLE).

A      USE THE OLD LIST OF DEPENDENCIES, AND  ADD  THE

FOLLOWING DEPENDENCIES. (FOLLOWING DEPENDENCIES

ARE..  THE DEPENDENCY ON THE CARD WITH LABEL=A

ITSELF PLUS THE DEPENDENCIES ON THE FOLLOWING

CARDS IF THESE CARDS START WITH LABEL=BLANK (OR

LABEL=A). IF NO OTHER LABEL-FUNCTION IS USED

THE LIST MUST BE TERMINATED WITH LABEL=E, BUT

IT CAN ALSO BE FOLLOWED BY A LIST OF DEPENDEN-

CIES TO BE DELETED (LABEL=D).

(LABEL=A  IS DESIGNED FOR ADDING DPENDENCIES

TO A CURRENT LIST DURING A -CONTINUE CYCLE- ).

D      USE THE OLD LIST OF DEPENDENCIES, BUT  DELETE

THE FOLLOWING DEPENDENCIES. (FOR FURTHER EXPLA-

NATIONS SEE LABEL=A. CARDS WITH LABEL=A AND

LABEL=D CAN BE ARBITRARILY MIXED).

P      LIKE LABEL=O BUT IN ADDITION THE DEPENDENCY-

LIST WILL BE PRINTED. (NO FURTHER DEPENCY CARD

IS ALLOWED).

E      THIS IS THE END OF THE LIST (END CARD)

(THE LAST DEPENDECY MUST BE GIVEN ON THE CARD

BEFORE THE END CARD. THERE IS NO DEPENDENCY

READ FROM THE  END CARD  ITSELF).

BLANK  LABEL=BLANK HAS DIFFERENT MEANINGS DEPENDING ON

THE LAST LABEL READ BEFORE.

LABEL=BLANK ON THE FIRST DEPENDENCY CARD IS

INTERPRETED AS LABEL=N.

LABEL=BLANK AFTER LABEL=A OR LABEL=D IST INTER-

PRETED AS  A  OR  D  RESPECTIVELY.

NOT ONLY LABEL=BLANK BUT ALSO BLANKS IS COLUMN

2 AND THREE IS INTERPRETED AS LABEL=E.

(ALL CARDS AFTER LABEL=N TILL THE END CARD MUST

START WITH LABEL=BLANK).

2- 3  IDPAR1  NUMBER OF THE ATOM TO WHICH THE  DEPENDANT PARAMETER

BELONGS.

5- 6  IDPAR2  NUMBER OF THE DEPENDANT PARAMETER (SEE TABLE BELOW).

7- 9  INPAR   NUMBER OF THE ATOM THE FIRST INDEPENDANT PARAMETER

BELONGS TO. (SPECIAL PARAMETERS ARE ARBITRARILY

INDICATED AS ATOM NUMBER  0 ).

10-12  INPAR   PARAMETER NUMBER OF THE FIRST INDEPENDANT PARAMETER.

(ATOM NUMBER 0 AND PARAMETER NUMBER 0 DESIGNATES AN

INITIALIZING CONSTANT ON A LINEAR DEPENDENCY

FUNCTION).

13-20  CNPAR   COEFFICIENT OF FIRST RELATIONSHIP.

21-23  INPAR   ATOM NUMBER OF SECOND INDEPENDENT PARAMETER.

24-26  INPAR   SECOND INDEPENDENT PARAMETER NUMBER.

27-34  CNPAR   SECOND COEFFICIENT

35-37

38-40          SAME FOR THE THIRD INDEPENDENT PARAMETER.

41-48

49-51

52-54          SAME FOR THE FOURTH INDEPENDENT PARAMETER.

55-62

63-65

66-68          SAME FOR THE FIFTH INDEPENDENT PARAMETER.

69-76

REMARK

IF THE INDEPENDENT ATOM NUMBER IS ZERO, DESIGNATING A SPECIAL PARA-

METER, AND THE CONSTRAINTS ARE NON LINEAR, THE SECOND AND FOURTH

INDEPENDENT PARAMETER NUMBERS MAY BE NEGATIVE TO DENOTE THE END OF AN

INCLUSIVE LIST BEGINNING WITH THE FIRST (THIRD) INDEPENDENT PARAMETER

NUMBER.

ATTENTION

IF A LIST OF DEPENDENCIES IS READ THE END OF THE LIST MUST BE INDI-

CATED BY AN  END CARD. (SEE LABEL E IN COLUMN 1).

IF THERE ARE NO DEPENDENCIES THE  END CARD  MUST BE GIVEN.

TABLE OF THE PARAMETER NUMBERS

PARAM.  PARAMETER       PARAM.  PARAMETER      PARAM.  PARAMETER

NO.                     NO.                    NO.

1     OCCUPANCY(OCCA)   2     X                5     B OR BETA(11)

(SEE REMARK       3     Y                6     BETA(22)

BELOW)           4     Z                7     BETA(33)

8     BETA(12)

9     BETA(13)

10     BETA(23)

11     C(111)           21     D(1111)         36     E(11111)

12     C(222)           22     D(2222)         37     E(22222)

13     C(333)           23     D(3333)         38     E(33333)

14     C(112)           24     D(1112)         39     E(11112)

15     C(122)           25     D(1113)         40     E(11113)

16     C(113)           26     D(1122)         41     E(11122)

17     C(133)           27     D(1123)         42     E(11123)

18     C(223)           28     D(1133)         43     E(11133)

19     C(233)           29     D(1222)         44     E(11222)

20     C(123)           30     D(1223)         45     E(11223)

31     D(1233)         46     E(11233)

32     D(1333)         47     E(11333)

33     D(2223)         48     E(12222)

34     D(2233)         49     E(12223)

35     D(2333)         50     E(12233)

51     E(12333)

52     E(13333)

53     E(22223)

54     E(22233)

55     E(22333)

56     E(23333)

57      F(111111)        67     F(111233)       77     F(122333)

58      F(222222)        68     F(111333)       78     F(123333)

59      F(333333)        69     F(112222)       79     F(133333)

60      F(111112)        70     F(112223)       80     F(222223)

61      F(111113)        71     F(112233)       81     F(222233)

62      F(111122)        72     F(112333)       82     F(222333)

63      F(111123)        73     F(113333)       83     F(223333)

64      F(111133)        74     F(122222)       84     F(233333)

65      F(111222)        75     F(122223)

66      F(111223)        76     F(122233)

REMARK

IN THE CURRENT VERSION OF THE PROGRAM NO DEPENDENCIES BETWEEN  TOTAL

OCCUPANCIES OF SITES ARE POSSIBLE. HOWEVER THIS MAY BE ACCOMPLISHED

IN THE FOLLOWING WAY..

- YOU DEFINE A  NULL SCATTERING CURVE  ON AN ADDITIONAL SCAT. CURVE

TYPE AND EXPONENT. FORM CARD.

- ON THE ATOM PARAMETER CARDS OF THE ATOMS WHO S OCCUPANCIES SHOULD

BE DEPENDANT YOU WRITE THE NUMBER OF THIS SCATTERING CURVE FOR

SPECIES B (COLUMNS 3-4 ON CARD 2 OF THE ATOM PARAMETER CARDS).

- ON CARD 1 OF THE  ATOM PARAMETER CARDS OF THESE ATOMS YOU HAVE TO

WRITE FOR  OCCUP  (TOTAL OCCUPANCY) THE MAXIMUM OCCUPANCY WHICH IS

POSSIBLE FOR SPECIES A ON THIS SITE.

FOR  OCCA  YOU HAVE TO SET THE STARTING VALUE FOR THE OCCUPANCY

OF SPECIES A.

- NOW YOU CAN DEFINE DEPENDENCIES BETWEEN THE PARAMETERS NO. 1

OF THESE ATOMS, E.G. DEPENDENCIES BETWEEN THE OCCUPANCIES OF

SPECIES A. (THE DIFFERENCE TO THE MAXIMUM OCCUPANCY OF EACH SITE

IS ALWAYS FILLED WITH THE  NULL SCATTERING CURVE).

EXAMPLE

SCATTERING LENGTHS.. (NEUTRON DIFFRACTION)

AG

0.      0.      0.      0.      0.      0.      0.      0.      6.02

NULL

0.      0.      0.      0.      0.      0.      0.      0.      0.

ATOM PARAMETER CARDS

AG1     12  .3333 .26564

1 2    .25    .0     .5     3.

AG2      6  .6667 .13575

1 2    .0     .5     .5     2.5

ATOM REFINE CARDS

10001

00001

DEPENDENCY CARDS

N 2  1  0  0 .6667    1  1 -2.

E

EXPLANATION

THE OCCUPANCY PER SITE OF AG2 ON 0,1/2,1/2 IS  2/3 MINUS TWO TIMES

THE OCCUPANCY PER SITE OF AG1 ON 1/4,0,1/2.

(A LATER VERSION OF PROMETHEUS WILL BE CHANGED IN THIS POINT).

DIFFRACTOMETER CARD

(ONLY NEEDED IF  NEW(8) IS GREATER THAN 5 ).

FORMAT (A4)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 4  MACHIN  INDICATOR FOR THE DEFINITION OF THE COORDINATE SYSTEM

ON THE DIFFRACTOMETER USED FOR THE MEASUREMENT.

BUSI   THE COORDINATE SYSTEM DEFINITION OF BUSING AND

LEVY IS USED. (SEE DESCRIPTION PART).

DUMM   A SPECIAL DEFINITION IS USED WHICH HAS TO BE

PROGRAMMED BY THE USER. FOR DETAIL SEE DESCRIP-

TION PART.

ANYTHING

ELSE   THE COORDINATE SYSTEM DEFINITION OF THE PHILIPS

PW1100 DIFFRACTOMETER IS USED. (SEE DESCRIPTION

PART).

ORIENTATION CARDS

(NEEDED ONLY IF  NEW(8) IS GREATER THAN 5 ).

FORMAT OF EACH CARD  (3F8.0)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

CARD 1

1- 8  UB(1)   U11    COMPONENTS OF THE FIRST LINE OF THE

9-16  UB(2)   U12    ORIENTATION MATRIX. (DEFINITION OF THE

17-24  UB(3)   U13    ORIENTATION MATRIX SEE DESCRIPTION PART).

CARD 2

1- 8  UB(4)   U21

9-16  UB(5)   U22    SECOND LINE.

17-24  UB(6)   U23

CARD 3

1- 8  UB(7)   U31

9-16  UB(8)   U32    THIRD LINE.

17-24  UB(9)   U33

FORMAT CARD

(ONLY NEEDED IF  NEW(8) NOT EQUQL 0 ).

FORMAT (18A4)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1-72  FRMT    INDICATOR FOR THE FORMAT OF THE HKL-CARDS.

1- 4          STAN   THE HKL-CARDS ARE READ WITH THE STANDARD FORMAT

(A3,10X,3I3,3X,I3,3X,F9.0,9X,F5.0)

(SEQUENCE OF VARIABLES SEE HKL-CARDS BELOW OR

NEW(8) ON THE  MAIN CONTROL CARD).

IF  STAN  IS TYPED IN COLUMNS 1-4 NO FURTHER

MUST BE GIVEN ON THIS CARD.

(A3,   THE HKL-CARDS TO BE READ HAVE NO STANDARD

FORMAT. BEHIND  (A3,  THERE MUST FOLLOW THE

REST OF THE FORMAT TO BE USED, WRITTEN IN STAN-

DARD FORTRAN NOTATION.

(THE BEGINNING WITH  (A3,  IS NECESSARY TO

RECOGNIZE THE  END  CARD INDICATING THE END OF

THE HKL-CARDS).

DATA DEFINING CARD

(ONLY NEEDED IF  NEW(8) NOT EQUAL 0 ).

FORMAT (I2,2I1F8.0)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 2  ITEMP   INPUT UNIT INDICATOR

0      HKL CARDS ARE ON STANDARD INPUT UNIT (IN).

NOT 0  ITEMP IS THE LOGICAL NUMBER OF THE INPUT

UNIT CONTAINING THE HKL-CARDS..

3  IFOBS   INTENSITY OR STRUCTURE FACTOR INDICATOR

0      INTENSITIES IN CARDS.

1      STRUCTURE FACTORS ON CARDS.

4  IWT     STANDARD DEVIATION INDICATOR

0      STANDARD DEVIATION OF I ON CARDS.

(I = INTENSITY).

REMARKS

IF THE STANDARD DEVIATION IS MODIFIED BY AN IGNORANCE

FACTOR (PARAMETER  WTFAC  ON THIS CARD) THE FOLLOWING

EQUATION IS USED

SIGMA(F*F) = SQRT ( (SIGMA(I))**2 + (WTFAC*I)**2 )

TO CALCULATE THE STANDARD DEVIATION OF F THE FORMALISM

OF REES IS USED (SEE DESCRIPTION PART).

1      STANDARD DEVIATION OF F ON CARDS.

REMARK

MODIFICATION BY AN IGNORANCE FACTOR (PARAMETER  WTFAC

ON THIS CARD ) IS DONE IN THE FOLLOWING WAY

SIGMA(F) = SQRT( (SIGMA(F))**2 + (WTFAC * F)**2 )

2      WEIGHT ON CARDS.

IF REFINEMENT IS REQUESTED ON F THE PROGRAMME

PRESUMES

WEIGHT = 1 / (SIGMA(F) **2 )

(THIS PRESUMPTION IS USED IF SIGMA(F) IS

MODIFIED BY AN IGNORANCE FACTOR AS DESCRIBED

UNDER IWT = 1 ).

IF REFINEMENT IS REQUESTED ON F*F THE PROGRAMME

PRESUMES

WEIGHT = 1 / (SIGMA(I) **2 )

AND A MODIFICATION BY AN IGNORANCE FACTOR IS

DONE AS DESCRIBED UNDER  IWT = 0 .

3      LIKE IWT=0 BUT FOR CALCULATION OF THE STANDARD

DEVIATION OF F INSTEAD OF THE FORMALISM OF REES

THERE IS USED THE FORMULA

SIGMA(F) = 0.5 * SIGMA(I) / F

4      STANDARD DEVIATION OF I ON CARDS.

CALCULATION OF THE STANDARD DEVIATION OF F

ACCORDING TO THE FORMULA OF REES.

THE VALUE OF SIGMA(F) CAN IN ADDITION BE

MODIFIED BY THE ROUTINE  WEIGHT  TO BE WRITTEN

BY THE USER.

5      STANDARD DEVIATION OF F ON CARDS.

THE STANDARD DEVIATION CAN BE MODIFIED BY THE

ROUTINE WEIGHT TO BE WRITTEN BY THE USER.

5  IREJL   0/1    DO NOT READ/READ REFLECTIONS NOT TO BE

INCLUDED IN THE REFINEMENT FROM REJECT-FILE

6-12  WTFAC   IGNORANCE FACTOR

0.     THE STANDARD DEVIATION WILL NOT BE MODIFIED BY

AN IGNORANCE FACTOR.

NOT 0. THE STANDARD DEVIATION WILL BE MODIFIED BY THE

IGNORANCE FACTOR  WTFAC  AS DESCRIBED UNDER

THE PARAMETER  IWT  ABOVE.

MEASUREMENT CARD

(ONLY NEEDED IF  NEW(8) NOT EQUAL 0 ).

FORMAT (2I1,2F8.0)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1  INEUT   RADIATION AND MONOCHROMATOR INDICATOR

0      REFLECTIONS WERE MEASURED WITH XRAY-RADIATION

USING A GRAPHITE MONOCHROMATOR MOUNTED ON THE

DIFFRACTOMETER WITH A DIHEDRALE ANGLE OF 9

DEGREES. (IF NO  MO-K-ALPHA RADIATION WAS USED

INEUT MUST BE 2 ).

1      REFLECTIONS WERE MEASURED WITH NEUTRON

RADIATION.

2      REFLECTIONS WERE MEASURED WITH XRAY-RADIATION.

READ IN THE MONOCHROMATOR DATA (MONOCHROMATOR

CARD).

2  LPCORP  LORENTZ AND POLARISATION CORRECTION SELECTOR

0      DO NOT CALCULATE A CORRECTION.

1      CALCULATE A LORENTZ AND POLARISATION CORRECTION

(IF  INEUT=1  ONLY LORENTZ CORRECTION).

3-10  WVLNGT  WAVELENGTH IN ANGSTROMS

11-18  TBAR    ABSORPTION WEIGHTED MEAN PATH LENGTH IN MICROMETERS TO

BE USED FOR ALL REFLECTIONS. THE VALUE GIVEN HERE IS

OVERWRITTEN, IF TBAR IS READ FROM THE HKL-CARDS

(NEW(8) = 2 (7)  OR  4 (9) ).

(FOR A SPHERE TBAR MUST BE 3/2 TIMES THE RADIUS).

0      THE DEFAULT VALUE OF 300 MICROMETERS IS USED.

NOT 0  TBAR IS THE PATH LENGTH IN MICROMETERS.

MONOCHROMATOR CARD

(ONLY NEEDED IF  NEW(8) NOT EQUAL 0  AND THE PARAMETER  INEUT ON THE

MEASUREMENT CARD EQUAL 2 ).

FORMAT (2F8.0,I2)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 8  DIHED   DIHEDRALE ANGLE IN DEGREES.

9-16  TM      IF ICOS = 0     TM = MONOCHROMATOR ANGLE IN DEGREES.

18  ICOS    IF ICOS = 1     TM = SQUARE OF THE COSINE OF THE

MONOCHROMATOR ANGLE.

PARTIAL CARDS

(ONLY NEEDED IF THE PARAMETER  IPART  ON THE  MAIN CONTROL CARD  IS

NOT EQUAL 0 AND  NEW(8)=0 ).

FORMAT (I2)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1- 2  I       NUMBER OF THE ATOM TO BE DELETED .

0      INDICATOR THAT ALL PARTIAL CARDS WERE READ.

NOT 0  NUMBER OF AN ATOM. THE NEXT CARD WILL BE

AGAIN A PARTIAL CARD.

HKL CARDS

(ONLY NEEDED IF NEW(8) NOT EQUAL 0  AND THE HKL-CARDS SHOULD BE READ

FROM STANDARD INPUT UNIT (PARAMETER TEMP ON THE DATA DEFINING CARD

EQUAL 0) ).

THE INFORMATION READ FROM THE HKL-CARDS IS DESCRIBED UNDER THE PARA-

METER  NEW(8) ON THE MAIN CONTROL CARD, AND THE FORMAT OF THESE CARDS

DEPENDS ON THE INPUT GIVEN ON THE  FORMAT CARD.

CONTINUE CARD

FORMAT (4I1)

COLUMN NAME    FUNCTION AND POSSIBLE VALUES

1  IEF     NEW CYCLE SELECTOR

0      CALCULATE NO MORE CYCLE. END OF THE PROGRAMME

REFINE.

1      CALCULATE ONE MORE CYCLE. (THE CYCLE IS STARTED

WITH READING THE  CYCLE START CARD ).

2  NFLAG   PARAMETER OUTPUT SELECTOR

0      DO NOT PRINT ANY ATOM PARAMETERS ON THE

UNIT WITH THE LOGICAL NUMBER  IPUN (IPUN=04).

1      PRINT UIJ-ATOMCARD OF XRAY SYSTEM.

(FOR UIJ-ATOMCARD SEE XRAY SYSTEM).

2      PRINT THE FOLLOWING CARDS

SCALE FACTOR    CARD

EXTINCTION      CARD

ATOM PARAMETER  CARDS

SPECIAL PARAM.  CARD   (ODER SPECIAL VALUE CARD

ALPHA PARAMETER CARD

T-FACTOR TYPR   CARD

3      PRINT THE CARDS DESCRIBED UNDER NFLAG=1 PLUS

THOSE UNDER NFLAG=2.

3-4    ISC1    LOGICAL NUMBER OF THE MAINFILE TO BE WRITTEN IF THIS

IS THE LAST CYCLE OF THE CURRENT RUN.

0      THE DEFAULT VALUE 10 WILL BE USED.

NOT 0  ISC1 IS THE LOGICAL NUMBER OF THE MAINFILE.

S P H E R E

CARD ORDER SUMMARY OF THE PROGRAM SPHERE

TITLE             CARD

LABEL             CARD

INPUT UNIT        CARD

OUTPUT UNIT       CARD

ABSORPTION        CARD

CELL CONSTANT     CARDS

FORMAT            CARDS

LIST SELECTOR     CARD

NO CARD IS OPTIONAL

REMARK

THE CARD ORDERS OF SPHERE AND TDSCOR DIFFER FROM THOSE OF THE OTHER

PROGRAMS OF THE SYSTEM PROMETHEUS. SPACE IS LEFT TO WRITE THE MEANING

OF EACH PARAMETER IN FRONT OF ITS POSITION ON THE CARD SO THAT THE

USER CAN INSERT OR CHANGE PARAMETERS WITHOUT HAVING TO REFER TO THE

CARDORDER. (THE PARAMETERS ARE READ FORM THE CARDS WITH A FORMAT THAT

AGREES WITH THE THE POSITIONS IN THE EXAMPLE BELOW)

EXAMPLE OF A CALL OF THE PROGRAM SPHERE

TITEL                     =LIKSO4 022C

LABEL                     =LIK022

INPUT -UNIT OF HKL CARDS  =1

OUTPUT-UNIT OF HKL CARDS  =11

MUE*R   =0.57707  RADIUS  = 0.335      WAVELENGTH = .71069

A       = 5.145          B       = 5.145          C       = 3.632

ALPHA   = 90.            BETA    = 90.               GAMMA= 120.

INPUT FORMAT (A3,10X,3I3,3X,2I3,F9.0,9X,F5.0)

OUTPUT FORMAT(A3,4X,A4,A2,3I3,3X,2I3,F9.0,F9.5,F5.0,7X,F8.4)

LISTING OF HKL CARDS=YES

FORMAT (7X,16A4)

COLUMN    NAME       FUNCTION AND POSSIBLE VALUES

8-72      TITLE      ANY SEQUENCE OF SYMBOLS USED AS TITLE OF THE PROGRAM

LABEL CARD

FORMAT (7X,A4,A2)

COLUMN     NAME       FUNCTION AND POSSIBLE VALUES

8-14       LABEL      ANY SEQUENCE OF SYMBOLS USED TO LABEL THE HKL CARDS

INPUT-UNIT CARD

FORMAT(25X,I5)

COLUMN     NAME       FUNCTION AND POSSIBLE VALUES

26-30      NUIN       LOGICAL NUMBER OF THE INPUT FILE CONTAINING THE

HKL CARDS

NOT 0  NUIN IS THE LOGICAL NUMBER OF THE INPUT FILE

OUTPUT-UNIT CARD

FORMAT (25X,I5)

COLUMN     NAME       FUNCTION AND POSSIBLE VALUES

26-30      NUOUT      LOGICAL NUMBER OF THE OUTPUT FILE

0         THE DEFAULT VALUE 02 WILL BE USED

NOT 0     NUOUT IS THE LOGICAL NUMBER OF THE OUTPUT FILE

ABSORPTION CARD

FORMAT (7X,F10.5,8X,F10.5,12X,F10.5)

COLUMN     NAME      FUNCTION AND POSSIBLE VALUES

8-17       UMR       ABSORPTION COEFFICIENT * RADIUS OT THE CRYSTAL

27-36      RADIUS    RADIUS OF THE SPHERE IN MILLIMETERS

48-57      WLGTH     WAVELENGTH OF THE RADIATION IN ANGSTROEMS

0       THE DEFAULT VALUE 0.71069 ANGSTROEM IS USED

NOT 0   WLGTH IS THE WAVELENGTH IN ANGSTROEMS

CELL CONSTANT CARDS

CARD 1

FORMAT (7X,F10.5,6X,F12.5,7X,F10.5)

COLUMN     NAME       FUNCTION AND POSSIBLE VALUES

8-17       A          LENGTH OF THE A-AXIS IN ANGSTROEMS

24-35      B          LENGTH OF THE B-AXIS

43-52      C          LENGTH OF THE C-AXIS

CARD 2

FORMAT (7X,F10.5,6X,F12.5,7X,Fl0.5)

COLUMN     NAME       FUNCTION AND POSSIBLE VALUES

8-17       ALPHA      VALUE OF ALPHA IN DEGREES

24-35      BETA       VALUE OF BETA

43-52      GAMMA      VALUE OF GAMMA

FORMAT CARDS

CARD 1

FORMAT (13X,13A4)

COLUMN     NAME       FUNCTION AND POSSIBLE VALUES

14-65      FORMIN     INPUT FORMAT FOR HKL CARDS

THE FOLLOWING PARAMETERS ARE READ BY THE PROGRAM

THE LABEL HKL, THE MILLER INDICES H, K, L, THE

REJECT FLAG, THE SCALE GROUP. THE INTENSITY OR

FOBS AND SIGMA

0        DEFAULT FORMAT USED IS

(A3,10X,3I3,3X,2I3,F9.0,9X,F5.0)

NOT 0    FORMIN IS THE INPUT FORMAT

CARD 2

FORMAT (13X,13A4)

COLUMN     NAME       FUNCTION AND POSSIBLE VALUES

14-65      FOROUT     OUTPUT FORMAT FOR HKL CARDS

0          DEFAULT FORMAT USED IS

(A3,4X,A4,A2,3I3,3X,2I3,F9.0,F9.5,F5.0,7X,F8.4)

NOT 0      FOROUT IS THE OUTPUT FORMAT

LIST SELECTOR CARD

FORMAT (9X,A4)

COLUMN     NAME       FUNCTION AND POSSIBLE VALUES

10-13      LIS        SWITCH FOR PRINT A LIST OF REFLECTIONS

YES          A LIST IS PRINTED

NOT YES      NO LIST IS PRINTED

T D S C O R

CARD ORDER SUMMARY OF THE PROGRAM  TDSCOR

TITLE               CARD

INPUT-UNIT          CARD

OUTPUT-UNIT         CARD

TENSOR TYPE         CARD

TENSOR VALUE        CARDS

DETECTOR            CARD

SCAN                CARD

WAVELENGTH          CARD

TEMPERATURE         CARD

SPECIFIC WEIGHT     CARD

LIST SELECTOR       CARD

INTEGRATION         CARD

CELL                CARDS

ROTATION            CARDS

* HKL                 CARDS

* END                 CARD

*   THESE CARDS ARE OPTIONAL

REMARK

THE CARDS FOR THE PROGRAM  TDSCOR  LIKE THOSE OF THE PROGRAM  SPHERE DIFFER FROM THE CARDS OF THE OTHER PARTS OF THE SYSTEM  PROMETHEUS. TEXT CAN BE INCLUDED ON THE CARDS WHICH EXPLAINS THE MEANING OF THE PARAMETERS. THE USER CAN INSERT HIS PARAMETER VALUES WITHOUT HAVING TO REFER TO THE CARD ORDER. (THE CARDS ARE NOT READ UNFORMATED. THE FORMAT IS ONLY CHOSEN SO THAT THE VALUES ARE READ AT THE POSITIONS SHOWN IN THE EXAMPLE BELOW).

EXAMPLE OF A CALL OF THE PROGRAM  TDSCOR..

TDSCOR

THIS IS THE TITLE CARD FOR THE PROGRAM  TDSCOR

INPUT-UNIT OF HKL CARDS=            01

OUTPUT-UNIT OF HKL CARDS=        02

STIFFNESS

1.15   0.39    0.18    0.00    0.00    0

1.15    0.18    0.00    0.00    0

1.18    0.00    0.00    0

0.17    0.00    0

0.17    0

0.38

DET-HEIGHT= 1.5  WIDTH=  2.

OMEGA-SCAN TILL  60. DEGREE   (PARAMETER SCT OF PW1100)

WAVELENGTH                      =0.70169

TEMPERATURE(C)                  = 615.

SPECIFIC WEIGHT                 = 1.293

LISTING OF HKL-CARDS            = YES

NUMBER OF GRIDS FOR INTEGRATION =       2

A= 3.7235 B= 3.7235  C= 3.881

ALPHA= 90. BETA= 90. GAMMA= 120.

ROTATION ANGLE OF CRYSTALL    LEFT    TILL MAXIMUM=      1.5

ROTATION ANGLE OF CRYSTALL    RIGHT   TILL MAXIMUM=      1.5

FINISH

TITLE CARD

FORMAT (18A4)

COLUMN    NAME        FUNCTION AND POSSIBLE VALUES

1-72      TITEL       ANY SEQUENCE OF SYMBOLS USED AS A TITLE OF THE PROGRAM

INPUT-UNIT CARD

FORMAT (25X,Il0)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

26-35     INUNIT       LOGICAL NUMBER OF THE INPUT FILE CONTAINING THE

HKL-CARDS.

0       THE DEFAULT VALUE  01  WILL BE USED.

NOT 0   INUNIT IS THE LOGICAL NUMBER OF THE INPUT FILE.

(INPUT FILE  CAN ALSO BE THE STANDARD INPUT UNIT).

OUTPUT-UNIT CARD

FORMAT (25X,Il0)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

26-35     OTUNIT       LOGICAL NUMBER OF THE OUTPUT FILE.

0       THE DEFAULT VALUE  02  WILL BE USED.

NOT 0   OTUNIT IS THE LOGICAL NUMBER OF THE OUTPUT FILE

TENSOR TYPE CARD

FORMAT (A4)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

l- 4      ITYPE        INDICATOR FOR COMPLIANCE OR STIFFNESS TENSOR GIVEN

ON THE FOLLOWING TENSOR VALUE CARDS.

COMP         COMPLIANCE TENSOR

STIF         STIFFNESS TENSOR

TENSOR VALUE CARDS

THE FOLLOWING SIX CARDS CONTAIN THE VALUES OF THE TENSOR ELEMENTS.

(UNIT   IN CASE OF STIFFNESS

GRAMMS*10**12/(CM*S**2) = 10**12 DYN/(CM**2)

AND THE INVERSE IN CASE OF COMPLIANCE).

BECAUSE BOTH TENSORS (COMPLIANCE TENSOR AND STIFFNESS TENSOR) ARE

SYMMETRIC, ONLY THE DIAGONAL ELEMENTS AND THE ELEMENTS ABOVE THE

DIAGONAL ARE READ BY THE PROGRAM.

CARD 1

FORMAT (6F8.0)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

1-48      RMAT         VALUES OF THE FIRST LINE OF THE TENSOR.

1- 8  C(11)  RESP.  S(ll)

9-16  C(12)

17-24  C(13)

25-32  C(14)

33-40  C(15)

41-48  C(16)

CARD 2

FORMAT (8X,7F8.0)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

9-48      RMAT         VALUES OF THE SECOND ROW OF THE TENSOR STARTING

WITH THE DIAGONAL ELEMENT.

9-16  C(22)  RESP.  S(22)

17-24  C(23)

25-32  C(24)

33-40  C(25)

40-48  C(26)

CARD 3

FORMAT (16X,4F8.0)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

17-48     RMAT         VALUES OF THE THIRD LINE OF THE TENSOR STARTING

WITH THE DIAGONAL ELEMENT.

17-24  C(33)  RESP.  S(33)

25-32  C(34)

33-40  C(35)

40-48  C(36)

CARD 4

FORMAT (24X,3F8.0)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

25-48     RMAT         VALUES OF THE FOURTH LINE OF THE TENSOR STARTING

WITH THE DIAGONAL ELEMENT.

25-32  C(44)  RESP.  S(44)

33-40  C(45)

41-48  C(46)

CARD 5

FORMAT (32X,2F8.0)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

32-48     RMAT         VALUES OF THE FIFTH LINE OF THE TENSOR STARTING

WITH THE DIAGONAL ELEMENT.

32-40  C(55)  RESP.  S(55)

41-48  C(56)  RESP.  S(56)

CARD 6

FORMAT (40X,F8.0)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

41-48     RMAT         C(66)  RESP.  S(66)

DETECTOR CARD

FORMAT (llX,F6.0,6X,F6.0)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

12-17     DCHI         HIGHT OF DETECTOR APERTURE IN DEGREES.

24-29     DNY          WIDTH OF DETECTOR APERTURE IN DEGREES.

SCAN CARD

FORMAT (15X,F6.0)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

16-21     SCT          SCATTERING ANGLE IN DEGREES FOR THE TRANSITION FROM

MEASUREMENTS WITH THE OMEGA SCAN (BELOW SCT) TO MEA-

SUREMENTS WITH THE THETA-TWO THETA SCAN (ABOVE SCT).

(THIS ANGLE IS IDENTICAL WITH THE PARAMETER  SCT

OF THE PHILIPS PW1100 FOUR CIRCLE DIFFRACTOMETER).

WAVELENGTH CARD

FORMAT (llX,F8.0)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

12-19     WVLNTH       WAVELENGTH IN ANGSTROMS.

0           THE DEFAULT VALUE 0.71069 ANGSTROMS IS USED.

(MO-K(ALPHA) RADIATION)

NOT 0   WVLNTH IS THE WAVELENGTH IN ANGSTROMS.

TEMPERATURE CARD

FORMAT (12X,A1,3X,F8.O)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

13     ITEMP        IDENTIFIER FOR THE UNIT OF THE TEMPERATURE

C        TEMPERATURE IS GIVEN IN DEGREES CELSIUS.

NOT C    TEMPERATURE IS GIVEN IN KELVIN.

17-24     T            MEASURING TEMPERATURE IN THE UNIT DEFINED BY  ITEMP.

SPECIFIC WEIGHT CARD

FORMAT (16X,F8.0)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

17-24     RHO          SPECIFIC WEIGHT IN GRAMMS/(CM**3).

LIST SELECTOR CARD

FORMAT (21X,4Al)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

22-25     IPUT         SELECTOR FOR PRINT A LIST OF THE HKL-CARDS

WITH THE CORRECTED INTENSITIES.

IF IN ONE OF THE FOUR COLUMNS A   Y  IS READ THE LIST

WILL BE PRINTED (Y = YES). IF NO   Y   IS READ THE LIST

WILL NOT BE PRINTED.

INTEGRATION CARD

FORMAT (32X,I8)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

33-40     IGRID        NUMBER OF GRIDS TO BE USED FOR THE GAUSSAIN INTE-

GRATION.

0          THE DEFAULT VALUE  20  WILL BE USED.

NOT 0  IGRID IS THE NUMBER OF GRIDS TO BE USED.

CELL CARDS

THE FOLLOWING TWO CARDS CONTAIN THE PARAMTERS OF THE UNIT CELL.

CARD 1

FORMAT (3(2X,F8.0)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

3-10     CELL         LENGTH OF THE A-AXIS IN ANGSTROMS

13-20                                B-AXIS

23-30                                C-AXIS

CARD 2

FORMAT (6X,F8.0,5X,F8.0,6X,F8.0)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

7-14     CELL         VALUE OF ALPHA IN DEGREES

20-27                           BETA

34-41                           GAMMA

ROTATION ANGLE CARDS

THESE TWO CARDS ARE ALWAYS READ, BUT IF ONE OF THE TWO VALUES IS

THE INDIVIDUEL ROTATION ANGLES FOR EACH REFLECTION MUST BE GIVEN

ON THE HKL-CARDS.

FORMAT (51X,F8.0)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

52-59     SCMOG        CARD 1 ROTATION ANGLE OF THE CRYSTALL LEFT TILL PEAK

IF ALGORITHMS LIKE THE LEHMANN-LARSON (ACTA-

CRYST. A30,580(74)) ARE USED FOR THE DETERMI-

NATION OF THE INTEGRATED INTENSITIES, SCMOG

IS THE WIDTH OF THE CALCULATED REFLECTION PRO-

FILE FROM THE LEFT (RIGHT) SIDE TO THE PEAK-

MAXIMUM.

0         SCMOG IS INDIVIDUALLY GIVEN FOR EACH

REFLECTION ON THE HKL-CARDS

NOT 0     SCMOG IS GIVEN HERE

52-59     SCMOG        CARD 2 ROTATION ANGLE OF THE CRYSTALL RIGHT TILL PEAK

(SEE AB0VE)

HKL-CARDS

(ONLY NECESSARY HERE IF ON THE INPUT UNIT CARD THE LOGICAL NUMBER OF

THE STANDARD INPUT UNIT WAS SELECTED).

FORMAT (A4,9X,3I3,3X,2I3,2F9.0.F5.0,lX,F6.4.lX,F6.4)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

1- 4     LAB          LABEL TO RECOGNIZE THE END CARD (SEE BELOW).

14-22     IH           MILLER INDICES.

14-16  H

17-19  K

20-22  L

26-28     MREJ         INDICATOR FOR  LESS THAN  REFLECTIONS

THE INDICATOR IS NOT USED IN THIS PROGRAM. IT IS ONLY

REWRITTEN ON THE OUTPUT FILE.

29-31     ISC          SCALE GROUP

THE SCALE GROUP IS NOT USED IN THIS PROGRAM. IT IS

ONLY REWRITTEN ON THE OUTPUT FILE.

32-40     FI           INTENSITY

41-49     TBAR         ABSORPTION WEIGHTED MEAN PATH LENGTH (ONLY REWRITTEN

ON THE OUTPUT FILE).

50-54     SIGMAI       STANDARD DEVIATION OF THE INTENSITY.

56-61     RU           WIDTH OF THE PEAK LEFT FROM MAXIMUM IN DEGREES.

63-68     RO           WIDTH OF THE PEAK RIGHT FROM MAXIMUM IN DEGREES.

(RU AND RO ARE ONLY READ IF ROTATION ANGLE CARDS ARE

EMPTY).

END CARD

(THIS CARD IS ONLY NECESSARY IF THE HKL CARDS ARE READ FROM THE

STANDARD INPUT UNIT).

FORMAT (A3)

COLUMN    NAME         FUNCTION AND POSSIBLE VALUES

1- 3     LAB          IDENTIFIER OF THE CARD

END          THE LAST HKL CARD WAS READ. THE INPUT STREAM OF

THE PROGRAM IS TERMINATED.